/*

* Licensed to the OpenAirInterface (OAI) Software Alliance under one or more

* contributor license agreements.  See the NOTICE file distributed with

* this work for additional information regarding copyright ownership.

* The OpenAirInterface Software Alliance licenses this file to You under

* the OAI Public License, Version 1.1  (the "License"); you may not use this file

* except in compliance with the License.

* You may obtain a copy of the License at

*

*      http://www.openairinterface.org/?page_id=698

*

* Unless required by applicable law or agreed to in writing, software

* distributed under the License is distributed on an "AS IS" BASIS,

* WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

* See the License for the specific language governing permissions and

* limitations under the License.

*-------------------------------------------------------------------------------

* For more information about the OpenAirInterface (OAI) Software Alliance:

*      contact@openairinterface.org

*/

/*! \file gNB_scheduler_ulsch.c

* \brief gNB procedures for the ULSCH transport channel

* \author Navid Nikaein and Raymond Knopp, Guido Casati

* \date 2019

* \email: guido.casati@iis.fraunhofer.de

* \version 1.0

* @ingroup _mac

*/

#include "LAYER2/NR_MAC_gNB/mac_proto.h"

#include "executables/softmodem-common.h"

#include "common/utils/nr/nr_common.h"

#include "utils.h"

#include <openair2/UTIL/OPT/opt.h>

#include "LAYER2/nr_rlc/nr_rlc_oai_api.h"

#include <time.h>

#include <math.h>

//#define SRS_IND_DEBUG

/* Configuration parameters for time-varying PRB allocation */

#define TIME_CYCLE_DURATION 20.0  // 20 seconds cycle (10s intervals)

#define MAX_TIME_CONFIGS 20       // Maximum time intervals per cycle

/* Time-based configuration structure */

typedef struct {

double start_time;    // Start time in seconds (within 20s cycle)

double end_time;      // End time in seconds (within 20s cycle)

int prb_start;

int prb_size;

} time_config_t;

/* Dynamic PRB allocation configuration for different UEs */

typedef struct {

uint64_t ue_id; /* CU-UE-ID if available, otherwise RNTI */

time_config_t time_configs[MAX_TIME_CONFIGS];  // Fixed array for better performance

int num_configs;                               // Number of time configurations

int current_config_idx;                        // Current active configuration index

int last_cycle;                                // Last processed cycle number

int last_seen_cycle;                           // Last cycle in which this UE was seen bound

bool enabled;

} dynamic_prb_config_t;

/* Time-varying PRB configurations with 20-second cycles (10-second intervals)

* Note: ue_id == 0 means unassigned; the first two UEs seen will be bound lazily.

*/

static dynamic_prb_config_t dynamic_prb_configs[] = {

// UE 1 configuration - alternating pattern: 20 PRBs → 40 PRBs (10s intervals)

{

.ue_id = 0,  // Will be bound to first UE

.time_configs = {

{0.0, 10.0, 0, 20},   // 20 PRBs from 0-10 seconds (PRBs 0-19)

{10.0, 20.0, 0, 40},  // 40 PRBs from 10-20 seconds (PRBs 0-39)

// At 20s, cycle resets to 0s automatically

},

.num_configs = 2,

.current_config_idx = 0,

.last_cycle = -1,  // Initialize to -1 to detect first cycle

.last_seen_cycle = -1,

.enabled = true

},

// UE 2 configuration - alternating pattern: 40 PRBs → 20 PRBs (10s intervals)

{

.ue_id = 0,  // Will be bound to second UE

.time_configs = {

{0.0, 10.0, 40, 40},  // 40 PRBs from 0-10 seconds (PRBs 40-79)

{10.0, 20.0, 40, 20}, // 20 PRBs from 10-20 seconds (PRBs 40-59)

// At 20s, cycle resets to 0s automatically

},

.num_configs = 2,

.current_config_idx = 0,

.last_cycle = -1,

.last_seen_cycle = -1,

.enabled = true

},

{.ue_id = 0, .num_configs = 0, .enabled = false}  // Terminator

};

/* Get current simulation time in seconds with cyclic reset */

static double get_current_time_seconds(frame_t frame, slot_t slot) {

// For numerology 1 (30kHz SCS): 1ms = 30 slots, 1 slot = 1/30 ms

const double slot_duration_ms = 1.0 / 30.0;  // ms per slot

double absolute_time = (frame * 20.0 + slot * slot_duration_ms) / 1000.0;  // Convert to seconds

// Apply cyclic reset every TIME_CYCLE_DURATION seconds

double cyclic_time = fmod(absolute_time, TIME_CYCLE_DURATION);

// Debug logging every 50 frames to track timing more frequently

static int last_debug_frame = -1;

if (frame % 50 == 0 && frame != last_debug_frame) {

LOG_I(NR_MAC, "Timing Debug: Frame %d, Slot %d, Absolute Time %.3fs, Cyclic Time %.3fs\n",

frame, slot, absolute_time, cyclic_time);

last_debug_frame = frame;

}

return cyclic_time;

}

/* Get cycle number for logging/debugging */

static int get_current_cycle(frame_t frame, slot_t slot) {

const double slot_duration_ms = 1.0 / 30.0;

double absolute_time = (frame * 20.0 + slot * slot_duration_ms) / 1000.0;

return (int)(absolute_time / TIME_CYCLE_DURATION);

}

/* Update configuration based on current cyclic time */

static void update_time_based_config(dynamic_prb_config_t *config, double cyclic_time, int current_cycle) {

if (!config || !config->enabled || config->num_configs == 0)

return;

// Check if we've entered a new cycle

if (config->last_cycle != current_cycle) {

if (config->last_cycle >= 0) {  // Not the first cycle

LOG_I(NR_MAC, "UE-ID 0x%lx: Starting new 20s cycle %d (time reset to 0)\n",

config->ue_id, current_cycle);

}

config->last_cycle = current_cycle;

config->current_config_idx = 0;  // Reset to first configuration in new cycle

}

// Find the appropriate configuration for current cyclic time

int new_config_idx = -1;

for (int i = 0; i < config->num_configs; i++) {

if (cyclic_time >= config->time_configs[i].start_time &&

cyclic_time < config->time_configs[i].end_time) {

new_config_idx = i;

break;

}

}

if (new_config_idx >= 0 && config->current_config_idx != new_config_idx) {

config->current_config_idx = new_config_idx;

LOG_I(NR_MAC, "UE-ID 0x%lx: Cycle %d, switching to config %d at time %.2fs (PRB start=%d, size=%d)\n",

config->ue_id, current_cycle, new_config_idx, cyclic_time,

config->time_configs[new_config_idx].prb_start,

config->time_configs[new_config_idx].prb_size);

} else if (new_config_idx < 0) {

// Handle case where current time is outside all configured ranges within the cycle

LOG_W(NR_MAC, "UE-ID 0x%lx: Cycle %d, time %.2fs outside configured ranges\n",

config->ue_id, current_cycle, cyclic_time);

}

}

/* \brief Print dynamic PRB configuration for debugging */

static void print_dynamic_prb_config(frame_t frame, slot_t slot) {

double cyclic_time = get_current_time_seconds(frame, slot);

int current_cycle = get_current_cycle(frame, slot);

LOG_I(NR_MAC, "=== Dynamic PRB Configuration ===\n");

LOG_I(NR_MAC, "Current Cycle: %d, Cyclic Time: %.3fs\n", current_cycle, cyclic_time);

for (int i = 0; dynamic_prb_configs[i].enabled; i++) {

if (dynamic_prb_configs[i].ue_id == 0) {

LOG_I(NR_MAC, "Config %d: UNASSIGNED\n", i);

for (int j = 0; j < dynamic_prb_configs[i].num_configs; j++) {

LOG_I(NR_MAC, "  Time [%.1f-%.1f]s: PRB Start: %d, PRB Size: %d\n",

dynamic_prb_configs[i].time_configs[j].start_time,

dynamic_prb_configs[i].time_configs[j].end_time,

dynamic_prb_configs[i].time_configs[j].prb_start,

dynamic_prb_configs[i].time_configs[j].prb_size);

}

} else {

int cfg_idx = dynamic_prb_configs[i].current_config_idx;

LOG_I(NR_MAC, "Config %d: UE-ID: 0x%lx, Active Config: %d\n",

i, (unsigned long)dynamic_prb_configs[i].ue_id, cfg_idx);

if (cfg_idx >= 0 && cfg_idx < dynamic_prb_configs[i].num_configs) {

time_config_t *current = &dynamic_prb_configs[i].time_configs[cfg_idx];

LOG_I(NR_MAC, "  Current: PRB Start: %d, PRB Size: %d\n",

current->prb_start, current->prb_size);

}

for (int j = 0; j < dynamic_prb_configs[i].num_configs; j++) {

LOG_I(NR_MAC, "  Time [%.1f-%.1f]s: PRB Start: %d, PRB Size: %d\n",

dynamic_prb_configs[i].time_configs[j].start_time,

dynamic_prb_configs[i].time_configs[j].end_time,

dynamic_prb_configs[i].time_configs[j].prb_start,

dynamic_prb_configs[i].time_configs[j].prb_size);

}

}

}

LOG_I(NR_MAC, "==================================\n");

}

/* Get dynamic PRB configuration for a UE with cyclic time */

static dynamic_prb_config_t* get_dynamic_prb_config(const NR_UE_info_t *UE,

frame_t frame, slot_t slot) {

uint64_t key = (uint64_t)UE->rnti;

double cyclic_time = get_current_time_seconds(frame, slot);

int current_cycle = get_current_cycle(frame, slot);

/* Reclaim stale bindings from previous cycles so reconnecting UEs can rebind */

for (int i = 0; dynamic_prb_configs[i].enabled; i++) {

dynamic_prb_config_t *cfg = &dynamic_prb_configs[i];

if (cfg->ue_id != 0 && cfg->last_seen_cycle >= 0 && cfg->last_seen_cycle != current_cycle) {

LOG_I(NR_MAC, "Releasing stale dynamic PRB binding (UE-ID 0x%lx) from cycle %d (current %d)\n",

cfg->ue_id, cfg->last_seen_cycle, current_cycle);

cfg->ue_id = 0;

cfg->current_config_idx = 0;

cfg->last_cycle = -1;

cfg->last_seen_cycle = -1;

}

}

/* First, try to find an existing binding */

for (int i = 0; dynamic_prb_configs[i].enabled; i++) {

if (dynamic_prb_configs[i].ue_id == key) {

update_time_based_config(&dynamic_prb_configs[i], cyclic_time, current_cycle);

dynamic_prb_configs[i].last_seen_cycle = current_cycle;

return &dynamic_prb_configs[i];

}

}

/* Otherwise, bind the UE to the first free slot */

for (int i = 0; dynamic_prb_configs[i].enabled; i++) {

if (dynamic_prb_configs[i].ue_id == 0) {

dynamic_prb_configs[i].ue_id = key;

dynamic_prb_configs[i].last_cycle = current_cycle;

dynamic_prb_configs[i].last_seen_cycle = current_cycle;

update_time_based_config(&dynamic_prb_configs[i], cyclic_time, current_cycle);

LOG_I(NR_MAC, "Binding UE-ID 0x%lx to dynamic PRB config %d (RNTI 0x%04x) at cycle %d\n",

key, i, UE->rnti, current_cycle);

return &dynamic_prb_configs[i];

}

}

return NULL;  /* No dynamic slot available */

}

/* Get current PRB allocation for a UE with cycle information */

static bool get_current_prb_allocation(const NR_UE_info_t *UE, frame_t frame, slot_t slot,

int *prb_start, int *prb_size) {

dynamic_prb_config_t *config = get_dynamic_prb_config(UE, frame, slot);

if (!config || config->current_config_idx >= config->num_configs || config->current_config_idx < 0) {

LOG_D(NR_MAC, "[UE %04x] No dynamic config available (config=%p, idx=%d)\n",

UE->rnti, config, config ? config->current_config_idx : -1);

return false;

}

time_config_t *current = &config->time_configs[config->current_config_idx];

*prb_start = current->prb_start;

*prb_size = current->prb_size;

// Debug logging every 100 frames

static int last_debug_frame = -1;

if (frame % 100 == 0 && frame != last_debug_frame) {

LOG_I(NR_MAC, "[UE %04x] Dynamic PRB: start=%d, size=%d, config_idx=%d\n",

UE->rnti, *prb_start, *prb_size, config->current_config_idx);

last_debug_frame = frame;

}

return true;

}

/* Check if a PRB is within dynamic allocation for a UE */

static bool is_prb_in_dynamic_allocation(const NR_UE_info_t *UE, frame_t frame, slot_t slot, int prb_index) {

int prb_start, prb_size;

if (!get_current_prb_allocation(UE, frame, slot, &prb_start, &prb_size)) {

return false;

}

return (prb_index >= prb_start && prb_index < prb_start + prb_size);

}

/* Print current cycle status for debugging */

static void print_cycle_status(frame_t frame, slot_t slot) {

double cyclic_time = get_current_time_seconds(frame, slot);

int current_cycle = get_current_cycle(frame, slot);

LOG_D(NR_MAC, "=== Cycle Status ===\n");

LOG_D(NR_MAC, "Current Cycle: %d, Cyclic Time: %.3fs\n", current_cycle, cyclic_time);

for (int i = 0; dynamic_prb_configs[i].enabled; i++) {

if (dynamic_prb_configs[i].ue_id != 0) {

int cfg_idx = dynamic_prb_configs[i].current_config_idx;

if (cfg_idx >= 0 && cfg_idx < dynamic_prb_configs[i].num_configs) {

time_config_t *current = &dynamic_prb_configs[i].time_configs[cfg_idx];

LOG_D(NR_MAC, "UE-ID 0x%lx: Config %d, PRB start=%d, size=%d\n",

dynamic_prb_configs[i].ue_id, cfg_idx, current->prb_start, current->prb_size);

}

}

}

LOG_D(NR_MAC, "==================\n");

}

/* \brief Get the number of UL TDAs that could be used in slot, reachable

* via specific k2. The output parameter first_idx is a pointer to the first

* suitable TDA, and the function returns the number of suitable TDAs, or 0. */

int get_num_ul_tda(gNB_MAC_INST *nrmac, int slot, int k2, const NR_tda_info_t **first_idx)

{

/* we assume that this function is mutex-protected from outside */

NR_SCHED_ENSURE_LOCKED(&nrmac->sched_lock);

const frame_structure_t *fs = &nrmac->frame_structure;

const int slot_period = slot % fs->numb_slots_period;

const tdd_bitmap_t *bm = &fs->period_cfg.tdd_slot_bitmap[slot_period];

/* For some reason, we only store the number of symbols if it's mixed */

const int num_ul_symbols = bm->slot_type == TDD_NR_MIXED_SLOT ? bm->num_ul_symbols : 14;

const uint16_t ul_bitmap = SL_to_bitmap(14 - num_ul_symbols, num_ul_symbols);

*first_idx = NULL;

FOR_EACH_SEQ_ARR(NR_tda_info_t *, tda, &nrmac->ul_tda) {

DevAssert(tda->valid_tda);

// nr_rrc_config_ul_tda() orders by k2, so skip smaller and return for

// bigger ones

if (tda->k2 < k2)

continue;

if (tda->k2 > k2)

break; // there won't be a suitable k2 anymore

uint16_t tda_bitmap = SL_to_bitmap(tda->startSymbolIndex, tda->nrOfSymbols);

// nr_rrc_config_ul_tda() assures TDAs are from largest to smallest symbol number.

// check that this TDA fits entirely into this slot's mask (slot might be

// smaller in mixed slots)., A mixed slot TDA would be checked last, so a

// full slot TDA will be used for a full UL slot.

if ((tda_bitmap & ul_bitmap) == tda_bitmap) { // if TDA fits entiry

*first_idx = tda;

break;

}

}

if (*first_idx == NULL) /* nothing fit */

return 0;

NR_tda_info_t *end_it = seq_arr_next(&nrmac->ul_tda, *first_idx);

while (end_it != seq_arr_end(&nrmac->ul_tda) && end_it->k2 == k2) {

/* the following TDAs should all fit as long as the k2 is the same */

uint16_t tda_bitmap = SL_to_bitmap(end_it->startSymbolIndex, end_it->nrOfSymbols);

AssertFatal((tda_bitmap & ul_bitmap) == tda_bitmap,

"TDA should fit inside slot, but is not the case for k2 %ld bitmap 0x%04x\n",

end_it->k2,

tda_bitmap);

end_it = seq_arr_next(&nrmac->ul_tda, end_it);

}

ptrdiff_t diff = seq_arr_dist(&nrmac->ul_tda, *first_idx, end_it);

AssertFatal(diff > 0 && diff <= seq_arr_size(&nrmac->ul_tda), "dist %ld\n", diff);

return diff;

}

static void get_max_rb_range(const uint16_t *vrb_map_ul, const uint16_t *ulprbbl, uint16_t mask, int *rb_start, int *rb_len)

{

int best_start = *rb_start;

int best_len = 0;

int current_start = *rb_start;

int current_len = 0;

for (int rb = *rb_start; rb < *rb_len; ++rb) {

if (ulprbbl[rb] == 0 && (mask & vrb_map_ul[rb]) == 0) {

current_len++;

continue;

}

/* this RB is blocked for UL, or already in use */

if (current_len > best_len) {

best_start = current_start;

best_len = current_len;

}

current_start = rb + 1; /* will start in next RB, or updated later */

current_len = 0;

}

if (current_len > best_len) {

best_start = current_start;

best_len = current_len;

}

*rb_start = best_start;

*rb_len = best_len;

}

const NR_tda_info_t *get_best_ul_tda(const gNB_MAC_INST *nrmac, int beam, const NR_tda_info_t *tdas, int n_tda, int frame, int slot, int *rb_start, int *rb_len)

{

/* there is a mixed slot only when in TDD */

const frame_structure_t *fs = &nrmac->frame_structure;

const int index = ul_buffer_index(frame, slot, fs->numb_slots_frame, nrmac->vrb_map_UL_size);

uint16_t *vrb_map_UL = &nrmac->common_channels[0].vrb_map_UL[beam][index * MAX_BWP_SIZE];

DevAssert(n_tda <= 16);

const NR_tda_info_t *best_tda = tdas;

uint64_t score = 0;

int check_rb_start = *rb_start;

int check_rb_len = *rb_len;

for (int i = 0; i < n_tda; ++i, tdas++) {

int start = check_rb_start;

int len = check_rb_len;

uint16_t tda_mask = SL_to_bitmap(tdas->startSymbolIndex, tdas->nrOfSymbols);

get_max_rb_range(vrb_map_UL, nrmac->ulprbbl, tda_mask, &start, &len);

uint64_t s = (uint64_t)tdas->nrOfSymbols * len;

if (s > score) {

best_tda = tdas;

score = s;

*rb_start = start;

*rb_len = len;

}

LOG_D(NR_MAC, "%4d.%2d tda k2 %ld mask 0x%04x has PRB start/len %d/%d score %ld\n", frame, slot, tdas->k2, tda_mask, start, len, s);

}

return best_tda;

}

bwp_info_t get_pusch_bwp_start_size(NR_UE_info_t *UE)

{

NR_UE_UL_BWP_t *ul_bwp = &UE->current_UL_BWP;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

bwp_info_t bwp_info;

bwp_info.bwpStart = ul_bwp->BWPStart;

// 3GPP TS 38.214 Section 6.1.2.2.2 Uplink resource allocation type 1

// In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of

// contiguously allocated non-interleaved virtual resource blocks within the active bandwidth part of size   PRBs except for the

// case when DCI format 0_0 is decoded in any common search space in which case the size of the initial UL bandwidth part shall

// be used.

if (ul_bwp->dci_format == NR_UL_DCI_FORMAT_0_0 && sched_ctrl->search_space->searchSpaceType

&& sched_ctrl->search_space->searchSpaceType->present == NR_SearchSpace__searchSpaceType_PR_common) {

bwp_info.bwpSize = min(ul_bwp->BWPSize, UE->sc_info.initial_ul_BWPSize);

} else {

bwp_info.bwpSize = ul_bwp->BWPSize;

}

return bwp_info;

}

static int compute_ph_factor(int mu, int tbs_bits, int rb, int n_layers, int n_symbols, int n_dmrs, long *deltaMCS, bool include_bw)

{

// 38.213 7.1.1

// if the PUSCH transmission is over more than one layer delta_tf = 0

float delta_tf = 0;

if(deltaMCS != NULL && n_layers == 1) {

const int n_re = (NR_NB_SC_PER_RB * n_symbols - n_dmrs) * rb;

const float BPRE = (float) tbs_bits/n_re;  //TODO change for PUSCH with CSI

const float f = pow(2, BPRE * 1.25);

const float beta = 1.0f; //TODO change for PUSCH with CSI

delta_tf = (10 * log10((f - 1) * beta));

LOG_D(NR_MAC,

"PH factor delta_tf %f (n_re %d, n_rb %d, n_dmrs %d, n_symbols %d, tbs %d BPRE %f f %f)\n",

delta_tf,

n_re,

rb,

n_dmrs,

n_symbols,

tbs_bits,

BPRE,

f);

}

const float bw_factor = (include_bw) ? 10 * log10(rb << mu) : 0;

return ((int)roundf(delta_tf + bw_factor));

}

/* \brief over-estimate the BSR index, given real_index.

*

* BSR does not account for headers, so we need to estimate. See 38.321

* 6.1.3.1: "The size of the RLC headers and MAC subheaders are not considered

* in the buffer size computation." */

static int overestim_bsr_index(int real_index)

{

/* if UE reports BSR 0, it means "no data"; otherwise, overestimate to

* account for headers */

const int add_overestim = 1;

return real_index > 0 ? real_index + add_overestim : real_index;

}

static int estimate_ul_buffer_short_bsr(const NR_BSR_SHORT *bsr)

{

/* NOTE: the short BSR might be for different LCGID than 0, but we do not

* differentiate them */

int rep_idx = bsr->Buffer_size;

int estim_idx = overestim_bsr_index(rep_idx);

int max = NR_SHORT_BSR_TABLE_SIZE - 1;

int idx = min(estim_idx, max);

int estim_size = get_short_bsr_value(idx);

LOG_D(NR_MAC, "short BSR LCGID %d index %d estim index %d size %d\n", bsr->LcgID, rep_idx, estim_idx, estim_size);

return estim_size;

}

static int estimate_ul_buffer_long_bsr(const NR_BSR_LONG *bsr)

{

LOG_D(NR_MAC,

"LONG BSR, LCG ID(7-0) %d/%d/%d/%d/%d/%d/%d/%d\n",

bsr->LcgID7,

bsr->LcgID6,

bsr->LcgID5,

bsr->LcgID4,

bsr->LcgID3,

bsr->LcgID2,

bsr->LcgID1,

bsr->LcgID0);

bool bsr_active[8] = {bsr->LcgID0 != 0, bsr->LcgID1 != 0, bsr->LcgID2 != 0, bsr->LcgID3 != 0, bsr->LcgID4 != 0, bsr->LcgID5 != 0, bsr->LcgID6 != 0, bsr->LcgID7 != 0};

int estim_size = 0;

int max = NR_LONG_BSR_TABLE_SIZE - 1;

uint8_t *payload = ((uint8_t*) bsr) + 1;

int m = 0;

const int total_lcgids = 8; /* see 38.321 6.1.3.1 */

for (int n = 0; n < total_lcgids; n++) {

if (!bsr_active[n])

continue;

int rep_idx = payload[m];

int estim_idx = overestim_bsr_index(rep_idx);

int idx = min(estim_idx, max);

estim_size += get_long_bsr_value(idx);

LOG_D(NR_MAC, "LONG BSR LCGID/m %d/%d Index %d estim index %d size %d", n, m, rep_idx, estim_idx, estim_size);

m++;

}

return estim_size;

}

//  For both UL-SCH except:

//   - UL-SCH: fixed-size MAC CE(known by LCID)

//   - UL-SCH: padding

//   - UL-SCH: MSG3 48-bits

//  |0|1|2|3|4|5|6|7|  bit-wise

//  |R|F|   LCID    |

//  |       L       |

//  |0|1|2|3|4|5|6|7|  bit-wise

//  |R|F|   LCID    |

//  |       L       |

//  |       L       |

//

//  For:

//   - UL-SCH: fixed-size MAC CE(known by LCID)

//   - UL-SCH: padding, for single/multiple 1-oct padding CE(s)

//   - UL-SCH: MSG3 48-bits

//  |0|1|2|3|4|5|6|7|  bit-wise

//  |R|R|   LCID    |

//

//  LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

//  L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the L field is indicated by the F field;

//  F: length of L is 0:8 or 1:16 bits wide

//  R: Reserved bit, set to zero.

// return: length of subPdu header

// 3GPP TS 38.321 Section 6

uint8_t decode_ul_mac_sub_pdu_header(uint8_t *pduP, uint8_t *lcid, uint16_t *length)

{

uint16_t mac_subheader_len = 1;

*lcid = pduP[0] & 0x3F;

switch (*lcid) {

case UL_SCH_LCID_CCCH_64_BITS:

*length = 8;

break;

case UL_SCH_LCID_SRB1:

case UL_SCH_LCID_SRB2:

case UL_SCH_LCID_DTCH ...(UL_SCH_LCID_DTCH + 28):

case UL_SCH_LCID_L_TRUNCATED_BSR:

case UL_SCH_LCID_L_BSR:

if (pduP[0] & 0x40) { // F = 1

mac_subheader_len = 3;

*length = (pduP[1] << 8) + pduP[2];

} else { // F = 0

mac_subheader_len = 2;

*length = pduP[1];

}

break;

case UL_SCH_LCID_CCCH_48_BITS_REDCAP:

case UL_SCH_LCID_CCCH_48_BITS:

*length = 6;

break;

case UL_SCH_LCID_SINGLE_ENTRY_PHR:

case UL_SCH_LCID_C_RNTI:

*length = 2;

break;

case UL_SCH_LCID_S_TRUNCATED_BSR:

case UL_SCH_LCID_S_BSR:

*length = 1;

break;

case UL_SCH_LCID_PADDING:

// Nothing to do

break;

default:

LOG_E(NR_MAC, "LCID %0x not handled yet!\n", *lcid);

break;

}

LOG_D(NR_MAC, "Decoded LCID 0x%X, header bytes: %d, payload bytes %d\n", *lcid, mac_subheader_len, *length);

return mac_subheader_len;

}

static rnti_t lcid_crnti_lookahead(uint8_t *pdu, uint32_t pdu_len)

{

while (pdu_len > 0) {

uint16_t mac_len = 0;

uint8_t lcid = 0;

uint16_t mac_subheader_len = decode_ul_mac_sub_pdu_header(pdu, &lcid, &mac_len);

// Check for valid PDU

if (mac_subheader_len + mac_len > pdu_len) {

LOG_E(NR_MAC,

"Invalid PDU! mac_subheader_len: %d, mac_len: %d, remaining pdu_len: %d\n",

mac_subheader_len,

mac_len,

pdu_len);

LOG_E(NR_MAC, "Residual UL MAC PDU: ");

uint32_t print_len = pdu_len > 30 ? 30 : pdu_len; // Only printf 1st - 30nd bytes

for (int i = 0; i < print_len; i++)

printf("%02x ", pdu[i]);

printf("\n");

return 0;

}

if (lcid == UL_SCH_LCID_C_RNTI) {

// Extract C-RNTI value

rnti_t crnti = ((pdu[1] & 0xFF) << 8) | (pdu[2] & 0xFF);

LOG_A(NR_MAC, "Received a MAC CE for C-RNTI with %04x\n", crnti);

return crnti;

} else if (lcid == UL_SCH_LCID_PADDING) {

// End of MAC PDU, can ignore the remaining bytes

return 0;

}

pdu += mac_len + mac_subheader_len;

pdu_len -= mac_len + mac_subheader_len;

}

return 0;

}

static int nr_process_mac_pdu(instance_t module_idP,

NR_UE_info_t *UE,

uint8_t CC_id,

frame_t frameP,

slot_t slot,

uint8_t *pduP,

uint32_t pdu_len,

const int8_t harq_pid)

{

int sdus = 0;

NR_UE_UL_BWP_t *ul_bwp = &UE->current_UL_BWP;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

if (pduP[0] != UL_SCH_LCID_PADDING) {

ws_trace_t tmp = {.nr = true,

.direction = DIRECTION_UPLINK,

.pdu_buffer = pduP,

.pdu_buffer_size = pdu_len,

.ueid = 0,

.rntiType = WS_C_RNTI,

.rnti = UE->rnti,

.sysFrame = frameP,

.subframe = slot,

.harq_pid = harq_pid};

trace_pdu(&tmp);

}

#ifdef ENABLE_MAC_PAYLOAD_DEBUG

LOG_I(NR_MAC, "In %s: dumping MAC PDU in %d.%d:\n", __func__, frameP, slot);

log_dump(NR_MAC, pduP, pdu_len, LOG_DUMP_CHAR, "\n");

#endif

while (pdu_len > 0) {

uint16_t mac_len = 0;

uint8_t lcid = 0;

uint16_t mac_subheader_len = decode_ul_mac_sub_pdu_header(pduP, &lcid, &mac_len);

// Check for valid PDU

if (mac_subheader_len + mac_len > pdu_len) {

LOG_E(NR_MAC,

"Invalid PDU in %d.%d for RNTI %04X! mac_subheader_len: %d, mac_len: %d, remaining pdu_len: %d\n",

frameP,

slot,

UE->rnti,

mac_subheader_len,

mac_len,

pdu_len);

LOG_E(NR_MAC, "Residual UL MAC PDU: ");

int print_len = pdu_len > 30 ? 30 : pdu_len; // Only printf 1st - 30nd bytes

for (int i = 0; i < print_len; i++)

printf("%02x ", pduP[i]);

printf("\n");

return 0;

}

LOG_D(NR_MAC,

"Received UL-SCH sub-PDU with LCID 0x%X in %d.%d for RNTI %04X (remaining PDU length %d)\n",

lcid,

frameP,

slot,

UE->rnti,

pdu_len);

unsigned char *ce_ptr;

switch (lcid) {

case UL_SCH_LCID_CCCH_64_BITS:

case UL_SCH_LCID_CCCH_48_BITS_REDCAP:

case UL_SCH_LCID_CCCH_48_BITS:

if (lcid == UL_SCH_LCID_CCCH_64_BITS) {

// Check if it is a valid CCCH1 message, we get all 00's messages very often

bool valid_pdu = false;

for (int i = 0; i < (mac_subheader_len + mac_len); i++) {

if (pduP[i] != 0) {

valid_pdu = true;

break;

}

}

if (!valid_pdu) {

LOG_D(NR_MAC, "%s() Invalid CCCH1 message!, pdu_len: %d\n", __func__, pdu_len);

return 0;

}

}

LOG_I(MAC, "[RAPROC] Received SDU for CCCH length %d for UE %04x\n", mac_len, UE->rnti);

if (lcid == UL_SCH_LCID_CCCH_48_BITS_REDCAP) {

LOG_I(MAC, "UE with RNTI %04x is RedCap\n", UE->rnti);

UE->is_redcap = true;

}

if (prepare_initial_ul_rrc_message(RC.nrmac[module_idP], UE)) {

nr_mac_rlc_data_ind(module_idP, UE->rnti, true, 0, (char *)(pduP + mac_subheader_len), mac_len);

} else {

LOG_E(NR_MAC, "prepare_initial_ul_rrc_message() returned false, cannot forward CCCH message\n");

}

break;

case UL_SCH_LCID_SRB1:

case UL_SCH_LCID_SRB2:

AssertFatal(UE->CellGroup,

"UE %04x %d.%d: Received LCID %d which is not configured (UE has no CellGroup)\n",

UE->rnti,

frameP,

slot,

lcid);

const nr_lc_config_t *srbc = nr_mac_get_lc_config(sched_ctrl, lcid);

if (!srbc || srbc->suspended) {

LOG_I(NR_MAC, "RNTI %04x LCID %d: ignoring %d bytes\n", UE->rnti, lcid, mac_len);

} else {

nr_mac_rlc_data_ind(module_idP, UE->rnti, true, lcid, (char *)(pduP + mac_subheader_len), mac_len);

UE->mac_stats.ul.total_sdu_bytes += mac_len;

UE->mac_stats.ul.lc_bytes[lcid] += mac_len;

}

T(T_GNB_MAC_LCID_UL, T_INT(UE->rnti), T_INT(frameP), T_INT(slot), T_INT(lcid), T_INT(mac_len * 8));

break;

case UL_SCH_LCID_DTCH ...(UL_SCH_LCID_DTCH + 28):

LOG_D(NR_MAC,

"[UE %04x] %d.%d : ULSCH -> UL-%s %d (gNB %ld, %d bytes)\n",

UE->rnti,

frameP,

slot,

lcid < 4 ? "DCCH" : "DTCH",

lcid,

module_idP,

mac_len);

const nr_lc_config_t *c = nr_mac_get_lc_config(sched_ctrl, lcid);

if (!c || c->suspended) {

LOG_I(NR_MAC, "RNTI %04x LCID %d: ignoring %d bytes\n", UE->rnti, lcid, mac_len);

} else {

UE->mac_stats.ul.lc_bytes[lcid] += mac_len;

nr_mac_rlc_data_ind(module_idP, UE->rnti, true, lcid, (char *)(pduP + mac_subheader_len), mac_len);

sdus += 1;

/* Updated estimated buffer when receiving data */

if (sched_ctrl->estimated_ul_buffer >= mac_len)

sched_ctrl->estimated_ul_buffer -= mac_len;

else

sched_ctrl->estimated_ul_buffer = 0;

}

T(T_GNB_MAC_LCID_UL, T_INT(UE->rnti), T_INT(frameP), T_INT(slot), T_INT(lcid), T_INT(mac_len * 8));

break;

case UL_SCH_LCID_RECOMMENDED_BITRATE_QUERY:

// 38.321 Ch6.1.3.20

break;

case UL_SCH_LCID_MULTI_ENTRY_PHR_4_OCT:

LOG_E(NR_MAC, "Multi entry PHR not supported\n");

break;

case UL_SCH_LCID_CONFIGURED_GRANT_CONFIRMATION:

// 38.321 Ch6.1.3.7

break;

case UL_SCH_LCID_MULTI_ENTRY_PHR_1_OCT:

LOG_E(NR_MAC, "Multi entry PHR not supported\n");

break;

case UL_SCH_LCID_SINGLE_ENTRY_PHR:

if (harq_pid < 0) {

LOG_E(NR_MAC, "Invalid HARQ PID %d\n", harq_pid);

return 0;

}

NR_sched_pusch_t *sched_pusch = &sched_ctrl->ul_harq_processes[harq_pid].sched_pusch;

/* Extract SINGLE ENTRY PHR elements for PHR calculation */

ce_ptr = &pduP[mac_subheader_len];

NR_SINGLE_ENTRY_PHR_MAC_CE *phr = (NR_SINGLE_ENTRY_PHR_MAC_CE *)ce_ptr;

/* Save the phr info */

int PH;

const int PCMAX = phr->PCMAX;

/* 38.133 Table10.1.17.1-1 */

if (phr->PH < 55) {

PH = phr->PH - 32;

} else if (phr->PH < 63) {

PH = 24 + (phr->PH - 55) * 2;

} else {

PH = 38;

}

// in sched_ctrl we set normalized PH wrt MCS and PRBs

long *deltaMCS = ul_bwp->pusch_Config ? ul_bwp->pusch_Config->pusch_PowerControl->deltaMCS : NULL;

sched_ctrl->ph = PH

+ compute_ph_factor(ul_bwp->scs,

sched_pusch->tb_size << 3,

sched_pusch->rbSize,

sched_pusch->nrOfLayers,

sched_pusch->tda_info.nrOfSymbols, // n_symbols

sched_pusch->dmrs_info.num_dmrs_symb * sched_pusch->dmrs_info.N_PRB_DMRS, // n_dmrs

deltaMCS,

true);

sched_ctrl->ph0 = PH;

/* 38.133 Table10.1.18.1-1 */

sched_ctrl->pcmax = PCMAX - 29;

LOG_D(NR_MAC,

"SINGLE ENTRY PHR %d.%d R1 %d PH %d (%d dB) R2 %d PCMAX %d (%d dBm)\n",

frameP,

slot,

phr->R1,

PH,

sched_ctrl->ph,

phr->R2,

PCMAX,

sched_ctrl->pcmax);

break;

case UL_SCH_LCID_C_RNTI:

if (UE->ra && UE->ra->ra_state == nrRA_gNB_IDLE) {

// Extract C-RNTI value

rnti_t crnti = ((pduP[1] & 0xFF) << 8) | (pduP[2] & 0xFF);

AssertFatal(false,

"Received MAC CE for C-RNTI %04x without RA running, procedure exists? Or is it a bug while decoding the "

"MAC PDU?\n",

crnti);

}

break;

case UL_SCH_LCID_S_TRUNCATED_BSR:

case UL_SCH_LCID_S_BSR:

/* Extract short BSR value */

ce_ptr = &pduP[mac_subheader_len];

sched_ctrl->estimated_ul_buffer = estimate_ul_buffer_short_bsr((NR_BSR_SHORT *)ce_ptr);

sched_ctrl->sched_ul_bytes = 0;

LOG_D(NR_MAC, "SHORT BSR at %4d.%2d, est buf %d\n", frameP, slot, sched_ctrl->estimated_ul_buffer);

break;

case UL_SCH_LCID_L_TRUNCATED_BSR:

case UL_SCH_LCID_L_BSR:

/* Extract long BSR value */

ce_ptr = &pduP[mac_subheader_len];

sched_ctrl->estimated_ul_buffer = estimate_ul_buffer_long_bsr((NR_BSR_LONG *)ce_ptr);

sched_ctrl->sched_ul_bytes = 0;

LOG_D(NR_MAC, "LONG BSR at %4d.%2d, estim buf %d\n", frameP, slot, sched_ctrl->estimated_ul_buffer);

break;

case UL_SCH_LCID_PADDING:

// End of MAC PDU, can ignore the rest.

return 0;

default:

LOG_E(NR_MAC, "RNTI %0x [%d.%d], received unknown MAC header (LCID = 0x%02x)\n", UE->rnti, frameP, slot, lcid);

return -1;

break;

}

#ifdef ENABLE_MAC_PAYLOAD_DEBUG

if (lcid < 45 || lcid == 52 || lcid == 63) {

LOG_I(NR_MAC, "In %s: dumping UL MAC SDU sub-header with length %d (LCID = 0x%02x):\n", __func__, mac_subheader_len, lcid);

log_dump(NR_MAC, pduP, mac_subheader_len, LOG_DUMP_CHAR, "\n");

LOG_I(NR_MAC, "In %s: dumping UL MAC SDU with length %d (LCID = 0x%02x):\n", __func__, mac_len, lcid);

log_dump(NR_MAC, pduP + mac_subheader_len, mac_len, LOG_DUMP_CHAR, "\n");

} else {

LOG_I(NR_MAC, "In %s: dumping UL MAC CE with length %d (LCID = 0x%02x):\n", __func__, mac_len, lcid);

log_dump(NR_MAC, pduP + mac_subheader_len + mac_len, mac_len, LOG_DUMP_CHAR, "\n");

}

#endif

pduP += (mac_subheader_len + mac_len);

pdu_len -= (mac_subheader_len + mac_len);

}

UE->mac_stats.ul.num_mac_sdu += sdus;

return 0;

}

static void finish_nr_ul_harq(NR_UE_sched_ctrl_t *sched_ctrl, int harq_pid)

{

NR_UE_ul_harq_t *harq = &sched_ctrl->ul_harq_processes[harq_pid];

harq->ndi ^= 1;

harq->round = 0;

add_tail_nr_list(&sched_ctrl->available_ul_harq, harq_pid);

}

static void abort_nr_ul_harq(NR_UE_info_t *UE, int8_t harq_pid)

{

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_ul_harq_t *harq = &sched_ctrl->ul_harq_processes[harq_pid];

finish_nr_ul_harq(sched_ctrl, harq_pid);

UE->mac_stats.ul.errors++;

/* the transmission failed: the UE won't send the data we expected initially,

* so retrieve to correctly schedule after next BSR */

sched_ctrl->sched_ul_bytes -= harq->sched_pusch.tb_size;

if (sched_ctrl->sched_ul_bytes < 0)

sched_ctrl->sched_ul_bytes = 0;

}

static void handle_nr_ul_harq(gNB_MAC_INST *nrmac,

NR_UE_info_t *UE,

frame_t frame,

slot_t slot,

rnti_t rnti,

int crc_harq_id,

bool crc_status)

{

if (nrmac->radio_config.disable_harq) {

LOG_D(NR_MAC, "skipping UL feedback handling as HARQ is disabled\n");

return;

}

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

int8_t harq_pid = sched_ctrl->feedback_ul_harq.head;

LOG_D(NR_MAC, "Comparing crc harq_id vs feedback harq_pid = %d %d\n", crc_harq_id, harq_pid);

while (crc_harq_id != harq_pid || harq_pid < 0) {

LOG_W(NR_MAC, "Unexpected ULSCH HARQ PID %d (have %d) for RNTI 0x%04x\n", crc_harq_id, harq_pid, rnti);

if (harq_pid < 0)

return;

remove_front_nr_list(&sched_ctrl->feedback_ul_harq);

sched_ctrl->ul_harq_processes[harq_pid].is_waiting = false;

if(sched_ctrl->ul_harq_processes[harq_pid].round >= nrmac->ul_bler.harq_round_max - 1) {

abort_nr_ul_harq(UE, harq_pid);

} else {

sched_ctrl->ul_harq_processes[harq_pid].round++;

add_tail_nr_list(&sched_ctrl->retrans_ul_harq, harq_pid);

}

harq_pid = sched_ctrl->feedback_ul_harq.head;

}

remove_front_nr_list(&sched_ctrl->feedback_ul_harq);

NR_UE_ul_harq_t *harq = &sched_ctrl->ul_harq_processes[harq_pid];

DevAssert(harq->is_waiting);

harq->feedback_slot = -1;

harq->is_waiting = false;

if (!crc_status) {

finish_nr_ul_harq(sched_ctrl, harq_pid);

LOG_D(NR_MAC,

"Ulharq id %d crc passed for RNTI %04x\n",

harq_pid,

rnti);

} else if (harq->round >= nrmac->ul_bler.harq_round_max  - 1) {

abort_nr_ul_harq(UE, harq_pid);

LOG_D(NR_MAC,

"RNTI %04x: Ulharq id %d crc failed in all rounds\n",

rnti,

harq_pid);

} else {

harq->round++;

LOG_D(NR_MAC,

"Ulharq id %d crc failed for RNTI %04x\n",

harq_pid,

rnti);

add_tail_nr_list(&sched_ctrl->retrans_ul_harq, harq_pid);

}

}

static void handle_msg3_failed_rx(gNB_MAC_INST *mac, NR_RA_t *ra, rnti_t rnti, int harq_round_max)

{

// for CFRA (NSA) do not schedule retransmission of msg3

if (ra->cfra) {

LOG_W(NR_MAC, "UE %04x RA failed at state %s (NSA msg3 reception failed)\n", rnti, nrra_text[ra->ra_state]);

nr_release_ra_UE(mac, rnti);

return;

}

if (ra->msg3_round >= harq_round_max - 1) {

LOG_W(NR_MAC, "UE %04x RA failed at state %s (Reached msg3 max harq rounds)\n", rnti, nrra_text[ra->ra_state]);

nr_release_ra_UE(mac, rnti);

return;

}

LOG_D(NR_MAC, "UE %04x Msg3 CRC did not pass\n", rnti);

ra->msg3_round++;

ra->ra_state = nrRA_Msg3_retransmission;

}

static void nr_rx_ra_sdu(const module_id_t mod_id,

const int CC_id,

const frame_t frame,

const sub_frame_t slot,

rnti_t rnti,

uint8_t *sdu,

const uint32_t sdu_len,

uint8_t harq_pid,

const uint16_t timing_advance,

const uint8_t ul_cqi,

const uint16_t rssi)

{

gNB_MAC_INST *mac = RC.nrmac[mod_id];

NR_UE_info_t *UE = find_ra_UE(&mac->UE_info, rnti);

if (!UE) {

LOG_E(NR_MAC, "UL SDU discarded. Couldn't finde UE with RNTI %04x \n", rnti);

return;

}

NR_RA_t *ra = UE->ra;

if (ra->ra_type == RA_4_STEP && ra->ra_state != nrRA_WAIT_Msg3) {

LOG_W(NR_MAC, "UL SDU discarded for RNTI %04x RA state not waiting for PUSCH\n", UE->rnti);

return;

}

const int target_snrx10 = mac->pusch_target_snrx10;

if (!sdu) { // NACK

if (ra->ra_state != nrRA_WAIT_Msg3)

return;

if((frame!=ra->Msg3_frame) || (slot!=ra->Msg3_slot))

return;

if (ul_cqi != 0xff)

ra->msg3_TPC = nr_get_tpc(target_snrx10, ul_cqi, 30, 0);

handle_msg3_failed_rx(mac, ra, rnti, mac->ul_bler.harq_round_max);

return;

}

bool no_sig = true;

for (uint32_t k = 0; k < sdu_len; k++) {

if (sdu[k] != 0) {

no_sig = false;

break;

}

}

T(T_GNB_MAC_UL_PDU_WITH_DATA, T_INT(mod_id), T_INT(CC_id),

T_INT(rnti), T_INT(frame), T_INT(slot), T_INT(-1) /* harq_pid */,

T_BUFFER(sdu, sdu_len));

if (no_sig) {

LOG_W(NR_MAC, "MSG3 ULSCH with no signal\n");

handle_msg3_failed_rx(mac, ra, rnti, mac->ul_bler.harq_round_max);

return;

}

if (ra->ra_type == RA_2_STEP) {

// random access pusch with RA-RNTI

if (ra->RA_rnti != rnti) {

LOG_E(NR_MAC, "expected TC_RNTI %04x to match current RNTI %04x\n", ra->RA_rnti, rnti);

return;

}

}

// re-initialize ta update variables after RA procedure completion

UE->UE_sched_ctrl.ta_frame = frame;

LOG_A(NR_MAC, "%4d.%2d PUSCH with TC_RNTI 0x%04x received correctly\n", frame, slot, rnti);

NR_UE_sched_ctrl_t *UE_scheduling_control = &UE->UE_sched_ctrl;

DevAssert(harq_pid >= 0 && harq_pid < 8);

if (ul_cqi != 0xff) {

NR_UE_ul_harq_t *harq = &UE_scheduling_control->ul_harq_processes[harq_pid];

UE_scheduling_control->tpc0 = nr_get_tpc(target_snrx10, ul_cqi, 30, harq->sched_pusch.phr_txpower_calc);

UE_scheduling_control->pusch_snrx10 = ul_cqi * 5 - 640 - harq->sched_pusch.phr_txpower_calc * 10;

}

if (timing_advance != 0xffff)

UE_scheduling_control->ta_update = timing_advance;

UE_scheduling_control->raw_rssi = rssi;

LOG_D(NR_MAC, "[UE %04x] PUSCH TPC %d and TA %d\n", UE->rnti, UE_scheduling_control->tpc0, UE_scheduling_control->ta_update);

NR_ServingCellConfigCommon_t *scc = mac->common_channels[0].ServingCellConfigCommon;

if (ra->cfra) {

LOG_A(NR_MAC, "(rnti 0x%04x) CFRA procedure succeeded!\n", UE->rnti);

nr_mac_reset_ul_failure(UE_scheduling_control);

reset_dl_harq_list(UE_scheduling_control);

reset_ul_harq_list(UE_scheduling_control);

process_addmod_bearers_cellGroupConfig(&UE->UE_sched_ctrl, UE->CellGroup->rlc_BearerToAddModList);

int ss_type;

// we configure the UE using common search space with DCIX0 while waiting for a reconfiguration in SA

// in NSA (or do-ra) there is no reconfiguration in NR

if (IS_SA_MODE(get_softmodem_params()))

ss_type = NR_SearchSpace__searchSpaceType_PR_common;

else

ss_type = NR_SearchSpace__searchSpaceType_PR_ue_Specific;

configure_UE_BWP(mac, scc, UE, false, ss_type, -1, -1);

if (!transition_ra_connected_nr_ue(mac, UE)) {

LOG_E(NR_MAC, "cannot add UE %04x: list is full\n", UE->rnti);

delete_nr_ue_data(UE, NULL, &mac->UE_info.uid_allocator);

return;

}

} else {

LOG_D(NR_MAC, "[RAPROC] Received %s:\n", ra->ra_type == RA_2_STEP ? "MsgA-PUSCH" : "Msg3");

for (uint32_t k = 0; k < sdu_len; k++) {

LOG_D(NR_MAC, "(%i): 0x%x\n", k, sdu[k]);

}

// 3GPP TS 38.321 Section 5.4.3 Multiplexing and assembly

// Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

// - MAC CE for C-RNTI, or data from UL-CCCH;

// This way, we need to process MAC CE for C-RNTI if RA is active and it is present in the MAC PDU

// Search for MAC CE for C-RNTI

rnti_t crnti = lcid_crnti_lookahead(sdu, sdu_len);

if (crnti != 0) { // 3GPP TS 38.321 Table 7.1-1: RNTI values, RNTI 0x0000: N/A

// Replace the current UE by the UE identified by C-RNTI

NR_UE_info_t *old_UE = find_nr_UE(&mac->UE_info, crnti);

if (!old_UE) {

// The UE identified by C-RNTI no longer exists at the gNB

// Let's abort the current RA, so the UE will trigger a new RA later but using RRCSetupRequest instead. A better

// solution may be implemented

LOG_W(NR_MAC, "No UE found with C-RNTI %04x, ignoring Msg3 to have UE come back with new RA attempt\n", UE->rnti);

nr_release_ra_UE(mac, rnti);

return;

}

// in case UE beam has changed

old_UE->UE_beam_index = UE->UE_beam_index;

// Reset UL failure for old UE

nr_mac_reset_ul_failure(&old_UE->UE_sched_ctrl);

// Reset HARQ processes

reset_dl_harq_list(&old_UE->UE_sched_ctrl);

reset_ul_harq_list(&old_UE->UE_sched_ctrl);

// Only trigger RRCReconfiguration if UE is not performing RRCReestablishment

// The RRCReconfiguration will be triggered by the RRCReestablishmentComplete

if (!old_UE->reconfigSpCellConfig) {

LOG_I(NR_MAC, "Received UL_SCH_LCID_C_RNTI with C-RNTI 0x%04x, triggering RRC Reconfiguration\n", crnti);

// Trigger RRCReconfiguration

nr_mac_trigger_reconfiguration(mac, old_UE, -1);

// we configure the UE using common search space with DCIX0 while waiting for a reconfiguration

configure_UE_BWP(mac, scc, old_UE, false, NR_SearchSpace__searchSpaceType_PR_common, -1, -1);

}

nr_release_ra_UE(mac, rnti);

LOG_A(NR_MAC, "%4d.%2d RA with C-RNTI %04x complete\n", frame, slot, crnti);

// Decode the entire MAC PDU

// It may have multiple MAC subPDUs, for example, a MAC subPDU with LCID 1 caring a RRCReestablishmentComplete

nr_process_mac_pdu(mod_id, old_UE, CC_id, frame, slot, sdu, sdu_len, -1);

return;

}

// UE Contention Resolution Identity

// Store the first 48 bits belonging to the uplink CCCH SDU within Msg3 to fill in Msg4

// First byte corresponds to R/LCID MAC sub-header

memcpy(ra->cont_res_id, &sdu[1], sizeof(uint8_t) * 6);

// Decode MAC PDU

// the function is only called to decode the contention resolution sub-header

// harq_pid set a non-valid value because it is not used in this call

nr_process_mac_pdu(mod_id, UE, CC_id, frame, slot, sdu, sdu_len, -1);

LOG_I(NR_MAC,

"Activating scheduling %s for TC_RNTI 0x%04x (state %s)\n",

ra->ra_type == RA_2_STEP ? "MsgB" : "Msg4",

UE->rnti,

nrra_text[ra->ra_state]);

ra->ra_state = ra->ra_type == RA_2_STEP ? nrRA_MsgB : nrRA_Msg4;

LOG_D(NR_MAC, "TC_RNTI 0x%04x next RA state %s\n", UE->rnti, nrra_text[ra->ra_state]);

return;

}

}

static void _nr_rx_sdu(const module_id_t gnb_mod_idP,

const int CC_idP,

const frame_t frameP,

const slot_t slotP,

const rnti_t rntiP,

uint8_t *sduP,

const uint32_t sdu_lenP,

const int8_t harq_pid,

const uint16_t timing_advance,

const uint8_t ul_cqi,

const uint16_t rssi)

{

gNB_MAC_INST *gNB_mac = RC.nrmac[gnb_mod_idP];

const int current_rnti = rntiP;

LOG_D(NR_MAC, "rx_sdu for rnti %04x\n", current_rnti);

const int target_snrx10 = gNB_mac->pusch_target_snrx10;

const int rssi_threshold = gNB_mac->pusch_rssi_threshold;

const int pusch_failure_thres = gNB_mac->pusch_failure_thres;

NR_UE_info_t *UE = find_nr_UE(&gNB_mac->UE_info, current_rnti);

if (UE) {

NR_UE_sched_ctrl_t *UE_scheduling_control = &UE->UE_sched_ctrl;

if (sduP)

T(T_GNB_MAC_UL_PDU_WITH_DATA, T_INT(gnb_mod_idP), T_INT(CC_idP),

T_INT(rntiP), T_INT(frameP), T_INT(slotP), T_INT(harq_pid),

T_BUFFER(sduP, sdu_lenP));

UE->mac_stats.ul.total_bytes += sdu_lenP;

LOG_D(NR_MAC, "[gNB %d][PUSCH %d] CC_id %d %d.%d Received ULSCH sdu from PHY (rnti %04x) ul_cqi %d TA %d sduP %p, rssi %d\n",

gnb_mod_idP,

harq_pid,

CC_idP,

frameP,

slotP,

current_rnti,

ul_cqi,

timing_advance,

sduP,

rssi);

if (harq_pid < 0) {

LOG_E(NR_MAC, "UE %04x received ULSCH when feedback UL HARQ %d (unexpected ULSCH transmission)\n", rntiP, harq_pid);

return;

}

// if not missed detection (10dB threshold for now)

if (rssi > 0) {

int txpower_calc = UE_scheduling_control->ul_harq_processes[harq_pid].sched_pusch.phr_txpower_calc;

UE->mac_stats.deltaMCS = txpower_calc;

UE->mac_stats.NPRB = UE_scheduling_control->ul_harq_processes[harq_pid].sched_pusch.rbSize;

if (ul_cqi != 0xff)

UE_scheduling_control->tpc0 = nr_get_tpc(target_snrx10, ul_cqi, 30, txpower_calc);

if (UE_scheduling_control->ph < 0 && UE_scheduling_control->tpc0 > 1)

UE_scheduling_control->tpc0 = 1;

UE_scheduling_control->tpc0 = nr_limit_tpc(UE_scheduling_control->tpc0, rssi, rssi_threshold);

if (timing_advance != 0xffff)

UE_scheduling_control->ta_update = timing_advance;

UE_scheduling_control->raw_rssi = rssi;

UE_scheduling_control->pusch_snrx10 = ul_cqi * 5 - 640 - (txpower_calc * 10);

if (UE_scheduling_control->tpc0 > 1)

LOG_D(NR_MAC,

"[UE %04x] %d.%d. PUSCH TPC %d and TA %d pusch_snrx10 %d rssi %d phrx_tx_power %d PHR (1PRB) %d mcs %d, nb_rb %d\n",

UE->rnti,

frameP,

slotP,

UE_scheduling_control->tpc0,

UE_scheduling_control->ta_update,

UE_scheduling_control->pusch_snrx10,

UE_scheduling_control->raw_rssi,

txpower_calc,

UE_scheduling_control->ph,

UE_scheduling_control->ul_harq_processes[harq_pid].sched_pusch.mcs,

UE_scheduling_control->ul_harq_processes[harq_pid].sched_pusch.rbSize);

NR_UE_ul_harq_t *cur_harq = &UE_scheduling_control->ul_harq_processes[harq_pid];

if (cur_harq->round == 0)

UE->mac_stats.pusch_snrx10 = UE_scheduling_control->pusch_snrx10;

LOG_D(NR_MAC, "[UE %04x] PUSCH TPC %d and TA %d\n",UE->rnti,UE_scheduling_control->tpc0,UE_scheduling_control->ta_update);

}

else{

LOG_D(NR_MAC,"[UE %04x] Detected DTX : increasing UE TX power\n",UE->rnti);

UE_scheduling_control->tpc0 = 1;

}

#if defined(ENABLE_MAC_PAYLOAD_DEBUG)

LOG_I(NR_MAC, "Printing received UL MAC payload at gNB side: %d \n");

for (uint32_t i = 0; i < sdu_lenP; i++) {

// harq_process_ul_ue->a[i] = (unsigned char) rand();

// printf("a[%d]=0x%02x\n",i,harq_process_ul_ue->a[i]);

printf("%02x ", (unsigned char)sduP[i]);

}

printf("\n");

#endif

if (sduP != NULL) {

LOG_D(NR_MAC, "Received PDU at MAC gNB \n");

UE->UE_sched_ctrl.pusch_consecutive_dtx_cnt = 0;

UE_scheduling_control->sched_ul_bytes -= sdu_lenP;

if (UE_scheduling_control->sched_ul_bytes < 0)

UE_scheduling_control->sched_ul_bytes = 0;

nr_process_mac_pdu(gnb_mod_idP, UE, CC_idP, frameP, slotP, sduP, sdu_lenP, harq_pid);

} else {

if (ul_cqi == 0xff || ul_cqi <= 128) {

UE->UE_sched_ctrl.pusch_consecutive_dtx_cnt++;

UE->mac_stats.ulsch_DTX++;

}

if (!get_softmodem_params()->phy_test && UE->UE_sched_ctrl.pusch_consecutive_dtx_cnt >= pusch_failure_thres) {

LOG_W(NR_MAC,

"UE %04x: Detected UL Failure on PUSCH after %d PUSCH DTX, stopping scheduling\n",

UE->rnti,

UE->UE_sched_ctrl.pusch_consecutive_dtx_cnt);

nr_mac_trigger_ul_failure(&UE->UE_sched_ctrl, UE->current_UL_BWP.scs);

}

}

handle_nr_ul_harq(gNB_mac, UE, frameP, slotP, current_rnti, harq_pid, sduP == NULL);

} else {

nr_rx_ra_sdu(gnb_mod_idP, CC_idP, frameP, slotP, current_rnti, sduP, sdu_lenP, harq_pid, timing_advance, ul_cqi, rssi);

}

}

void nr_rx_sdu(const module_id_t gnb_mod_idP,

const int CC_idP,

const frame_t frameP,

const slot_t slotP,

const rnti_t rntiP,

uint8_t *sduP,

const uint32_t sdu_lenP,

const int8_t harq_pid,

const uint16_t timing_advance,

const uint8_t ul_cqi,

const uint16_t rssi)

{

gNB_MAC_INST *gNB_mac = RC.nrmac[gnb_mod_idP];

NR_SCHED_LOCK(&gNB_mac->sched_lock);

start_meas(&gNB_mac->rx_ulsch_sdu);

_nr_rx_sdu(gnb_mod_idP, CC_idP, frameP, slotP, rntiP, sduP, sdu_lenP, harq_pid, timing_advance, ul_cqi, rssi);

stop_meas(&gNB_mac->rx_ulsch_sdu);

NR_SCHED_UNLOCK(&gNB_mac->sched_lock);

}

static uint32_t calc_power_complex(const int16_t *x, const int16_t *y, const uint32_t size)

{

// Real part value

int64_t sum_x = 0;

int64_t sum_x2 = 0;

for(int k = 0; k<size; k++) {

sum_x = sum_x + x[k];

sum_x2 = sum_x2 + x[k]*x[k];

}

uint32_t power_re = sum_x2/size - (sum_x/size)*(sum_x/size);

// Imaginary part power

int64_t sum_y = 0;

int64_t sum_y2 = 0;

for(int k = 0; k<size; k++) {

sum_y = sum_y + y[k];

sum_y2 = sum_y2 + y[k]*y[k];

}

uint32_t power_im = sum_y2/size - (sum_y/size)*(sum_y/size);

return power_re+power_im;

}

static c16_t nr_h_times_w(c16_t h, char w)

{

c16_t output;

switch (w) {

case '0': // 0

output.r = 0;

output.i = 0;

break;

case '1': // 1

output.r = h.r;

output.i = h.i;

break;

case 'n': // -1

output.r = -h.r;

output.i = -h.i;

break;

case 'j': // j

output.r = -h.i;

output.i = h.r;

break;

case 'o': // -j

output.r = h.i;

output.i = -h.r;

break;

default:

AssertFatal(1==0,"Invalid precoder value %c\n", w);

}

return output;

}

static uint8_t get_max_tpmi(const NR_PUSCH_Config_t *pusch_Config,

const uint16_t num_ue_srs_ports,

const uint8_t *nrOfLayers,

int *additional_max_tpmi)

{

uint8_t max_tpmi = 0;

if (!pusch_Config

|| (pusch_Config->txConfig != NULL && *pusch_Config->txConfig == NR_PUSCH_Config__txConfig_nonCodebook)

|| num_ue_srs_ports == 1)

return max_tpmi;

long max_rank = *pusch_Config->maxRank;

long *ul_FullPowerTransmission = pusch_Config->ext1 ? pusch_Config->ext1->ul_FullPowerTransmission_r16 : NULL;

long *codebookSubset = pusch_Config->codebookSubset;

if (num_ue_srs_ports == 2) {

if (max_rank == 1) {

if (ul_FullPowerTransmission && *ul_FullPowerTransmission == NR_PUSCH_Config__ext1__ul_FullPowerTransmission_r16_fullpowerMode1) {

max_tpmi = 2;

} else {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

max_tpmi = 1;

} else {

max_tpmi = 5;

}

}

} else {

if (ul_FullPowerTransmission && *ul_FullPowerTransmission == NR_PUSCH_Config__ext1__ul_FullPowerTransmission_r16_fullpowerMode1) {

max_tpmi = *nrOfLayers == 1 ? 2 : 0;

} else {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

max_tpmi = *nrOfLayers == 1 ? 1 : 0;

} else {

max_tpmi = *nrOfLayers == 1 ? 5 : 2;

}

}

}

} else if (num_ue_srs_ports == 4) {

if (max_rank == 1) {

if (ul_FullPowerTransmission && *ul_FullPowerTransmission == NR_PUSCH_Config__ext1__ul_FullPowerTransmission_r16_fullpowerMode1) {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

max_tpmi = 3;

*additional_max_tpmi = 13;

} else {

max_tpmi = 15;

}

} else {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

max_tpmi = 3;

} else if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_partialAndNonCoherent) {

max_tpmi = 11;

} else {

max_tpmi = 27;

}

}

} else {

if (ul_FullPowerTransmission && *ul_FullPowerTransmission == NR_PUSCH_Config__ext1__ul_FullPowerTransmission_r16_fullpowerMode1) {

if (max_rank == 2) {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

max_tpmi = *nrOfLayers == 1 ? 3 : 6;

if (*nrOfLayers == 1) {

*additional_max_tpmi = 13;

}

} else {

max_tpmi = *nrOfLayers == 1 ? 15 : 13;

}

} else {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

switch (*nrOfLayers) {

case 1:

max_tpmi = 3;

*additional_max_tpmi = 13;

break;

case 2:

max_tpmi = 6;

break;

case 3:

max_tpmi = 1;

break;

case 4:

max_tpmi = 0;

break;

default:

LOG_E(NR_MAC,"Number of layers %d is invalid!\n", *nrOfLayers);

}

} else {

switch (*nrOfLayers) {

case 1:

max_tpmi = 15;

break;

case 2:

max_tpmi = 13;

break;

case 3:

case 4:

max_tpmi = 2;

break;

default:

LOG_E(NR_MAC,"Number of layers %d is invalid!\n", *nrOfLayers);

}

}

}

} else {

if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_nonCoherent) {

switch (*nrOfLayers) {

case 1:

max_tpmi = 3;

break;

case 2:

max_tpmi = 5;

break;

case 3:

case 4:

max_tpmi = 0;

break;

default:

LOG_E(NR_MAC,"Number of layers %d is invalid!\n", *nrOfLayers);

}

} else if (codebookSubset && *codebookSubset == NR_PUSCH_Config__codebookSubset_partialAndNonCoherent) {

switch (*nrOfLayers) {

case 1:

max_tpmi = 11;

break;

case 2:

max_tpmi = 13;

break;

case 3:

case 4:

max_tpmi = 2;

break;

default:

LOG_E(NR_MAC,"Number of layers %d is invalid!\n", *nrOfLayers);

}

} else {

switch (*nrOfLayers) {

case 1:

max_tpmi = 28;

break;

case 2:

max_tpmi = 22;

break;

case 3:

max_tpmi = 7;

break;

case 4:

max_tpmi = 5;

break;

default:

LOG_E(NR_MAC,"Number of layers %d is invalid!\n", *nrOfLayers);

}

}

}

}

}

return max_tpmi;

}

// TS 38.211 - Table 6.3.1.5-1: Precoding matrix W for single-layer transmission using two antenna ports, 'n' = -1 and 'o' = -j

const char table_38211_6_3_1_5_1[6][2][1] = {

{{'1'}, {'0'}}, // tpmi 0

{{'0'}, {'1'}}, // tpmi 1

{{'1'}, {'1'}}, // tpmi 2

{{'1'}, {'n'}}, // tpmi 3

{{'1'}, {'j'}}, // tpmi 4

{{'1'}, {'o'}}  // tpmi 5

};

// TS 38.211 - Table 6.3.1.5-2: Precoding matrix W for single-layer transmission using four antenna ports with transform precoding enabled, 'n' = -1 and 'o' = -j

const char table_38211_6_3_1_5_2[28][4][1] = {

{{'1'}, {'0'}, {'0'}, {'0'}}, // tpmi 0

{{'0'}, {'1'}, {'0'}, {'0'}}, // tpmi 1

{{'0'}, {'0'}, {'1'}, {'0'}}, // tpmi 2

{{'0'}, {'0'}, {'0'}, {'1'}}, // tpmi 3

{{'1'}, {'0'}, {'1'}, {'0'}}, // tpmi 4

{{'1'}, {'0'}, {'n'}, {'0'}}, // tpmi 5

{{'1'}, {'0'}, {'j'}, {'0'}}, // tpmi 6

{{'1'}, {'0'}, {'o'}, {'0'}}, // tpmi 7

{{'0'}, {'1'}, {'0'}, {'1'}}, // tpmi 8

{{'0'}, {'1'}, {'0'}, {'n'}}, // tpmi 9

{{'0'}, {'1'}, {'0'}, {'j'}}, // tpmi 10

{{'0'}, {'1'}, {'0'}, {'o'}}, // tpmi 11

{{'1'}, {'1'}, {'1'}, {'n'}}, // tpmi 12

{{'1'}, {'1'}, {'j'}, {'j'}}, // tpmi 13

{{'1'}, {'1'}, {'n'}, {'1'}}, // tpmi 14

{{'1'}, {'1'}, {'o'}, {'o'}}, // tpmi 15

{{'1'}, {'j'}, {'1'}, {'j'}}, // tpmi 16

{{'1'}, {'j'}, {'j'}, {'1'}}, // tpmi 17

{{'1'}, {'j'}, {'n'}, {'o'}}, // tpmi 18

{{'1'}, {'j'}, {'o'}, {'n'}}, // tpmi 19

{{'1'}, {'n'}, {'1'}, {'1'}}, // tpmi 20

{{'1'}, {'n'}, {'j'}, {'o'}}, // tpmi 21

{{'1'}, {'n'}, {'n'}, {'n'}}, // tpmi 22

{{'1'}, {'n'}, {'o'}, {'j'}}, // tpmi 23

{{'1'}, {'o'}, {'1'}, {'o'}}, // tpmi 24

{{'1'}, {'o'}, {'j'}, {'n'}}, // tpmi 25

{{'1'}, {'o'}, {'n'}, {'j'}}, // tpmi 26

{{'1'}, {'o'}, {'o'}, {'1'}}  // tpmi 27

};

// TS 38.211 - Table 6.3.1.5-3: Precoding matrix W for single-layer transmission using four antenna ports with transform precoding disabled, 'n' = -1 and 'o' = -j

const char table_38211_6_3_1_5_3[28][4][1] = {

{{'1'}, {'0'}, {'0'}, {'0'}}, // tpmi 0

{{'0'}, {'1'}, {'0'}, {'0'}}, // tpmi 1

{{'0'}, {'0'}, {'1'}, {'0'}}, // tpmi 2

{{'0'}, {'0'}, {'0'}, {'1'}}, // tpmi 3

{{'1'}, {'0'}, {'1'}, {'0'}}, // tpmi 4

{{'1'}, {'0'}, {'n'}, {'0'}}, // tpmi 5

{{'1'}, {'0'}, {'j'}, {'0'}}, // tpmi 6

{{'1'}, {'0'}, {'o'}, {'0'}}, // tpmi 7

{{'0'}, {'1'}, {'0'}, {'1'}}, // tpmi 8

{{'0'}, {'1'}, {'0'}, {'n'}}, // tpmi 9

{{'0'}, {'1'}, {'0'}, {'j'}}, // tpmi 10

{{'0'}, {'1'}, {'0'}, {'o'}}, // tpmi 11

{{'1'}, {'1'}, {'1'}, {'1'}}, // tpmi 12

{{'1'}, {'1'}, {'j'}, {'j'}}, // tpmi 13

{{'1'}, {'1'}, {'n'}, {'n'}}, // tpmi 14

{{'1'}, {'1'}, {'o'}, {'o'}}, // tpmi 15

{{'1'}, {'j'}, {'1'}, {'j'}}, // tpmi 16

{{'1'}, {'j'}, {'j'}, {'n'}}, // tpmi 17

{{'1'}, {'j'}, {'n'}, {'o'}}, // tpmi 18

{{'1'}, {'j'}, {'o'}, {'1'}}, // tpmi 19

{{'1'}, {'n'}, {'1'}, {'n'}}, // tpmi 20

{{'1'}, {'n'}, {'j'}, {'o'}}, // tpmi 21

{{'1'}, {'n'}, {'n'}, {'1'}}, // tpmi 22

{{'1'}, {'n'}, {'o'}, {'j'}}, // tpmi 23

{{'1'}, {'o'}, {'1'}, {'o'}}, // tpmi 24

{{'1'}, {'o'}, {'j'}, {'1'}}, // tpmi 25

{{'1'}, {'o'}, {'n'}, {'j'}}, // tpmi 26

{{'1'}, {'o'}, {'o'}, {'n'}}  // tpmi 27

};

// TS 38.211 - Table 6.3.1.5-4: Precoding matrix W for two-layer transmission using two antenna ports, 'n' = -1 and 'o' = -j

const char table_38211_6_3_1_5_4[3][2][2] = {

{{'1', '0'}, {'0', '1'}}, // tpmi 0

{{'1', '1'}, {'1', 'n'}}, // tpmi 1

{{'1', '1'}, {'j', 'o'}}  // tpmi 2

};

// TS 38.211 - Table 6.3.1.5-5: Precoding matrix W for two-layer transmission using four antenna ports, 'n' = -1 and 'o' = -j

const char table_38211_6_3_1_5_5[22][4][2] = {

{{'1', '0'}, {'0', '1'}, {'0', '0'}, {'0', '0'}}, // tpmi 0

{{'1', '0'}, {'0', '0'}, {'0', '1'}, {'0', '0'}}, // tpmi 1

{{'1', '0'}, {'0', '0'}, {'0', '0'}, {'0', '1'}}, // tpmi 2

{{'0', '0'}, {'1', '0'}, {'0', '1'}, {'0', '0'}}, // tpmi 3

{{'0', '0'}, {'1', '0'}, {'0', '0'}, {'0', '1'}}, // tpmi 4

{{'0', '0'}, {'0', '0'}, {'1', '0'}, {'0', '1'}}, // tpmi 5

{{'1', '0'}, {'0', '1'}, {'1', '0'}, {'0', 'o'}}, // tpmi 6

{{'1', '0'}, {'0', '1'}, {'1', '0'}, {'0', 'j'}}, // tpmi 7

{{'1', '0'}, {'0', '1'}, {'o', '0'}, {'0', '1'}}, // tpmi 8

{{'1', '0'}, {'0', '1'}, {'o', '0'}, {'0', 'n'}}, // tpmi 9

{{'1', '0'}, {'0', '1'}, {'n', '0'}, {'0', 'o'}}, // tpmi 10

{{'1', '0'}, {'0', '1'}, {'n', '0'}, {'0', 'j'}}, // tpmi 11

{{'1', '0'}, {'0', '1'}, {'j', '0'}, {'0', '1'}}, // tpmi 12

{{'1', '0'}, {'0', '1'}, {'j', '0'}, {'0', 'n'}}, // tpmi 13

{{'1', '1'}, {'1', '1'}, {'1', 'n'}, {'1', 'n'}}, // tpmi 14

{{'1', '1'}, {'1', '1'}, {'j', 'o'}, {'j', 'o'}}, // tpmi 15

{{'1', '1'}, {'j', 'j'}, {'1', 'n'}, {'j', 'o'}}, // tpmi 16

{{'1', '1'}, {'j', 'j'}, {'j', 'o'}, {'n', '1'}}, // tpmi 17

{{'1', '1'}, {'n', 'n'}, {'1', 'n'}, {'n', '1'}}, // tpmi 18

{{'1', '1'}, {'n', 'n'}, {'j', 'o'}, {'o', 'j'}}, // tpmi 19

{{'1', '1'}, {'o', 'o'}, {'1', 'n'}, {'o', 'j'}}, // tpmi 20

{{'1', '1'}, {'o', 'o'}, {'j', 'o'}, {'1', 'n'}}  // tpmi 21

};

static void get_precoder_matrix_coef(char *w,

const uint8_t ul_ri,

const uint16_t num_ue_srs_ports,

const long transform_precoding,

const uint8_t tpmi,

const uint8_t uI,

int layer_idx)

{

if (ul_ri == 0) {

if (num_ue_srs_ports == 2) {

*w = table_38211_6_3_1_5_1[tpmi][uI][layer_idx];

} else {

if (transform_precoding == NR_PUSCH_Config__transformPrecoder_enabled) {

*w = table_38211_6_3_1_5_2[tpmi][uI][layer_idx];

} else {

*w = table_38211_6_3_1_5_3[tpmi][uI][layer_idx];

}

}

} else if (ul_ri == 1) {

if (num_ue_srs_ports == 2) {

*w = table_38211_6_3_1_5_4[tpmi][uI][layer_idx];

} else {

*w = table_38211_6_3_1_5_5[tpmi][uI][layer_idx];

}

} else {

AssertFatal(1 == 0, "Function get_precoder_matrix_coef() does not support %i layers yet!\n", ul_ri + 1);

}

}

static int nr_srs_tpmi_estimation(const NR_PUSCH_Config_t *pusch_Config,

const long transform_precoding,

const uint8_t *channel_matrix,

const uint8_t normalized_iq_representation,

const uint16_t num_gnb_antenna_elements,

const uint16_t num_ue_srs_ports,

const uint16_t prg_size,

const uint16_t num_prgs,

const uint8_t ul_ri)

{

if (ul_ri > 1) {

LOG_D(NR_MAC, "TPMI computation for ul_ri %i is not implemented yet!\n", ul_ri);

return 0;

}

uint8_t tpmi_sel = 0;

const uint8_t nrOfLayers = ul_ri + 1;

int16_t precoded_channel_matrix_re[num_prgs * num_gnb_antenna_elements];

int16_t precoded_channel_matrix_im[num_prgs * num_gnb_antenna_elements];

c16_t *channel_matrix16 = (c16_t *)channel_matrix;

uint32_t max_precoded_signal_power = 0;

int additional_max_tpmi = -1;

char w;

uint8_t max_tpmi = get_max_tpmi(pusch_Config, num_ue_srs_ports, &nrOfLayers, &additional_max_tpmi);

uint8_t end_tpmi_loop = additional_max_tpmi > max_tpmi ? additional_max_tpmi : max_tpmi;

//                      channel_matrix                          x   precoder_matrix

// [ (gI=0,uI=0) (gI=0,uI=1) ... (gI=0,uI=num_ue_srs_ports-1) ] x   [uI=0]

// [ (gI=1,uI=0) (gI=1,uI=1) ... (gI=1,uI=num_ue_srs_ports-1) ]     [uI=1]

// [ (gI=2,uI=0) (gI=2,uI=1) ... (gI=2,uI=num_ue_srs_ports-1) ]     [uI=2]

//                           ...                                     ...

for (uint8_t tpmi = 0; tpmi <= end_tpmi_loop && end_tpmi_loop > 0; tpmi++) {

if (tpmi > max_tpmi) {

tpmi = end_tpmi_loop;

}

for (int pI = 0; pI < num_prgs; pI++) {

for (int gI = 0; gI < num_gnb_antenna_elements; gI++) {

uint16_t index_gI_pI = gI * num_prgs + pI;

precoded_channel_matrix_re[index_gI_pI] = 0;

precoded_channel_matrix_im[index_gI_pI] = 0;

for (int uI = 0; uI < num_ue_srs_ports; uI++) {

for (int layer_idx = 0; layer_idx < nrOfLayers; layer_idx++) {

uint16_t index = uI * num_gnb_antenna_elements * num_prgs + index_gI_pI;

get_precoder_matrix_coef(&w, ul_ri, num_ue_srs_ports, transform_precoding, tpmi, uI, layer_idx);

c16_t h_times_w = nr_h_times_w(channel_matrix16[index], w);

precoded_channel_matrix_re[index_gI_pI] += h_times_w.r;

precoded_channel_matrix_im[index_gI_pI] += h_times_w.i;

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "(pI %i, gI %i,  uI %i, layer_idx %i) w = %c, channel_matrix --> real %i, imag %i\n",

pI, gI, uI, layer_idx, w, channel_matrix16[index].r, channel_matrix16[index].i);

#endif

}

}

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "(pI %i, gI %i) precoded_channel_coef --> real %i, imag %i\n",

pI, gI, precoded_channel_matrix_re[index_gI_pI], precoded_channel_matrix_im[index_gI_pI]);

#endif

}

}

uint32_t precoded_signal_power = calc_power_complex(precoded_channel_matrix_re,

precoded_channel_matrix_im,

num_prgs * num_gnb_antenna_elements);

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "(tpmi %i) precoded_signal_power = %i\n", tpmi, precoded_signal_power);

#endif

if (precoded_signal_power > max_precoded_signal_power) {

max_precoded_signal_power = precoded_signal_power;

tpmi_sel = tpmi;

}

}

return tpmi_sel;

}

void handle_nr_srs_measurements(const module_id_t module_id,

const frame_t frame,

const slot_t slot,

nfapi_nr_srs_indication_pdu_t *srs_ind)

{

gNB_MAC_INST *nrmac = RC.nrmac[module_id];

LOG_D(NR_MAC, "(%d.%d) Received SRS indication for UE %04x\n", frame, slot, srs_ind->rnti);

if (srs_ind->report_type == 0) {

//SCF 222.10.04 Table 3-129 Report type = 0 means a null report, we can skip unpacking it

return;

}

if (srs_ind->timing_advance_offset == 0xFFFF) {

LOG_W(NR_MAC, "Invalid timing advance offset for RNTI %04x\n", srs_ind->rnti);

return;

}

NR_SCHED_LOCK(&nrmac->sched_lock);

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "frame = %i\n", frame);

LOG_I(NR_MAC, "slot = %i\n", slot);

LOG_I(NR_MAC, "srs_ind->rnti = %04x\n", srs_ind->rnti);

LOG_I(NR_MAC, "srs_ind->timing_advance_offset = %i\n", srs_ind->timing_advance_offset);

LOG_I(NR_MAC, "srs_ind->timing_advance_offset_nsec = %i\n", srs_ind->timing_advance_offset_nsec);

LOG_I(NR_MAC, "srs_ind->srs_usage = %i\n", srs_ind->srs_usage);

LOG_I(NR_MAC, "srs_ind->report_type = %i\n", srs_ind->report_type);

#endif

NR_UE_info_t *UE = find_nr_UE(&RC.nrmac[module_id]->UE_info, srs_ind->rnti);

if (!UE) {

LOG_W(NR_MAC, "Could not find UE for RNTI %04x\n", srs_ind->rnti);

NR_SCHED_UNLOCK(&nrmac->sched_lock);

return;

}

gNB_MAC_INST *nr_mac = RC.nrmac[module_id];

NR_mac_stats_t *stats = &UE->mac_stats;

nfapi_srs_report_tlv_t *report_tlv = &srs_ind->report_tlv;

switch (srs_ind->srs_usage) {

case NR_SRS_ResourceSet__usage_beamManagement: {

nfapi_nr_srs_beamforming_report_t nr_srs_bf_report;

unpack_nr_srs_beamforming_report(report_tlv->value,

report_tlv->length,

&nr_srs_bf_report,

sizeof(nfapi_nr_srs_beamforming_report_t));

if (nr_srs_bf_report.wide_band_snr == 0xFF) {

LOG_W(NR_MAC, "Invalid wide_band_snr for RNTI %04x\n", srs_ind->rnti);

NR_SCHED_UNLOCK(&nrmac->sched_lock);

return;

}

int wide_band_snr_dB = (nr_srs_bf_report.wide_band_snr >> 1) - 64;

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "nr_srs_bf_report.prg_size = %i\n", nr_srs_bf_report.prg_size);

LOG_I(NR_MAC, "nr_srs_bf_report.num_symbols = %i\n", nr_srs_bf_report.num_symbols);

LOG_I(NR_MAC, "nr_srs_bf_report.wide_band_snr = %i (%i dB)\n", nr_srs_bf_report.wide_band_snr, wide_band_snr_dB);

LOG_I(NR_MAC, "nr_srs_bf_report.num_reported_symbols = %i\n", nr_srs_bf_report.num_reported_symbols);

LOG_I(NR_MAC, "nr_srs_bf_report.reported_symbol_list[0].num_prgs = %i\n", nr_srs_bf_report.reported_symbol_list[0].num_prgs);

for (int prg_idx = 0; prg_idx < nr_srs_bf_report.reported_symbol_list[0].num_prgs; prg_idx++) {

LOG_I(NR_MAC,

"nr_srs_bf_report.reported_symbol_list[0].prg_list[%3i].rb_snr = %i (%i dB)\n",

prg_idx,

nr_srs_bf_report.reported_symbol_list[0].prg_list[prg_idx].rb_snr,

(nr_srs_bf_report.reported_symbol_list[0].prg_list[prg_idx].rb_snr >> 1) - 64);

}

#endif

sprintf(stats->srs_stats, "UL-SNR %i dB", wide_band_snr_dB);

const int ul_prbblack_SNR_threshold = nr_mac->ul_prbblack_SNR_threshold;

uint16_t *ulprbbl = nr_mac->ulprbbl;

uint16_t num_rbs = nr_srs_bf_report.prg_size * nr_srs_bf_report.reported_symbol_list[0].num_prgs;

memset(ulprbbl, 0, num_rbs * sizeof(uint16_t));

for (int rb = 0; rb < num_rbs; rb++) {

int snr = (nr_srs_bf_report.reported_symbol_list[0].prg_list[rb / nr_srs_bf_report.prg_size].rb_snr >> 1) - 64;

if (snr < wide_band_snr_dB - ul_prbblack_SNR_threshold) {

ulprbbl[rb] = 0x3FFF; // all symbols taken

}

LOG_D(NR_MAC, "ulprbbl[%3i] = 0x%x\n", rb, ulprbbl[rb]);

}

break;

}

case NR_SRS_ResourceSet__usage_codebook: {

nfapi_nr_srs_normalized_channel_iq_matrix_t nr_srs_channel_iq_matrix;

unpack_nr_srs_normalized_channel_iq_matrix(report_tlv->value,

report_tlv->length,

&nr_srs_channel_iq_matrix,

sizeof(nfapi_nr_srs_normalized_channel_iq_matrix_t));

#ifdef SRS_IND_DEBUG

LOG_I(NR_MAC, "nr_srs_channel_iq_matrix.normalized_iq_representation = %i\n", nr_srs_channel_iq_matrix.normalized_iq_representation);

LOG_I(NR_MAC, "nr_srs_channel_iq_matrix.num_gnb_antenna_elements = %i\n", nr_srs_channel_iq_matrix.num_gnb_antenna_elements);

LOG_I(NR_MAC, "nr_srs_channel_iq_matrix.num_ue_srs_ports = %i\n", nr_srs_channel_iq_matrix.num_ue_srs_ports);

LOG_I(NR_MAC, "nr_srs_channel_iq_matrix.prg_size = %i\n", nr_srs_channel_iq_matrix.prg_size);

LOG_I(NR_MAC, "nr_srs_channel_iq_matrix.num_prgs = %i\n", nr_srs_channel_iq_matrix.num_prgs);

c16_t *channel_matrix16 = (c16_t *)nr_srs_channel_iq_matrix.channel_matrix;

c8_t *channel_matrix8 = (c8_t *)nr_srs_channel_iq_matrix.channel_matrix;

for (int uI = 0; uI < nr_srs_channel_iq_matrix.num_ue_srs_ports; uI++) {

for (int gI = 0; gI < nr_srs_channel_iq_matrix.num_gnb_antenna_elements; gI++) {

for (int pI = 0; pI < nr_srs_channel_iq_matrix.num_prgs; pI++) {

uint16_t index = uI * nr_srs_channel_iq_matrix.num_gnb_antenna_elements * nr_srs_channel_iq_matrix.num_prgs + gI * nr_srs_channel_iq_matrix.num_prgs + pI;

LOG_I(NR_MAC,

"(uI %i, gI %i, pI %i) channel_matrix --> real %i, imag %i\n",

uI,

gI,

pI,

nr_srs_channel_iq_matrix.normalized_iq_representation == 0 ? channel_matrix8[index].r : channel_matrix16[index].r,

nr_srs_channel_iq_matrix.normalized_iq_representation == 0 ? channel_matrix8[index].i : channel_matrix16[index].i);

}

}

}

#endif

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_UL_BWP_t *current_BWP = &UE->current_UL_BWP;

sched_ctrl->srs_feedback.sri = NR_SRS_SRI_0;

start_meas(&nr_mac->nr_srs_ri_computation_timer);

nr_srs_ri_computation(&nr_srs_channel_iq_matrix, current_BWP, &sched_ctrl->srs_feedback.ul_ri);

stop_meas(&nr_mac->nr_srs_ri_computation_timer);

start_meas(&nr_mac->nr_srs_tpmi_computation_timer);

sched_ctrl->srs_feedback.tpmi = nr_srs_tpmi_estimation(current_BWP->pusch_Config,

current_BWP->transform_precoding,

nr_srs_channel_iq_matrix.channel_matrix,

nr_srs_channel_iq_matrix.normalized_iq_representation,

nr_srs_channel_iq_matrix.num_gnb_antenna_elements,

nr_srs_channel_iq_matrix.num_ue_srs_ports,

nr_srs_channel_iq_matrix.prg_size,

nr_srs_channel_iq_matrix.num_prgs,

sched_ctrl->srs_feedback.ul_ri);

stop_meas(&nr_mac->nr_srs_tpmi_computation_timer);

sprintf(stats->srs_stats, "UL-RI %d, TPMI %d", sched_ctrl->srs_feedback.ul_ri + 1, sched_ctrl->srs_feedback.tpmi);

break;

}

case NR_SRS_ResourceSet__usage_nonCodebook:

case NR_SRS_ResourceSet__usage_antennaSwitching:

LOG_W(NR_MAC, "MAC procedures for this SRS usage are not implemented yet!\n");

break;

default:

AssertFatal(1 == 0, "Invalid SRS usage\n");

}

NR_SCHED_UNLOCK(&nrmac->sched_lock);

}

static bool nr_UE_is_to_be_scheduled(const frame_structure_t *fs,

NR_UE_info_t *UE,

frame_t frame,

slot_t slot,

uint32_t ulsch_max_frame_inactivity)

{

const int n = fs->numb_slots_frame;

const int now = frame * n + slot;

const NR_UE_sched_ctrl_t *sched_ctrl =&UE->UE_sched_ctrl;

/**

* Force the default transmission in a full slot as early

* as possible in the UL portion of TDD period (last_ul_slot) */

int num_slots_per_period = fs->numb_slots_period;

int last_ul_slot = fs->frame_type == TDD ? get_first_ul_slot(fs, false) : sched_ctrl->last_ul_slot;

const int last_ul_sched = sched_ctrl->last_ul_frame * n + last_ul_slot;

const int diff = (now - last_ul_sched + 1024 * n) % (1024 * n);

/* UE is to be scheduled if

* (1) we think the UE has more bytes awaiting than what we scheduled

* (2) there is a scheduling request

* (3) or we did not schedule it in more than 10 frames */

const bool has_data = sched_ctrl->estimated_ul_buffer > sched_ctrl->sched_ul_bytes;

const bool high_inactivity = diff >= (ulsch_max_frame_inactivity > 0 ? ulsch_max_frame_inactivity * n : num_slots_per_period);

LOG_D(NR_MAC,

"%4d.%2d UL inactivity %d slots has_data %d SR %d\n",

frame,

slot,

diff,

has_data,

sched_ctrl->SR);

return has_data || sched_ctrl->SR || high_inactivity;

}

static void update_ul_ue_R_Qm(int mcs, int mcs_table, const NR_PUSCH_Config_t *pusch_Config, uint16_t *R, uint8_t *Qm)

{

*R = nr_get_code_rate_ul(mcs, mcs_table);

*Qm = nr_get_Qm_ul(mcs, mcs_table);

if (pusch_Config && pusch_Config->tp_pi2BPSK && ((mcs_table == 3 && mcs < 2) || (mcs_table == 4 && mcs < 6))) {

*R >>= 1;

*Qm <<= 1;

}

}

static void nr_ue_max_mcs_min_rb(int mu,

int ph_limit,

NR_sched_pusch_t *sched_pusch,

NR_UE_UL_BWP_t *ul_bwp,

uint16_t minRb,

uint32_t tbs,

uint16_t *Rb,

uint8_t *mcs)

{

AssertFatal(*Rb >= minRb, "illegal Rb %d < minRb %d\n", *Rb, minRb);

AssertFatal(*mcs >= 0 && *mcs <= 28, "illegal MCS %d\n", *mcs);

int tbs_bits = tbs << 3;

uint16_t R;

uint8_t Qm;

update_ul_ue_R_Qm(*mcs, ul_bwp->mcs_table, ul_bwp->pusch_Config, &R, &Qm);

long *deltaMCS = ul_bwp->pusch_Config ? ul_bwp->pusch_Config->pusch_PowerControl->deltaMCS : NULL;

tbs_bits = nr_compute_tbs(Qm, R, *Rb,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

0, // nb_rb_oh

0,

sched_pusch->nrOfLayers);

int tx_power = compute_ph_factor(mu,

tbs_bits,

*Rb,

sched_pusch->nrOfLayers,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

deltaMCS,

true);

while (ph_limit < tx_power && *Rb > minRb) {

(*Rb)--;

tbs_bits = nr_compute_tbs(Qm, R, *Rb,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

0, // nb_rb_oh

0,

sched_pusch->nrOfLayers);

tx_power = compute_ph_factor(mu,

tbs_bits,

*Rb,

sched_pusch->nrOfLayers,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

deltaMCS,

true);

LOG_D(NR_MAC, "Checking %d RBs, MCS %d, ph_limit %d, tx_power %d\n",*Rb,*mcs,ph_limit,tx_power);

}

while (ph_limit < tx_power && *mcs > 0) {

(*mcs)--;

update_ul_ue_R_Qm(*mcs, ul_bwp->mcs_table, ul_bwp->pusch_Config, &R, &Qm);

tbs_bits = nr_compute_tbs(Qm, R, *Rb,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

0, // nb_rb_oh

0,

sched_pusch->nrOfLayers);

tx_power = compute_ph_factor(mu,

tbs_bits,

*Rb,

sched_pusch->nrOfLayers,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

deltaMCS,

true);

LOG_D(NR_MAC, "Checking %d RBs, MCS %d, ph_limit %d, tx_power %d\n",*Rb,*mcs,ph_limit,tx_power);

}

if (ph_limit < tx_power)

LOG_D(NR_MAC, "Normalized power %d based on current resources (RBs %d, MCS %d) exceed reported PHR %d (normalized value)\n",

tx_power, *Rb, *mcs, ph_limit);

}

static bool allocate_ul_retransmission(gNB_MAC_INST *nrmac,

post_process_pusch_t *pp_pusch,

uint16_t *rballoc_mask,

int *n_rb_sched,

int dci_beam_idx,

NR_UE_info_t *UE,

int harq_pid,

const NR_ServingCellConfigCommon_t *scc,

const int tda,

const NR_tda_info_t *tda_info)

{

const int CC_id = 0;

int frame = pp_pusch->frame;

int slot = pp_pusch->slot;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

/* Get previous PUSCH field info */

const NR_sched_pusch_t *retInfo = &sched_ctrl->ul_harq_processes[harq_pid].sched_pusch;

NR_sched_pusch_t new_sched = *retInfo; // potential new allocation

NR_UE_UL_BWP_t *ul_bwp = &UE->current_UL_BWP;

int rbStart = 0; // wrt BWP start

bwp_info_t bwp_info = get_pusch_bwp_start_size(UE);

const uint32_t bwpSize = bwp_info.bwpSize;

const uint32_t bwpStart = bwp_info.bwpStart;

const uint8_t nrOfLayers = retInfo->nrOfLayers;

LOG_D(NR_MAC,"retInfo->time_domain_allocation = %d, tda = %d\n", retInfo->time_domain_allocation, tda);

/* mark when retransmission will happen */

int slots_frame = nrmac->frame_structure.numb_slots_frame;

new_sched.frame = (frame + (slot + tda_info->k2 + get_NTN_Koffset(scc)) / slots_frame) % MAX_FRAME_NUMBER;

new_sched.slot = (slot + tda_info->k2 + get_NTN_Koffset(scc)) % slots_frame;

new_sched.bwp_info = bwp_info;

DevAssert(new_sched.ul_harq_pid == harq_pid);

bool reuse_old_tda = retInfo->time_domain_allocation == tda;

if (reuse_old_tda && nrOfLayers == retInfo->nrOfLayers) {

/* Check the resource is enough for retransmission */

const uint16_t slbitmap = SL_to_bitmap(retInfo->tda_info.startSymbolIndex, retInfo->tda_info.nrOfSymbols);

while (rbStart < bwpSize && (rballoc_mask[rbStart + bwpStart] & slbitmap))

rbStart++;

if (rbStart + retInfo->rbSize > bwpSize) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not allocate UL retransmission: no resources (rbStart %d, retInfo->rbSize %d, bwpSize %d) \n",

UE->rnti,

frame,

slot,

rbStart,

retInfo->rbSize,

bwpSize);

return false;

}

new_sched.rbStart = rbStart;

LOG_D(NR_MAC, "Retransmission keeping TDA %d and TBS %d\n", tda, retInfo->tb_size);

} else {

NR_pusch_dmrs_t dmrs_info = get_ul_dmrs_params(scc, ul_bwp, tda_info, nrOfLayers);

/* the retransmission will use a different time domain allocation, check

* that we have enough resources */

const uint16_t slbitmap = SL_to_bitmap(tda_info->startSymbolIndex, tda_info->nrOfSymbols);

while (rbStart < bwpSize && (rballoc_mask[rbStart + bwpStart] & slbitmap))

rbStart++;

int rbSize = 0;

while (rbStart + rbSize < bwpSize && !(rballoc_mask[rbStart + bwpStart + rbSize] & slbitmap))

rbSize++;

uint32_t new_tbs;

uint16_t new_rbSize;

bool success = nr_find_nb_rb(retInfo->Qm,

retInfo->R,

UE->current_UL_BWP.transform_precoding,

nrOfLayers,

tda_info->nrOfSymbols,

dmrs_info.N_PRB_DMRS * dmrs_info.num_dmrs_symb,

retInfo->tb_size,

1, /* minimum of 1RB: need to find exact TBS, don't preclude any number */

rbSize,

&new_tbs,

&new_rbSize);

if (!success || new_tbs != retInfo->tb_size) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] allocation of UL retransmission failed: new TBsize %d of new TDA does not match old TBS %d \n",

UE->rnti,

frame,

slot,

new_tbs,

retInfo->tb_size);

return false; /* the maximum TBsize we might have is smaller than what we need */

}

LOG_D(NR_MAC, "Retransmission with TDA %d->%d and TBS %d -> %d\n", retInfo->time_domain_allocation, tda, retInfo->tb_size, new_tbs);

/* we can allocate it. Overwrite the time_domain_allocation, the number

* of RBs, and the new TB size. The rest is done below */

new_sched.rbSize = new_rbSize;

new_sched.rbStart = rbStart;

new_sched.tb_size = new_tbs;

new_sched.time_domain_allocation = tda;

new_sched.tda_info = *tda_info;

new_sched.dmrs_info = dmrs_info;

}

/* Find a free CCE */

int CCEIndex = get_cce_index(nrmac,

CC_id,

slot,

UE->rnti,

&sched_ctrl->aggregation_level,

dci_beam_idx,

sched_ctrl->search_space,

sched_ctrl->coreset,

&sched_ctrl->sched_pdcch,

sched_ctrl->pdcch_cl_adjust);

if (CCEIndex<0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] no free CCE for retransmission UL DCI UE\n", UE->rnti, frame, slot);

sched_ctrl->ul_cce_fail++;

return false;

}

sched_ctrl->cce_index = CCEIndex;

fill_pdcch_vrb_map(nrmac, CC_id, &sched_ctrl->sched_pdcch, CCEIndex, sched_ctrl->aggregation_level, dci_beam_idx);

// signal new allocation

DevAssert(new_sched.time_domain_allocation == tda);

post_process_ulsch(nrmac, pp_pusch, UE, &new_sched);

LOG_D(NR_MAC,

"%4d.%2d Allocate UL retransmission RNTI %04x sched %4d.%2d (%d RBs)\n",

frame,

slot,

UE->rnti,

new_sched.frame,

new_sched.slot,

new_sched.rbSize);

/* Mark the corresponding RBs as used */

n_rb_sched -= new_sched.rbSize;

for (int rb = bwpStart; rb < new_sched.rbSize; rb++)

rballoc_mask[rb + new_sched.rbStart] |= SL_to_bitmap(new_sched.tda_info.startSymbolIndex, new_sched.tda_info.nrOfSymbols);

return true;

}

static uint32_t ul_pf_tbs[5][29]; // pre-computed, approximate TBS values for PF coefficient

typedef struct UEsched_s {

float coef;

bool sched_inactive;

NR_UE_info_t * UE;

int selected_mcs;

} UEsched_t;

static int comparator(const void *p, const void *q)

{

const UEsched_t *pp = p;

const UEsched_t *qq = q;

if (pp->sched_inactive && qq->sched_inactive)

return 0;

else if (pp->sched_inactive)

return -1;

else if (qq->sched_inactive)

return 1;

/* both UEs are !sched_inactive */

if (pp->coef < qq->coef)

return 1;

else if (pp->coef > qq->coef)

return -1;

return 0;

}

static int  pf_ul(gNB_MAC_INST *nrmac,

post_process_pusch_t *pp_pusch,

int tda,

const NR_tda_info_t *tda_info,

NR_UE_info_t *UE_list[],

int max_num_ue,

int num_beams,

int n_rb_sched[num_beams])

{

const int CC_id = 0;

int frame = pp_pusch->frame;

int slot = pp_pusch->slot;

NR_ServingCellConfigCommon_t *scc = nrmac->common_channels[CC_id].ServingCellConfigCommon;

int slots_per_frame = nrmac->frame_structure.numb_slots_frame;

DevAssert(tda_info->valid_tda);

const int k2 = tda_info->k2 + get_NTN_Koffset(scc);

const int sched_frame = (frame + (slot + k2) / slots_per_frame) % MAX_FRAME_NUMBER;

const int sched_slot = (slot + k2) % slots_per_frame;

DevAssert(is_ul_slot(sched_slot, &nrmac->frame_structure));

const int min_rb = nrmac->min_grant_prb;

// UEs that could be scheduled

UEsched_t UE_sched[MAX_MOBILES_PER_GNB + 1] = {0};

int remainUEs[num_beams];

for (int i = 0; i < num_beams; i++)

remainUEs[i] = max_num_ue;

int numUE = 0;

bool scheduled_something = false;

/* Loop UE_list to calculate throughput and coeff */

UE_iterator(UE_list, UE) {

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

if (!nr_mac_ue_is_active(UE))

continue;

LOG_D(NR_MAC,"pf_ul: preparing UL scheduling for UE %04x\n",UE->rnti);

NR_UE_UL_BWP_t *current_BWP = &UE->current_UL_BWP;

NR_mac_dir_stats_t *stats = &UE->mac_stats.ul;

/* Calculate throughput */

const float a = 0.01f;

const uint32_t b = stats->current_bytes;

UE->ul_thr_ue = (1 - a) * UE->ul_thr_ue + a * b;

stats->current_bytes = 0;

stats->current_rbs = 0;

int total_rem_ues = 0;

for (int i = 0; i < num_beams; i++)

total_rem_ues += remainUEs[i];

if (total_rem_ues == 0)

continue;

NR_beam_alloc_t dci_beam = beam_allocation_procedure(&nrmac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame);

if (dci_beam.idx < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Beam could not be allocated\n", UE->rnti, frame, slot);

continue;

}

NR_beam_alloc_t beam = beam_allocation_procedure(&nrmac->beam_info, sched_frame, sched_slot, UE->UE_beam_index, slots_per_frame);

if (beam.idx < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Beam could not be allocated\n", UE->rnti, frame, slot);

reset_beam_status(&nrmac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

continue;

}

const int index = ul_buffer_index(sched_frame, sched_slot, slots_per_frame, nrmac->vrb_map_UL_size);

uint16_t *rballoc_mask = &nrmac->common_channels[CC_id].vrb_map_UL[beam.idx][index * MAX_BWP_SIZE];

/* Check if retransmission is necessary */

int ul_harq_pid = sched_ctrl->retrans_ul_harq.head;

LOG_D(NR_MAC,"pf_ul: UE %04x harq_pid %d\n", UE->rnti, ul_harq_pid);

if (ul_harq_pid >= 0) {

/* Allocate retransmission*/

bool r = allocate_ul_retransmission(nrmac,

pp_pusch,

rballoc_mask,

&n_rb_sched[beam.idx],

dci_beam.idx,

UE,

ul_harq_pid,

scc,

tda,

tda_info);

if (!r) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] UL retransmission could not be allocated\n", UE->rnti, frame, slot);

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, UE->UE_beam_index, slots_per_frame, beam.new_beam);

reset_beam_status(&nrmac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

continue;

}

else

LOG_D(NR_MAC,"%4d.%2d UL Retransmission UE RNTI %04x to be allocated, max_num_ue %d\n", frame, slot, UE->rnti,max_num_ue);

/* reduce max_num_ue once we are sure UE can be allocated, i.e., has CCE */

remainUEs[beam.idx]--;

scheduled_something = true;

continue;

}

DevAssert(ul_harq_pid == -1); // all other allocations must be new ones

/* skip this UE if there are no free HARQ processes. This can happen e.g.

* if the UE disconnected in L2sim, in which case the gNB is not notified

* (this can be considered a design flaw) */

if (sched_ctrl->available_ul_harq.head < 0) {

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, UE->UE_beam_index, slots_per_frame, beam.new_beam);

reset_beam_status(&nrmac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] has no free UL HARQ process, skipping\n", UE->rnti, frame, slot);

continue;

}

const int B = max(0, sched_ctrl->estimated_ul_buffer - sched_ctrl->sched_ul_bytes);

/* preprocessor computed sched_frame/sched_slot */

const bool do_sched = nr_UE_is_to_be_scheduled(&nrmac->frame_structure,

UE,

sched_frame,

sched_slot,

nrmac->ulsch_max_frame_inactivity);

LOG_D(NR_MAC,"pf_ul: do_sched UE %04x => %s\n", UE->rnti, do_sched ? "yes" : "no");

if ((B == 0 && !do_sched) || nr_timer_is_active(&sched_ctrl->transm_interrupt)) {

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, UE->UE_beam_index, slots_per_frame, beam.new_beam);

reset_beam_status(&nrmac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

continue;

}

const NR_bler_options_t *bo = &nrmac->ul_bler;

const int max_mcs_table = (current_BWP->mcs_table == 0 || current_BWP->mcs_table == 2) ? 28 : 27;

const int max_mcs = min(bo->max_mcs, max_mcs_table); /* no per-user maximum MCS yet */

int selected_mcs;

if (bo->harq_round_max == 1) {

int nrOfLayers = get_ul_nrOfLayers(sched_ctrl, current_BWP->dci_format);

selected_mcs = get_mcs_from_SINRx10(current_BWP->mcs_table, sched_ctrl->pusch_snrx10, nrOfLayers);

selected_mcs = min(max_mcs, selected_mcs);

selected_mcs = max(bo->min_mcs, selected_mcs);

sched_ctrl->ul_bler_stats.mcs = selected_mcs;

} else {

selected_mcs = get_mcs_from_bler(bo, stats, &sched_ctrl->ul_bler_stats, max_mcs, frame);

LOG_D(NR_MAC, "%d.%d starting mcs %d bler %f\n", frame, slot, selected_mcs, sched_ctrl->ul_bler_stats.bler);

}

/* Create UE_sched for UEs eligibale for new data transmission*/

/* Calculate coefficient*/

const uint32_t tbs = ul_pf_tbs[current_BWP->mcs_table][selected_mcs];

float coeff_ue = (float) tbs / UE->ul_thr_ue;

bool sched_inactive = B == 0 && do_sched;

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] b %d, ul_thr_ue %f, tbs %d, coeff_ue %f, sched_inactive %d\n",

UE->rnti,

frame,

slot,

b,

UE->ul_thr_ue,

tbs,

coeff_ue,

sched_inactive);

UE_sched[numUE].coef = coeff_ue;

UE_sched[numUE].sched_inactive = sched_inactive;

UE_sched[numUE].UE = UE;

UE_sched[numUE].selected_mcs = selected_mcs;

numUE++;

}

qsort(UE_sched, numUE, sizeof(UEsched_t), comparator);

UEsched_t *iterator=UE_sched;

/* Loop UE_sched to find max coeff and allocate transmission */

while (iterator->UE != NULL) {

NR_UE_UL_BWP_t *current_BWP = &iterator->UE->current_UL_BWP;

NR_UE_sched_ctrl_t *sched_ctrl = &iterator->UE->UE_sched_ctrl;

NR_beam_alloc_t beam = beam_allocation_procedure(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame);

if (beam.idx < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Beam could not be allocated\n", iterator->UE->rnti, frame, slot);

iterator++;

continue;

}

if (remainUEs[beam.idx] == 0 || n_rb_sched[beam.idx] < min_rb) {

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

NR_beam_alloc_t dci_beam = beam_allocation_procedure(&nrmac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame);

if (dci_beam.idx < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Beam could not be allocated\n", iterator->UE->rnti, frame, slot);

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

int CCEIndex = get_cce_index(nrmac,

CC_id, slot, iterator->UE->rnti,

&sched_ctrl->aggregation_level,

dci_beam.idx,

sched_ctrl->search_space,

sched_ctrl->coreset,

&sched_ctrl->sched_pdcch,

sched_ctrl->pdcch_cl_adjust);

if (CCEIndex < 0) {

sched_ctrl->ul_cce_fail++;

reset_beam_status(&nrmac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] no free CCE for UL DCI\n", iterator->UE->rnti, frame, slot);

iterator++;

continue;

}

else

LOG_D(NR_MAC, "%4d.%2d free CCE for UL DCI UE %04x\n", frame, slot, iterator->UE->rnti);

const int index = ul_buffer_index(sched_frame, sched_slot, slots_per_frame, nrmac->vrb_map_UL_size);

uint16_t *rballoc_mask = &nrmac->common_channels[CC_id].vrb_map_UL[beam.idx][index * MAX_BWP_SIZE];

/* find maximum amount of RBs that we can schedule starting from first free RB */

int rbStart = 0;

const uint16_t slbitmap = SL_to_bitmap(tda_info->startSymbolIndex, tda_info->nrOfSymbols);

bwp_info_t bi = get_pusch_bwp_start_size(iterator->UE);

/* Check if this UE has dynamic PRB allocation */

int dynamic_prb_start = 0;

int dynamic_prb_size = 0;

bool has_dynamic_config = get_current_prb_allocation(iterator->UE, frame, slot, &dynamic_prb_start, &dynamic_prb_size);

if (has_dynamic_config) {

/* Use dynamic PRB allocation */

rbStart = dynamic_prb_start;

uint16_t available_rb = dynamic_prb_size;

/* Check if dynamic PRBs are available (not already allocated) */

bool dynamic_prbs_available = true;

for (int rb = 0; rb < available_rb; rb++) {

if (rballoc_mask[rbStart + bi.bwpStart + rb] & slbitmap) {

dynamic_prbs_available = false;

break;

}

}

if (!dynamic_prbs_available) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Dynamic PRBs not available, skipping UE\n",

iterator->UE->rnti, frame, slot);

reset_beam_status(&nrmac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] Using dynamic PRB allocation: start=%d, size=%d\n",

iterator->UE->rnti, frame, slot, rbStart, available_rb);

} else {

/* Use dynamic PRB allocation (original logic) */

while (rbStart < bi.bwpSize && (rballoc_mask[rbStart + bi.bwpStart] & slbitmap))

rbStart++;

}

/* if it's for inactivity, min_grant_prb is enough, otherwise check what

* would be the maximum */

uint16_t max_rbSize = iterator->sched_inactive ? min_rb : bi.bwpSize;

uint16_t available_rb = 1;

if (!has_dynamic_config) {

/* No dynamic allocation - find available contiguous PRBs */

while (rbStart + available_rb < bi.bwpSize && !(rballoc_mask[rbStart + bi.bwpStart + available_rb] & slbitmap) && available_rb < max_rbSize)

available_rb++;

} else {

/* Dynamic allocation - use configured size */

available_rb = dynamic_prb_size;

}

if (rbStart + min_rb > bi.bwpSize || available_rb < min_rb) {

reset_beam_status(&nrmac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, dci_beam.new_beam);

reset_beam_status(&nrmac->beam_info, sched_frame, sched_slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not allocate UL data: no resources (rbStart %d, min_rb %d, bwpSize %d)\n",

iterator->UE->rnti,

frame,

slot,

rbStart,

min_rb,

bi.bwpSize);

iterator++;

continue;

} else

LOG_D(NR_MAC,

"allocating UL data for RNTI %04x (rbStart %d, min_rb %d, available_rb %d, bwpSize %d)\n",

iterator->UE->rnti,

rbStart,

min_rb,

available_rb,

bi.bwpSize);

int nrOfLayers = get_ul_nrOfLayers(sched_ctrl, current_BWP->dci_format);

NR_sched_pusch_t sched = {

.frame = sched_frame,

.slot = sched_slot,

// rbSize set further below

.rbStart = rbStart,

.mcs = iterator->selected_mcs,

// R, Qm, tb_size set further below

.ul_harq_pid = -1, // new transmission

.nrOfLayers = nrOfLayers,

// tpmi in post-process

.time_domain_allocation = tda,

.tda_info = *tda_info,

.dmrs_info = get_ul_dmrs_params(scc, current_BWP, tda_info, nrOfLayers),

.bwp_info = bi,

// phr_txpower_calc below

};

/* Calculate the current scheduling bytes */

const int B = cmax(sched_ctrl->estimated_ul_buffer - sched_ctrl->sched_ul_bytes, 0);

/* adjust rbSize and MCS according to PHR and BPRE, only if there is data */

if((sched_ctrl->pcmax != 0 || sched_ctrl->ph != 0) && B > 0)

nr_ue_max_mcs_min_rb(current_BWP->scs, sched_ctrl->ph, &sched, current_BWP, min_rb, B, &available_rb, &sched.mcs);

if (sched.mcs < sched_ctrl->ul_bler_stats.mcs)

sched_ctrl->ul_bler_stats.mcs = sched.mcs; /* force estimated MCS down */

update_ul_ue_R_Qm(sched.mcs, current_BWP->mcs_table, current_BWP->pusch_Config, &sched.R, &sched.Qm);

if (has_dynamic_config) {

/* Force fixed PRB size for dynamically configured UE */

sched.rbSize = dynamic_prb_size;

sched.tb_size = nr_compute_tbs(sched.Qm,

sched.R,

sched.rbSize,

sched.tda_info.nrOfSymbols,

sched.dmrs_info.N_PRB_DMRS * sched.dmrs_info.num_dmrs_symb,

0,

0,

sched.nrOfLayers) >> 3;

} else if (!iterator->sched_inactive) {

nr_find_nb_rb(sched.Qm,

sched.R,

current_BWP->transform_precoding,

sched.nrOfLayers,

sched.tda_info.nrOfSymbols,

sched.dmrs_info.N_PRB_DMRS * sched.dmrs_info.num_dmrs_symb,

B,

min_rb,

available_rb,

&sched.tb_size,

&sched.rbSize);

} else {

sched.rbSize = min_rb;

sched.tb_size = nr_compute_tbs(sched.Qm,

sched.R,

sched.rbSize,

sched.tda_info.nrOfSymbols,

sched.dmrs_info.N_PRB_DMRS * sched.dmrs_info.num_dmrs_symb,

0,

0,

sched.nrOfLayers) >> 3;

}

// Calacualte the normalized tx_power for PHR

long *deltaMCS = current_BWP->pusch_Config ? current_BWP->pusch_Config->pusch_PowerControl->deltaMCS : NULL;

int tbs_bits = sched.tb_size << 3;

sched.phr_txpower_calc = compute_ph_factor(current_BWP->scs,

tbs_bits,

sched.rbSize,

sched.nrOfLayers,

sched.tda_info.nrOfSymbols,

sched.dmrs_info.N_PRB_DMRS * sched.dmrs_info.num_dmrs_symb,

deltaMCS,

false);

LOG_D(NR_MAC,

"rbSize %d (available_rb %d), TBS %d, est buf %d, sched_ul %d, B %d, CCE %d, num_dmrs_symb %d, N_PRB_DMRS %d\n",

sched.rbSize,

available_rb,

sched.tb_size,

sched_ctrl->estimated_ul_buffer,

sched_ctrl->sched_ul_bytes,

B,

sched_ctrl->cce_index,

sched.dmrs_info.num_dmrs_symb,

sched.dmrs_info.N_PRB_DMRS);

/* Mark the corresponding RBs as used */

sched_ctrl->cce_index = CCEIndex;

fill_pdcch_vrb_map(nrmac, CC_id, &sched_ctrl->sched_pdcch, CCEIndex, sched_ctrl->aggregation_level, dci_beam.idx);

/* save allocation to FAPI structures */

post_process_ulsch(nrmac, pp_pusch, iterator->UE, &sched);

n_rb_sched[beam.idx] -= sched.rbSize;

/* Mark PRBs as used - respect dynamic allocation boundaries */

if (has_dynamic_config) {

/* For dynamic allocation, only mark PRBs within the dynamic range */

int dynamic_start = dynamic_prb_start;

int dynamic_end = dynamic_start + dynamic_prb_size;

int actual_start = max(dynamic_start, sched.rbStart);

int actual_end = min(dynamic_end, sched.rbStart + sched.rbSize);

for (int rb = actual_start; rb < actual_end; rb++) {

rballoc_mask[rb + bi.bwpStart] |= slbitmap;

}

LOG_D(NR_MAC, "[UE %04x] Marked dynamic PRBs %d-%d as used\n",

iterator->UE->rnti, actual_start, actual_end - 1);

} else {

/* Dynamic allocation - mark all allocated PRBs */

for (int rb = 0; rb < sched.rbSize; rb++) {

rballoc_mask[rb + sched.rbStart + bi.bwpStart] |= slbitmap;

}

}

/* reduce max_num_ue once we are sure UE can be allocated, i.e., has CCE */

remainUEs[beam.idx]--;

iterator++;

scheduled_something = true;

}

int num_ue_sched = num_beams * max_num_ue;

for (int i = 0; i < num_beams; i++)

num_ue_sched -= remainUEs[i];

DevAssert((num_ue_sched == 0 && !scheduled_something) || (num_ue_sched > 0 && scheduled_something));

return num_ue_sched;

}

nfapi_nr_pusch_pdu_t *prepare_pusch_pdu(nfapi_nr_ul_tti_request_t *future_ul_tti_req,

const NR_UE_info_t *UE,

const NR_ServingCellConfigCommon_t *scc,

const NR_sched_pusch_t *sched_pusch,

int transform_precoding,

int harq_id,

int harq_round,

int fh,

int rnti)

{

nfapi_nr_pusch_pdu_t *pusch_pdu = &future_ul_tti_req->pdus_list[future_ul_tti_req->n_pdus].pusch_pdu;

memset(pusch_pdu, 0, sizeof(nfapi_nr_pusch_pdu_t));

const NR_UE_UL_BWP_t *ul_bwp = &UE->current_UL_BWP;

pusch_pdu->pdu_bit_map = PUSCH_PDU_BITMAP_PUSCH_DATA;

pusch_pdu->rnti = rnti;

pusch_pdu->handle = 0; //not yet used

pusch_pdu->bwp_size = sched_pusch->bwp_info.bwpSize;

pusch_pdu->bwp_start = sched_pusch->bwp_info.bwpStart;

pusch_pdu->subcarrier_spacing = ul_bwp->scs;

pusch_pdu->cyclic_prefix = 0;

pusch_pdu->target_code_rate = sched_pusch->R;

pusch_pdu->qam_mod_order = sched_pusch->Qm;

pusch_pdu->mcs_index = sched_pusch->mcs;

pusch_pdu->mcs_table = ul_bwp->mcs_table;

pusch_pdu->transform_precoding = transform_precoding;

if (ul_bwp->pusch_Config && ul_bwp->pusch_Config->dataScramblingIdentityPUSCH)

pusch_pdu->data_scrambling_id = *ul_bwp->pusch_Config->dataScramblingIdentityPUSCH;

else

pusch_pdu->data_scrambling_id = *scc->physCellId;

pusch_pdu->nrOfLayers = sched_pusch->nrOfLayers;

// DMRS

pusch_pdu->num_dmrs_cdm_grps_no_data = sched_pusch->dmrs_info.num_dmrs_cdm_grps_no_data;

pusch_pdu->dmrs_ports = ((1 << sched_pusch->nrOfLayers) - 1);

pusch_pdu->ul_dmrs_symb_pos = sched_pusch->dmrs_info.ul_dmrs_symb_pos;

pusch_pdu->dmrs_config_type = sched_pusch->dmrs_info.dmrs_config_type;

pusch_pdu->scid = sched_pusch->dmrs_info.scid; // DMRS sequence initialization [TS38.211, sec 6.4.1.1.1]

pusch_pdu->pusch_identity = sched_pusch->dmrs_info.pusch_identity;

pusch_pdu->ul_dmrs_scrambling_id = sched_pusch->dmrs_info.dmrs_scrambling_id;

/* Allocation in frequency domain */

pusch_pdu->resource_alloc = 1; //type 1

pusch_pdu->rb_start = sched_pusch->rbStart;

pusch_pdu->rb_size = sched_pusch->rbSize;

pusch_pdu->vrb_to_prb_mapping = 0;

pusch_pdu->frequency_hopping = fh;

/* Resource Allocation in time domain */

pusch_pdu->start_symbol_index = sched_pusch->tda_info.startSymbolIndex;

pusch_pdu->nr_of_symbols = sched_pusch->tda_info.nrOfSymbols;

/* PUSCH PDU */

pusch_pdu->pusch_data.rv_index = nr_get_rv(harq_round % 4);

pusch_pdu->pusch_data.harq_process_id = harq_id;

pusch_pdu->pusch_data.new_data_indicator = (harq_round == 0) ? 1 : 0; // not NDI but indicator for new transmission

pusch_pdu->pusch_data.tb_size = sched_pusch->tb_size;

pusch_pdu->pusch_data.num_cb = 0; // CBG not supported

// Beamforming

pusch_pdu->beamforming.num_prgs = 1;

pusch_pdu->beamforming.prg_size = pusch_pdu->bwp_size;

pusch_pdu->beamforming.dig_bf_interface = 1;

pusch_pdu->beamforming.prgs_list[0].dig_bf_interface_list[0].beam_idx = UE->UE_beam_index;

/* TRANSFORM PRECODING --------------------------------------------------------*/

if (pusch_pdu->transform_precoding == NR_PUSCH_Config__transformPrecoder_enabled) {

// U as specified in section 6.4.1.1.1.2 in 38.211, if sequence hopping and group hopping are disabled

pusch_pdu->dfts_ofdm.low_papr_group_number = sched_pusch->dmrs_info.pusch_identity % 30;

pusch_pdu->dfts_ofdm.low_papr_sequence_number = sched_pusch->dmrs_info.low_papr_sequence_number;

}

pusch_pdu->maintenance_parms_v3.ldpcBaseGraph = get_BG(sched_pusch->tb_size << 3, sched_pusch->R);

pusch_pdu->maintenance_parms_v3.tbSizeLbrmBytes = 0;

if (UE->sc_info.rateMatching_PUSCH) {

// TBS_LBRM according to section 5.4.2.1 of 38.212

long *maxMIMO_Layers = UE->sc_info.maxMIMO_Layers_PUSCH;

if (!maxMIMO_Layers && ul_bwp && ul_bwp->pusch_Config)

maxMIMO_Layers = ul_bwp->pusch_Config->maxRank;

AssertFatal (maxMIMO_Layers != NULL,"Option with max MIMO layers not configured is not supported\n");

pusch_pdu->maintenance_parms_v3.tbSizeLbrmBytes = nr_compute_tbslbrm(ul_bwp->mcs_table,

UE->sc_info.ul_bw_tbslbrm,

*maxMIMO_Layers);

}

/* PUSCH PTRS */

if (sched_pusch->dmrs_info.ptrsConfig) {

bool valid_ptrs_setup = false;

pusch_pdu->pusch_ptrs.ptrs_ports_list = calloc_or_fail(2, sizeof(nfapi_nr_ptrs_ports_t));

valid_ptrs_setup = set_ul_ptrs_values(sched_pusch->dmrs_info.ptrsConfig,

pusch_pdu->rb_size,

pusch_pdu->mcs_index,

pusch_pdu->mcs_table,

&pusch_pdu->pusch_ptrs.ptrs_freq_density,

&pusch_pdu->pusch_ptrs.ptrs_time_density,

&pusch_pdu->pusch_ptrs.ptrs_ports_list->ptrs_re_offset,

&pusch_pdu->pusch_ptrs.num_ptrs_ports,

&pusch_pdu->pusch_ptrs.ul_ptrs_power,

pusch_pdu->nr_of_symbols);

if (valid_ptrs_setup == true)

pusch_pdu->pdu_bit_map |= PUSCH_PDU_BITMAP_PUSCH_PTRS; // enable PUSCH PTRS

} else

pusch_pdu->pdu_bit_map &= ~PUSCH_PDU_BITMAP_PUSCH_PTRS; // disable PUSCH PTRS

return pusch_pdu;

}

void post_process_ulsch(gNB_MAC_INST *nr_mac, post_process_pusch_t *pusch, NR_UE_info_t *UE, NR_sched_pusch_t *sched_pusch)

{

frame_t frame = pusch->frame;

slot_t slot = pusch->slot;

NR_ServingCellConfigCommon_t *scc = nr_mac->common_channels[0].ServingCellConfigCommon;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_UL_BWP_t *current_BWP = &UE->current_UL_BWP;

/* the UE now has the grant for the request */

sched_ctrl->SR = false;

int8_t harq_id = sched_pusch->ul_harq_pid;

if (harq_id < 0) {

/* PP has not selected a specific HARQ Process, get a new one */

harq_id = sched_ctrl->available_ul_harq.head;

AssertFatal(harq_id >= 0, "no free HARQ process available for UE %04x\n", UE->rnti);

remove_front_nr_list(&sched_ctrl->available_ul_harq);

sched_pusch->ul_harq_pid = harq_id;

} else {

/* PP selected a specific HARQ process. Check whether it will be a new

* transmission or a retransmission, and remove from the corresponding

* list */

if (sched_ctrl->ul_harq_processes[harq_id].round == 0)

remove_nr_list(&sched_ctrl->available_ul_harq, harq_id);

else

remove_nr_list(&sched_ctrl->retrans_ul_harq, harq_id);

}

NR_UE_ul_harq_t *cur_harq = &sched_ctrl->ul_harq_processes[harq_id];

DevAssert(!cur_harq->is_waiting);

if (nr_mac->radio_config.disable_harq) {

finish_nr_ul_harq(sched_ctrl, harq_id);

} else {

add_tail_nr_list(&sched_ctrl->feedback_ul_harq, harq_id);

cur_harq->feedback_slot = sched_pusch->slot;

cur_harq->is_waiting = true;

}

/* Statistics */

AssertFatal(cur_harq->round < nr_mac->ul_bler.harq_round_max, "Indexing ulsch_rounds[%d] is out of bounds\n", cur_harq->round);

UE->mac_stats.ul.rounds[cur_harq->round]++;

/* Save information on MCS, TBS etc for the current initial transmission

* so we have access to it when retransmitting */

cur_harq->sched_pusch = *sched_pusch;

if (cur_harq->round == 0) {

UE->mac_stats.ulsch_total_bytes_scheduled += sched_pusch->tb_size;

sched_ctrl->sched_ul_bytes += sched_pusch->tb_size;

UE->mac_stats.ul.total_rbs += sched_pusch->rbSize;

} else {

UE->mac_stats.ul.total_rbs_retx += sched_pusch->rbSize;

}

UE->mac_stats.ul.current_bytes = sched_pusch->tb_size;

UE->mac_stats.ul.current_rbs = sched_pusch->rbSize;

sched_ctrl->last_ul_frame = sched_pusch->frame;

sched_ctrl->last_ul_slot = sched_pusch->slot;

LOG_D(NR_MAC,

"ULSCH/PUSCH: %4d.%2d RNTI %04x UL sched %4d.%2d DCI L %d start %2d RBS %3d TDA %2d dmrs_pos %x MCS "

"Table %2d MCS %2d nrOfLayers %2d num_dmrs_cdm_grps_no_data %2d TBS %4d HARQ PID %2d round %d RV %d NDI %d est %6d sched "

"%6d est BSR %6d TPC %d\n",

frame,

slot,

UE->rnti,

sched_pusch->frame,

sched_pusch->slot,

sched_ctrl->aggregation_level,

sched_pusch->rbStart,

sched_pusch->rbSize,

sched_pusch->time_domain_allocation,

sched_pusch->dmrs_info.ul_dmrs_symb_pos,

current_BWP->mcs_table,

sched_pusch->mcs,

sched_pusch->nrOfLayers,

sched_pusch->dmrs_info.num_dmrs_cdm_grps_no_data,

sched_pusch->tb_size,

harq_id,

cur_harq->round,

nr_get_rv(cur_harq->round % 4),

cur_harq->ndi,

sched_ctrl->estimated_ul_buffer,

sched_ctrl->sched_ul_bytes,

sched_ctrl->estimated_ul_buffer - sched_ctrl->sched_ul_bytes,

sched_ctrl->tpc0);

T(T_GNB_MAC_UL, T_INT(UE->rnti), T_INT(frame), T_INT(slot), T_INT(sched_pusch->mcs), T_INT(sched_pusch->tb_size));

/* PUSCH in a later slot, but corresponding DCI now! */

const int index = ul_buffer_index(sched_pusch->frame,

sched_pusch->slot,

nr_mac->frame_structure.numb_slots_frame,

nr_mac->UL_tti_req_ahead_size);

nfapi_nr_ul_tti_request_t *req = &nr_mac->UL_tti_req_ahead[0][index];

if (req->SFN != sched_pusch->frame || req->Slot != sched_pusch->slot)

LOG_W(NR_MAC,

"%d.%d future UL_tti_req's frame.slot %d.%d does not match PUSCH %d.%d\n",

frame,

slot,

req->SFN,

req->Slot,

sched_pusch->frame,

sched_pusch->slot);

AssertFatal(req->n_pdus < sizeof(req->pdus_list) / sizeof(req->pdus_list[0]),

"Invalid UL_tti_req_ahead->n_pdus %d\n",

req->n_pdus);

req->pdus_list[req->n_pdus].pdu_type = NFAPI_NR_UL_CONFIG_PUSCH_PDU_TYPE;

req->pdus_list[req->n_pdus].pdu_size = sizeof(nfapi_nr_pusch_pdu_t);

nfapi_nr_pusch_pdu_t *pusch_pdu = prepare_pusch_pdu(req,

UE,

scc,

sched_pusch,

current_BWP->transform_precoding,

harq_id,

cur_harq->round,

current_BWP->pusch_Config && current_BWP->pusch_Config->frequencyHopping,

UE->rnti);

req->n_pdus += 1;

// Calculate the normalized tx_power for PHR

long *deltaMCS = current_BWP->pusch_Config ? current_BWP->pusch_Config->pusch_PowerControl->deltaMCS : NULL;

int tbs_bits = pusch_pdu->pusch_data.tb_size << 3;

sched_pusch->phr_txpower_calc = compute_ph_factor(current_BWP->scs,

tbs_bits,

sched_pusch->rbSize,

sched_pusch->nrOfLayers,

sched_pusch->tda_info.nrOfSymbols,

sched_pusch->dmrs_info.N_PRB_DMRS * sched_pusch->dmrs_info.num_dmrs_symb,

deltaMCS,

false);

/* a PDCCH PDU groups DCIs per BWP and CORESET. Save a pointer to each

* allocated PDCCH so we can easily allocate UE's DCIs independent of any

* CORESET order */

NR_SearchSpace_t *ss = sched_ctrl->search_space;

NR_ControlResourceSet_t *coreset = sched_ctrl->coreset;

const int coresetid = coreset->controlResourceSetId;

nfapi_nr_dl_tti_pdcch_pdu_rel15_t *pdcch_pdu = pusch->pdcch_pdu_coreset[coresetid];

if (!pdcch_pdu) {

nfapi_nr_ul_dci_request_pdus_t *ul_dci_request_pdu = &pusch->ul_dci_req->ul_dci_pdu_list[pusch->ul_dci_req->numPdus];

memset(ul_dci_request_pdu, 0, sizeof(nfapi_nr_ul_dci_request_pdus_t));

ul_dci_request_pdu->PDUType = NFAPI_NR_DL_TTI_PDCCH_PDU_TYPE;

ul_dci_request_pdu->PDUSize = (uint8_t)(4 + sizeof(nfapi_nr_dl_tti_pdcch_pdu));

pdcch_pdu = &ul_dci_request_pdu->pdcch_pdu.pdcch_pdu_rel15;

pusch->ul_dci_req->numPdus += 1;

nr_configure_pdcch(pdcch_pdu, coreset, &sched_ctrl->sched_pdcch);

pusch->pdcch_pdu_coreset[coresetid] = pdcch_pdu;

}

LOG_D(NR_MAC, "Configuring ULDCI/PDCCH in %d.%d at CCE %d, rnti %04x\n", frame, slot, sched_ctrl->cce_index, UE->rnti);

/* Fill PDCCH DL DCI PDU */

nfapi_nr_dl_dci_pdu_t *dci_pdu = prepare_dci_pdu(pdcch_pdu,

scc,

ss,

coreset,

sched_ctrl->aggregation_level,

sched_ctrl->cce_index,

UE->UE_beam_index,

UE->rnti);

pdcch_pdu->numDlDci++;

dci_pdu_rel15_t uldci_payload;

memset(&uldci_payload, 0, sizeof(uldci_payload));

if (current_BWP->dci_format == NR_UL_DCI_FORMAT_0_1)

LOG_D(NR_MAC_DCI,

"add ul dci harq %d for %d.%d %d.%d round %d\n",

harq_id,

frame,

slot,

sched_pusch->frame,

sched_pusch->slot,

sched_ctrl->ul_harq_processes[harq_id].round);

// If nrOfLayers is the same as in srs_feedback, we use the best TPMI, i.e. the one in srs_feedback.

// Otherwise, we use the valid TPMI that we saved in the first transmission.

int *tpmi = NULL;

if (pusch_pdu->nrOfLayers != (sched_ctrl->srs_feedback.ul_ri + 1))

tpmi = &sched_pusch->tpmi;

config_uldci(&UE->sc_info,

pusch_pdu,

&uldci_payload,

&sched_ctrl->srs_feedback,

tpmi,

sched_pusch->time_domain_allocation,

UE->UE_sched_ctrl.tpc0,

cur_harq->ndi,

current_BWP,

ss->searchSpaceType->present);

// Reset TPC to 0 dB to not request new gain multiple times before computing new value for SNR

UE->UE_sched_ctrl.tpc0 = 1;

fill_dci_pdu_rel15(&UE->sc_info,

&UE->current_DL_BWP,

current_BWP,

dci_pdu,

&uldci_payload,

current_BWP->dci_format,

TYPE_C_RNTI_,

current_BWP->bwp_id,

ss,

coreset,

UE->pdsch_HARQ_ACK_Codebook,

nr_mac->cset0_bwp_size);

}

fsn_t fs_add_delta(const frame_structure_t *fs, uint32_t delta, fsn_t fsn)

{

const int slots_frame = fs->numb_slots_frame;

fsn_t res = {

.f = (fsn.f + (fsn.s + delta) / slots_frame) % MAX_FRAME_NUMBER,

.s = (fsn.s + delta) % slots_frame,

};

return res;

}

int fs_get_diff(const frame_structure_t *fs, fsn_t a, fsn_t b)

{

const int slots_frame = fs->numb_slots_frame;

int diff = (a.f * slots_frame + a.s) - (b.f * slots_frame + b.s);

if (diff < -512 * slots_frame)

diff += 1024 * slots_frame;

else if (diff > 512 * slots_frame)

diff -= 1024 * slots_frame;

return diff;

}

fsn_t fs_get_max(const frame_structure_t *fs, fsn_t a, fsn_t b)

{

if (fs_get_diff(fs, a, b) <= 0) {

return b;

} else {

return a;

}

}

static void nr_ulsch_preprocessor(gNB_MAC_INST *nr_mac, post_process_pusch_t *pp_pusch)

{

int frame = pp_pusch->frame;

int slot = pp_pusch->slot;

NR_COMMON_channels_t *cc = nr_mac->common_channels;

NR_ServingCellConfigCommon_t *scc = cc->ServingCellConfigCommon;

AssertFatal(scc, "We need one serving cell config common\n");

const frame_structure_t *fs = &nr_mac->frame_structure;

// we assume the same K2 for all UEs

const int koffset = get_NTN_Koffset(scc);

const int min_rxtx = nr_mac->radio_config.minRXTXTIME + koffset;

int num_beams = nr_mac->beam_info.beam_allocation ? nr_mac->beam_info.beams_per_period : 1;

int bw = scc->uplinkConfigCommon->frequencyInfoUL->scs_SpecificCarrierList.list.array[0]->carrierBandwidth;

int average_agg_level = 4; // TODO find a better estimation

int max_dci = bw / (average_agg_level * NR_NB_REG_PER_CCE);

// FAPI cannot handle more than MAX_DCI_CORESET DCIs

max_dci = min(max_dci, MAX_DCI_CORESET);

fsn_t current = {frame, slot};

fsn_t min_next = fs_add_delta(fs, min_rxtx, current);

/* if it's the last DL slot, we should try all TDAs to make sure that the

* scheduler can reach e.g. the next mixed slot. Otherwise, if we don't, we

* might starve HARQ processes that need a retransmission in a specific slot

* but we might not necessarily reach it */

bool last_dl = (current.s % fs->numb_slots_period) == (fs->period_cfg.num_dl_slots - 1);

fsn_t *next = &nr_mac->ul_next;

while (max_dci > 0) {

/* go to the next UL slot, skipping DL if necessary */

*next = fs_get_max(fs, *next, min_next);

while (!is_ul_slot(next->s, fs))

*next = fs_add_delta(fs, 1, *next);

if (!is_dl_slot(current.s, fs)) // if current slot is not DL, nothing to do

break;

/* get a TDA so that next UL scheduling slot can be reached from current,

* and exit if there is no such TDA. Remove koffset, as it is

* "cell-specific", i.e., the UE will add it on the computation. */

int k2 = fs_get_diff(fs, *next, current) - koffset;

DevAssert(k2 > 0);

// we assume that all beams have the same symbol utilization in all RBs for

// simplification (but this might not be true). Otherwise, we would need to

// check it separately for all beams and give a list of TDAs (one for each

// beam) in pf_ul(), which is complex. We should rewrite pf_ul() to

// schedule for one beam only instead (and for one BWP).

int beam = 0;

const NR_tda_info_t *tda_info = NULL;

int n_tda = get_num_ul_tda(nr_mac, next->s, k2, &tda_info);

if (n_tda == 0) /* no TDA fulfills this */

break;

int rb_start = 0;

int rb_len = bw;

tda_info = get_best_ul_tda(nr_mac, beam, tda_info, n_tda, next->f, next->s, &rb_start, &rb_len);

DevAssert(tda_info->valid_tda);

int tda = seq_arr_dist(&nr_mac->ul_tda, seq_arr_front(&nr_mac->ul_tda), tda_info);

AssertFatal(tda >= 0 && tda < 16, "illegal TDA index %d\n", tda);

int len[num_beams];

for (int i = 0; i < num_beams; i++)

len[i] = rb_len;

/* proportional fair scheduling algorithm */

int sched = pf_ul(nr_mac, pp_pusch, tda, tda_info, nr_mac->UE_info.connected_ue_list, max_dci, num_beams, len);

LOG_D(NR_MAC, "run pf_ul() at %4d.%2d with tda %d k2 %d (ULSCH at %4d.%2d) scheduled %d last_dl %d\n", frame, slot, tda, k2, next->f, next->s, sched, last_dl);

/* if we did not schedule anything, and it's not the last slot, break. In

* the case we did schedule or it's the last slot (see above!), continue

* advancing till there is no TDA anymore */

if (sched == 0 && !last_dl)

break;

max_dci -= sched;

*next = fs_add_delta(fs, 1, *next);

}

}

nr_pp_impl_ul nr_init_ulsch_preprocessor(int CC_id)

{

/* during initialization: no mutex needed */

/* in the PF algorithm, we have to use the TBsize to compute the coefficient.

* This would include the number of DMRS symbols, which in turn depends on

* the time domain allocation. In case we are in a mixed slot, we do not want

* to recalculate all these values, and therefore we provide a look-up table

* which should approximately(!) give us the TBsize. In particular, the

* number of symbols, the number of DMRS symbols, and the exact Qm and R, are

* not correct*/

for (int mcsTableIdx = 0; mcsTableIdx < 5; ++mcsTableIdx) {

for (int mcs = 0; mcs < 29; ++mcs) {

if (mcs > 27 && (mcsTableIdx == 1 || mcsTableIdx == 3 || mcsTableIdx == 4))

continue;

const uint8_t Qm = nr_get_Qm_ul(mcs, mcsTableIdx);

const uint16_t R = nr_get_code_rate_ul(mcs, mcsTableIdx);

/* note: we do not update R/Qm based on low MCS or pi2BPSK */

ul_pf_tbs[mcsTableIdx][mcs] = nr_compute_tbs(Qm,

R,

1, /* rbSize */

10, /* hypothetical number of slots */

0, /* N_PRB_DMRS * N_DMRS_SLOT */

0 /* N_PRB_oh, 0 for initialBWP */,

0 /* tb_scaling */,

1 /* nrOfLayers */)

>> 3;

}

}

return nr_ulsch_preprocessor;

}

void nr_schedule_ulsch(module_id_t module_id, frame_t frame, slot_t slot, nfapi_nr_ul_dci_request_t *ul_dci_req)

{

gNB_MAC_INST *nr_mac = RC.nrmac[module_id];

/* already mutex protected: held in gNB_dlsch_ulsch_scheduler() */

NR_SCHED_ENSURE_LOCKED(&nr_mac->sched_lock);

/* Print dynamic PRB configuration on first call */

static bool config_printed = false;

if (!config_printed) {

print_dynamic_prb_config(frame, slot);

config_printed = true;

}

ul_dci_req->SFN = frame;

ul_dci_req->Slot = slot;

post_process_pusch_t pusch = { .frame = frame, .slot = slot, .ul_dci_req = ul_dci_req, .pdcch_pdu_coreset = {NULL}, };

nr_mac->pre_processor_ul(nr_mac, &pusch);

}