# Tab 1

Relation between the threshold I set vs the the buffer of ue (that runs iperf)

How even when iperf download is executed for lower bitrate than the threshold the mode change happens?

| What you set | What it measures |
| --- | --- |
| thr_on = 7,000,000 bytes | Instantaneous buffer size (backlog waiting in RLC) |
| 50 Mbps iperf | Data rate (how fast data arrives) |

They are not directly comparable. The threshold is not a rate — it is a snapshot of how much data is piled up at one moment.

Simple analogy — Water Tank

iperf server

(fills tank)     →   [ RLC Buffer / Tank ]   →   Scheduler drains

50 Mbps                                          (limited by PRBs)

- Inlet pipe = iperf pushing data in at 50 Mbps

- Outlet pipe = scheduler draining based on PRBs + MCS

- Tank level = num_total_bytes (what is compared to threshold)

The problem — inlet > outlet

With 25 PRBs at MCS 13:

Approx drain rate ≈ 25 PRBs × MCS13 × slots

≈ roughly 15–20 Mbps

So:

Filling rate  =  50 Mbps  =  6,250,000 bytes/sec

Drain rate    ≈  15 Mbps  =  1,875,000 bytes/sec

Net accumulation ≈ 34 Mbps = ~4,250,000 bytes/sec

Buffer grows by ~4.2 MB every second.

Your log proves this exactly

Frame 512.0  → buffer = 5,134,514 bytes

Frame 572.14 → buffer = 7,002,412 bytes

Time difference = ~60ms

Buffer grew:

7,002,412 - 5,134,514 = 1,867,898 bytes in 60ms

≈ 31 Mbps accumulation rate

This matches — inlet (50 Mbps) minus drain (~15 Mbps) = ~35 Mbps accumulation.

Key insight

The threshold 7,000,000 bytes would never be crossed only if the scheduler could drain data as fast or faster than iperf fills it.

For that you would need:

drain rate ≥ 50 Mbps

With 25 PRBs that is simply not possible. Even with 53 PRBs after sharing, MCS needs to be high enough to sustain a 50 Mbps drain rate.

So the real question is

Not "is iperf rate < threshold?"

But "can the scheduler drain the buffer faster than iperf fills it?"

If drain < fill → buffer grows → eventually crosses threshold → mode changes.

Think of it like a BUCKET

Rain (iperf)     →   [ Bucket ]   →   Hole at bottom (scheduler)

- Rain = iperf sending data

- Bucket = RLC buffer

- Hole = scheduler draining data via PRBs

- Water level = num_total_bytes

- Threshold = a mark drawn on the bucket (7,000,000 bytes)

Your situation

Rain speed  = 50 Mbps  (fast)

Hole size   = 25 PRBs  (small hole, drains slowly ~15 Mbps)

So every second:

- 50 Mbps water coming IN

- 15 Mbps water going OUT

- 35 Mbps water stays in bucket → level RISES

The threshold mark on bucket

The 7,000,000 bytes mark is just a LINE drawn on the bucket.

It does NOT care about rain speed.

It only asks ONE question:

"Is the water level above this line right now?"

Why mode changed even at 50 Mbps

Because:

Rain (50 Mbps) >> Hole (15 Mbps)

→ bucket fills up

→ water level crosses the 7MB mark

→ mode changes

It would happen even at 20 Mbps because the hole is only 15 Mbps.

When would mode NOT change?

Only if:

Rain speed < Hole drain speed

For example:

- iperf at 10 Mbps (rain)

- Scheduler drains at 15 Mbps (hole)

- Bucket never fills up → level stays low → threshold never crossed

One line summary

The threshold checks how full the bucket is, not how fast the rain is.

With the below html file we can visually see how it works:
two_ue_rlc_buffer_bucket.html

# presentation images

Multiple PDU sessions

[NR_MAC] Frame.Slot 640.0 UE RNTI 945d CU-UE-ID 1 in-sync PH 34 dB PCMAX 20 dBm, average RSRP -100 (16 meas) UE 945d: CQI 15, RI 1, PMI (0,0) UE 945d: UL-RI 1, TPMI 0 UE 945d: dlsch_rounds 15717/895/0/0, dlsch_errors 0, pucch0_DTX 0 (SNR 22.0+2.0 dB), BLER 0.00000 MCS (0) 0 CCE fail 6, goodput 0.00 Mbps UE 945d: ulsch_rounds 44216/5/1/1, ulsch_errors 1, ulsch_DTX 4, BLER 0.00000 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 5 SNR 17.5 (+2.5) dB CCE fail 0, goodput 0.00 Mbps UE 945d: LCID 1: TX 5172 RX 564 bytes UE 945d: LCID 2: TX 0 RX 0 bytes UE 945d: LCID 4: TX 12908728 RX 8105 bytes ------> PDU 1 UE 945d: LCID 5: TX 12888694 RX 6529 bytes ------> PDU 2 UE 945d: LCID 6: TX 12875712 RX 6954 bytes ------> PDU 3 UE 945d: LCID 7: TX 12936431 RX 9805 bytes ------> PDU 4 UE RNTI ccc1 CU-UE-ID 2 in-sync PH 34 dB PCMAX 20 dBm, average RSRP -99 (16 meas) UE ccc1: CQI 15, RI 1, PMI (0,0) UE ccc1: UL-RI 1, TPMI 0 UE ccc1: dlsch_rounds 28866/3769/612/139, dlsch_errors 122, pucch0_DTX 627 (SNR 21.8+1.8 dB), BLER 0.0 0000 MCS (0) 0 CCE fail 0, goodput 0.00 Mbps UE ccc1: ulsch_rounds 45481/911/243/214, ulsch_errors 208, ulsch_DTX 343, BLER 0.00000 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 5 SNR 17.0 (+2.0) dB CCE fail 0, goodput 0.00 Mbps UE ccc1: LCID 1: TX 2820 RX 773 bytes UE ccc1: LCID 2: TX 0 RX 0 bytes UE ccc1: LCID 4: TX 13030885 RX 6563 bytes ------> PDU 1 UE ccc1: LCID 5: TX 11307640 RX 5754 bytes ------> PDU 2 UE ccc1: LCID 6: TX 1864457 RX 13023 bytes ------> PDU 3 UE ccc1: LCID 7: TX 12929878 RX 16187 bytes ------> PDU 4

Time varying prbs

[NR_MAC] Frame.Slot 896.0 UE RNTI 2947 CU-UE-ID 1 in-sync PH 40 dB PCMAX 20 dBm, average RSRP -96 (16 meas) UE 2947: UL-RI 1, TPMI 0 UE 2947: dlsch_rounds 50767/3948/350/54, dlsch_errors 26, pucch0_DTX 532, BLER 0.00428 MCS (0) 8 CCE fail 150 UE 2947: ulsch_rounds 34041/924/216/167, ulsch_errors 137, ulsch_DTX 564, BLER 0.00000 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 40 SNR 38.0 dB CCE fail 0 UE 2947: MAC: TX 199763550 RX 32616189 bytes UE 2947: LCID 1: TX 2952 RX 457 bytes UE 2947: LCID 2: TX 0 RX 0 bytes UE 2947: LCID 4: TX 196034047 RX 26230677 bytes UE RNTI 688f CU-UE-ID 2 in-sync PH 35 dB PCMAX 20 dBm, average RSRP -101 (16 meas) UE 688f: UL-RI 1, TPMI 0 UE 688f: dlsch_rounds 96424/7132/854/461, dlsch_errors 62, pucch0_DTX 371, BLER 0.04177 MCS (0) 6 CCE fail 81 UE 688f: ulsch_rounds 56456/4058/120/71, ulsch_errors 51, ulsch_DTX 403, BLER 0.00080 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 20 SNR 26.0 dB CCE fail 0 UE 688f: MAC: TX 155011783 RX 65299029 bytes UE 688f: LCID 1: TX 552 RX 301 bytes UE 688f: LCID 2: TX 0 RX 0 bytes UE 688f: LCID 4: TX 151891171 RX 58104188 bytes [NR_MAC] Timing Debug: Frame 950, Slot 1, Absolute Time 19.000s, Cyclic Time 19.000s [NR_MAC] Timing Debug: Frame 1000, Slot 1, Absolute Time 20.000s, Cyclic Time 0.000s [NR_MAC] UE-ID 0x2947: Starting new 10s cycle 1 (time reset to 0) [NR_MAC] [UE 2947] Dynamic PRB: start=0, size=20, config_idx=0 [NR_MAC] UE-ID 0x688f: Starting new 10s cycle 1 (time reset to 0) [NR_MAC] Frame.Slot 0.0 UE RNTI 2947 CU-UE-ID 1 in-sync PH 42 dB PCMAX 20 dBm, average RSRP -96 (16 meas) UE 2947: UL-RI 1, TPMI 0 UE 2947: dlsch_rounds 51524/3954/350/54, dlsch_errors 26, pucch0_DTX 538, BLER 0.00808 MCS (0) 12 CCE fail 150 UE 2947: ulsch_rounds 34284/926/217/167, ulsch_errors 137, ulsch_DTX 566, BLER 0.01500 MCS (0) 7 (Qm 2 deltaMCS 0 dB) NPRB 20 SNR 38.5 dB CCE fail 0 UE 2947: MAC: TX 201337593 RX 32669399 bytes UE 2947: LCID 1: TX 2952 RX 457 bytes UE 2947: LCID 2: TX 0 RX 0 bytes UE 2947: LCID 4: TX 197601644 RX 26269673 bytes UE RNTI 688f CU-UE-ID 2 in-sync PH 35 dB PCMAX 20 dBm, average RSRP -101 (16 meas) UE 688f: UL-RI 1, TPMI 0 UE 688f: dlsch_rounds 96425/7132/854/461, dlsch_errors 62, pucch0_DTX 371, BLER 0.03759 MCS (0) 5 CCE fail 81 UE 688f: ulsch_rounds 56469/4058/120/71, ulsch_errors 51, ulsch_DTX 403, BLER 0.00020 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 40 SNR 26.5 dB CCE fail 0 UE 688f: MAC: TX 155011846 RX 65300312 bytes UE 688f: LCID 1: TX 552 RX 301 bytes UE 688f: LCID 2: TX 0 RX 0 bytes UE 688f: LCID 4: TX 151891171 RX 58104188 bytes

Different static PRBs for two UEs

[NR_MAC] Frame.Slot 384.0 UE RNTI 520f CU-UE-ID 1 in-sync PH 28 dB PCMAX 20 dBm, average RSRP -110 (16 meas) UE 520f: UL-RI 1, TPMI 0 UE 520f: dlsch_rounds 45292/3581/230/30, dlsch_errors 16, pucch0_DTX 236, BLER 0.00448 MCS (0) 7 CCE fail 397 UE 520f: ulsch_rounds 29618/255/47/23, ulsch_errors 11, ulsch_DTX 306, BLER 0.00000 MCS (0) 0 (Qm 2 deltaMCS 0 dB) NPRB 25 SNR 36.5 dB CCE fail 0 UE 520f: MAC: TX 151096037 RX 27029562 bytes UE 520f: LCID 1: TX 1552 RX 394 bytes UE 520f: LCID 2: TX 0 RX 0 bytes UE 520f: LCID 4: TX 148675753 RX 22175153 bytes UE RNTI d32b CU-UE-ID 2 in-sync PH 36 dB PCMAX 20 dBm, average RSRP -100 (16 meas) UE d32b: UL-RI 1, TPMI 0 UE d32b: dlsch_rounds 19499/1027/4/0, dlsch_errors 0, pucch0_DTX 2, BLER 0.01618 MCS (0) 18 CCE fail 30 UE d32b: ulsch_rounds 11030/99/6/0, ulsch_errors 0, ulsch_DTX 7, BLER 0.00014 MCS (0) 17 (Qm 6 deltaMCS 0 dB) NPRB 55 SNR 36.0 dB CCE fail 0 UE d32b: MAC: TX 118588408 RX 25572831 bytes UE d32b: LCID 1: TX 552 RX 289 bytes UE d32b: LCID 2: TX 0 RX 0 bytes UE d32b: LCID 4: TX 116398842 RX 20548814 bytes

Max. MCS & PRBs

[NR_MAC] Frame.Slot 768.0 UE RNTI c0bf CU-UE-ID 1 in-sync PH 37 dB PCMAX 20 dBm, average RSRP -99 (16 meas) UE c0bf: UL-RI 1, TPMI 0 UE c0bf: dlsch_rounds 18405/690/3/0, dlsch_errors 0, pucch0_DTX 0, BLER 0.00000 MCS (0) 28 CCE fail 2 UE c0bf: ulsch_rounds 8852/1875/1/0, ulsch_errors 0, ulsch_DTX 1, BLER 0.53363 MCS (0) 28 (Qm 6 deltaMCS 0 dB) NPRB 106 SNR 34.5 dB CCE fail 0 UE c0bf: MAC: TX 97327153 RX 63345032 bytes UE c0bf: LCID 1: TX 544 RX 265 bytes UE c0bf: LCID 2: TX 0 RX 0 bytes UE c0bf: LCID 4: TX 94342682 RX 45829527 bytes

OAI’s Management Plane monitoring

"bwp3gpp:isInitialBWP": true, "bwp3gpp:numberOfRBs": 106, "bwp3gpp:startRB": 0, "bwp3gpp:subCarrierSpacing": 30 }], "ul": [{ "bwp3gpp:isInitialBWP": true, "bwp3gpp:numberOfRBs": 106, "bwp3gpp:startRB": 0, "bwp3gpp:subCarrierSpacing": 30 }] }, "NRCELLDU": { "nrcelldu3gpp:ssbFrequency": 641280, "nrcelldu3gpp:arfcnDL": 640008, "nrcelldu3gpp:bSChannelBwDL": 40, "nrcelldu3gpp:arfcnUL": 640008, "nrcelldu3gpp:bSChannelBwUL": 40, "nrcelldu3gpp:nRPCI": 0, "nrcelldu3gpp:nRTAC": 1, "nrcelldu3gpp:mcc": "001", "nrcelldu3gpp:mnc": "01", "nrcelldu3gpp:sd": 16777215, "nrcelldu3gpp:sst": 1 }, "device": { "gnbId": 3584, "gnbName": "gNB-OAI", "vendor": "OpenAirInterface" } }, "O1-Operational": { "frame-type": "tdd", "band-number": 78, "num-ues": 2, "ues": [29802, 50422], "load": 37, "ues-thp": [ {"rnti": 29802, "dl": 11186, "ul": 108}, {"rnti": 50422, "dl": 41908, "ul": 306} ] } } OK softmodem_gnb>

Doppler estimation and compensation

[PHY] Doppler compensation done (UL): frame 620 slot 7 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (UL): frame 620 slot 7 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (DL): frame 620 slot 17 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (DL): frame 620 slot 17 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (UL): frame 620 slot 17 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (UL): frame 620 slot 17 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (DL): frame 621 slot 7 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (DL): frame 621 slot 7 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (UL): frame 621 slot 7 UE f90a shift -0.94 Hz [PHY] Doppler compensation done (UL): frame 621 slot 7 UE f90a shift -0.94 Hz [NR_MAC] UE f90a Doppler estimate: raw -0.55 Hz, filtered -0.86 Hz at 624.8 (phase -1.301 rad) [PHY] Doppler compensation done (UL): frame 631 slot 8 UE f90a shift -0.86 Hz [PHY] Doppler compensation done (DL): frame 631 slot 8 UE f90a shift -0.86 Hz [NR_MAC] UE f90a Doppler estimate: raw -0.18 Hz, filtered -0.73 Hz at 632.8 (phase -0.955 rad)

Per PDU session resource allocation

[NR_MAC] Frame.Slot 0.0 UE RNTI 4d08 CU-UE-ID 1 in-sync PH 44 dB PCMAX 20 dBm, average RSRP -93 (16 meas) UE 4d08: CQI 15, RI 1, PMI (0,0) UE 4d08: UL-RI 1, TPMI 0 UE 4d08: dlsch_rounds 2389/6/2/0, dlsch_errors 0, pucch0_DTX 8 (SNR 21.8+1.8 dB), BLER 0.00159 MCS (0) 15 NPRB 106 CCE fail 6, goodput 22.33 Mbps UE 4d08: ulsch_rounds 5355/26/3/2, ulsch_errors 2, ulsch_DTX 21, BLER 0.00928 MCS (0) 15 (Qm 4 deltaMCS 0 dB) NPRB 5 SNR 16.6 (+1.6) dB CCE fail 0, goodput 0.03 Mbps UE 4d08: LCID 1: TX 1851 RX 542 bytes, PRB-equiv(slot) 0/106, PRB-equiv(cumulative) 518 UE 4d08: LCID 2: TX 0 RX 0 bytes, PRB-equiv(slot) 0/106, PRB-equiv(cumulative) 0 UE 4d08: LCID 4: TX 1186071 RX 2010 bytes, PRB-equiv(slot) 37/106, PRB-equiv(cumulative) 60899 ------> PDU 1 ( ≈ 35 % ) UE 4d08: LCID 5: TX 1149706 RX 1960 bytes, PRB-equiv(slot) 36/106, PRB-equiv(cumulative) 58296 ------> PDU 2 ( ≈ 35 % ) UE 4d08: LCID 6: TX 510219 RX 1712 bytes, PRB-equiv(slot) 15/106, PRB-equiv(cumulative) 27435 ------> PDU 3 ( ≈ 15 % ) UE 4d08: LCID 7: TX 466464 RX 2219 bytes, PRB-equiv(slot) 15/106, PRB-equiv(cumulative) 23430 ------> PDU 4 ( ≈ 15 % )