/*

* Licensed to the OpenAirInterface (OAI) Software Alliance under one or more

* contributor license agreements.  See the NOTICE file distributed with

* this work for additional information regarding copyright ownership.

* The OpenAirInterface Software Alliance licenses this file to You under

* the OAI Public License, Version 1.1  (the "License"); you may not use this file

* except in compliance with the License.

* You may obtain a copy of the License at

*

*      http://www.openairinterface.org/?page_id=698

*

* Unless required by applicable law or agreed to in writing, software

* distributed under the License is distributed on an "AS IS" BASIS,

* WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

* See the License for the specific language governing permissions and

* limitations under the License.

*-------------------------------------------------------------------------------

* For more information about the OpenAirInterface (OAI) Software Alliance:

*      contact@openairinterface.org

*/

/*! \file       gNB_scheduler_dlsch.c

* \brief       procedures related to gNB for the DLSCH transport channel

* \author      Guido Casati

* \date        2019

* \email:      guido.casati@iis.fraunhofe.de

* \version     1.0

* @ingroup     _mac

*/

#include "common/utils/nr/nr_common.h"

/*MAC*/

#include "NR_MAC_COMMON/nr_mac.h"

#include "NR_MAC_gNB/nr_mac_gNB.h"

#include "LAYER2/NR_MAC_gNB/mac_proto.h"

#include "openair2/LAYER2/nr_rlc/nr_rlc_oai_api.h"

#include "openair2/F1AP/f1ap_ids.h"

/*TAG*/

#include "NR_TAG-Id.h"

/*Softmodem params*/

#include "executables/softmodem-common.h"

#include "../../../nfapi/oai_integration/vendor_ext.h"

////////////////////////////////////////////////////////

/////* DLSCH MAC PDU generation (6.1.2 TS 38.321) */////

////////////////////////////////////////////////////////

#define OCTET 8

#define WORD 32

//#define SIZE_OF_POINTER sizeof (void *)

int get_dl_tda(const gNB_MAC_INST *nrmac, int slot)

{

/* we assume that this function is mutex-protected from outside */

const frame_structure_t *fs = &nrmac->frame_structure;

// Use special TDA in case of CSI-RS

if (nrmac->UE_info.sched_csirs > 0)

return 1;

if (fs->frame_type == TDD) {

int s = get_slot_idx_in_period(slot, fs);

// if there is a mixed slot where we can transmit DL

const tdd_bitmap_t *tdd_slot_bitmap = fs->period_cfg.tdd_slot_bitmap;

if (tdd_slot_bitmap[s].num_dl_symbols > 1 && is_mixed_slot(s, fs)) {

return 2;

}

}

return 0; // if FDD or not mixed slot in TDD, for now use default TDA

}

// Compute and write all MAC CEs and subheaders, and return number of written bytes

int nr_write_ce_dlsch_pdu(module_id_t module_idP,

const NR_UE_sched_ctrl_t *ue_sched_ctl,

unsigned char *mac_pdu,

unsigned char drx_cmd,

unsigned char *ue_cont_res_id)

{

gNB_MAC_INST *gNB = RC.nrmac[module_idP];

/* already mutex protected: called below and in _RA.c */

NR_SCHED_ENSURE_LOCKED(&gNB->sched_lock);

NR_MAC_SUBHEADER_FIXED *mac_pdu_ptr = (NR_MAC_SUBHEADER_FIXED *) mac_pdu;

uint8_t last_size = 0;

int offset = 0, mac_ce_size, i, timing_advance_cmd, tag_id = 0;

// MAC CEs

uint8_t mac_header_control_elements[16], *ce_ptr;

ce_ptr = &mac_header_control_elements[0];

// DRX command subheader (MAC CE size 0)

if (drx_cmd != 255) {

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_DRX;

//last_size = 1;

mac_pdu_ptr++;

}

// Timing Advance subheader

/* This was done only when timing_advance_cmd != 31

// now TA is always send when ta_timer resets regardless of its value

// this is done to avoid issues with the timeAlignmentTimer which is

// supposed to monitor if the UE received TA or not */

if (ue_sched_ctl->ta_apply) {

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_TA_COMMAND;

//last_size = 1;

mac_pdu_ptr++;

// TA MAC CE (1 octet)

timing_advance_cmd = ue_sched_ctl->ta_update;

AssertFatal(timing_advance_cmd < 64, "timing_advance_cmd %d > 63\n", timing_advance_cmd);

((NR_MAC_CE_TA *) ce_ptr)->TA_COMMAND = timing_advance_cmd;    //(timing_advance_cmd+31)&0x3f;

tag_id = gNB->tag->tag_Id;

((NR_MAC_CE_TA *) ce_ptr)->TAGID = tag_id;

LOG_D(NR_MAC, "NR MAC CE timing advance command = %d (%d) TAG ID = %d\n", timing_advance_cmd, ((NR_MAC_CE_TA *) ce_ptr)->TA_COMMAND, tag_id);

mac_ce_size = sizeof(NR_MAC_CE_TA);

// Copying  bytes for MAC CEs to the mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *) ce_ptr, mac_ce_size);

ce_ptr += mac_ce_size;

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

// Contention resolution fixed subheader and MAC CE

if (ue_cont_res_id) {

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_CON_RES_ID;

mac_pdu_ptr++;

//last_size = 1;

// contention resolution identity MAC ce has a fixed 48 bit size

// this contains the UL CCCH SDU. If UL CCCH SDU is longer than 48 bits,

// it contains the first 48 bits of the UL CCCH SDU

LOG_D(NR_MAC,

"[gNB ][RAPROC] Generate contention resolution msg: %x.%x.%x.%x.%x.%x\n",

ue_cont_res_id[0],

ue_cont_res_id[1],

ue_cont_res_id[2],

ue_cont_res_id[3],

ue_cont_res_id[4],

ue_cont_res_id[5]);

// Copying bytes (6 octects) to CEs pointer

mac_ce_size = 6;

memcpy(ce_ptr, ue_cont_res_id, mac_ce_size);

// Copying bytes for MAC CEs to mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *) ce_ptr, mac_ce_size);

ce_ptr += mac_ce_size;

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

//TS 38.321 Sec 6.1.3.15 TCI State indication for UE Specific PDCCH MAC CE SubPDU generation

if (ue_sched_ctl->UE_mac_ce_ctrl.tci_state_ind.is_scheduled) {

//filling subheader

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_TCI_STATE_IND_UE_SPEC_PDCCH;

mac_pdu_ptr++;

//Creating the instance of CE structure

const tciStateInd_t *tcisi = &ue_sched_ctl->UE_mac_ce_ctrl.tci_state_ind;

NR_TCI_PDCCH  nr_UESpec_TCI_StateInd_PDCCH = {

.CoresetId1 = (tcisi->coresetId & 0xF) >> 1,

.ServingCellId = 0, // TODO this is likely ServingCellIndex, see 38.331

.TciStateId = tcisi->tciStateId & 0x7F,

.CoresetId2 = tcisi->coresetId & 0x1,

};

LOG_I(NR_MAC, "NR MAC CE TCI state indication for UE Specific PDCCH = %d \n", nr_UESpec_TCI_StateInd_PDCCH.TciStateId);

mac_ce_size = sizeof(NR_TCI_PDCCH);

// Copying  bytes for MAC CEs to the mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *)&nr_UESpec_TCI_StateInd_PDCCH, mac_ce_size);

//incrementing the PDU pointer

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

//TS 38.321 Sec 6.1.3.16, SP CSI reporting on PUCCH Activation/Deactivation MAC CE

if (ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.is_scheduled) {

//filling the subheader

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_SP_CSI_REP_PUCCH_ACT;

mac_pdu_ptr++;

//creating the instance of CE structure

NR_PUCCH_CSI_REPORTING nr_PUCCH_CSI_reportingActDeact;

//filling the CE structure

nr_PUCCH_CSI_reportingActDeact.BWP_Id = (ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.bwpId) & 0x3; //extracting LSB 2 bibs

nr_PUCCH_CSI_reportingActDeact.ServingCellId = (ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.servingCellId) & 0x1F; //extracting LSB 5 bits

nr_PUCCH_CSI_reportingActDeact.S0 = ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.s0tos3_actDeact[0];

nr_PUCCH_CSI_reportingActDeact.S1 = ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.s0tos3_actDeact[1];

nr_PUCCH_CSI_reportingActDeact.S2 = ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.s0tos3_actDeact[2];

nr_PUCCH_CSI_reportingActDeact.S3 = ue_sched_ctl->UE_mac_ce_ctrl.SP_CSI_reporting_pucch.s0tos3_actDeact[3];

nr_PUCCH_CSI_reportingActDeact.R2 = 0;

mac_ce_size = sizeof(NR_PUCCH_CSI_REPORTING);

// Copying MAC CE data to the mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *)&nr_PUCCH_CSI_reportingActDeact, mac_ce_size);

//incrementing the PDU pointer

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

//TS 38.321 Sec 6.1.3.14, TCI State activation/deactivation for UE Specific PDSCH MAC CE

if (ue_sched_ctl->UE_mac_ce_ctrl.pdsch_TCI_States_ActDeact.is_scheduled) {

//Computing the number of octects to be allocated for Flexible array member

//of MAC CE structure

uint8_t num_octects = (ue_sched_ctl->UE_mac_ce_ctrl.pdsch_TCI_States_ActDeact.highestTciStateActivated) / 8 + 1; //Calculating the number of octects for allocating the memory

//filling the subheader

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->R = 0;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->F = 0;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->LCID = DL_SCH_LCID_TCI_STATE_ACT_UE_SPEC_PDSCH;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->L = sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t);

last_size = 2;

//Incrementing the PDU pointer

mac_pdu_ptr += last_size;

//allocating memory for CE Structure

NR_TCI_PDSCH_APERIODIC_CSI *nr_UESpec_TCI_StateInd_PDSCH = (NR_TCI_PDSCH_APERIODIC_CSI *)malloc(sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t));

//initializing to zero

memset((void *)nr_UESpec_TCI_StateInd_PDSCH, 0, sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t));

//filling the CE Structure

nr_UESpec_TCI_StateInd_PDSCH->BWP_Id = (ue_sched_ctl->UE_mac_ce_ctrl.pdsch_TCI_States_ActDeact.bwpId) & 0x3; //extracting LSB 2 Bits

nr_UESpec_TCI_StateInd_PDSCH->ServingCellId = (ue_sched_ctl->UE_mac_ce_ctrl.pdsch_TCI_States_ActDeact.servingCellId) & 0x1F; //extracting LSB 5 bits

for(i = 0; i < (num_octects * 8); i++) {

if(ue_sched_ctl->UE_mac_ce_ctrl.pdsch_TCI_States_ActDeact.tciStateActDeact[i])

nr_UESpec_TCI_StateInd_PDSCH->T[i / 8] = nr_UESpec_TCI_StateInd_PDSCH->T[i / 8] | (1 << (i % 8));

}

mac_ce_size = sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t);

//Copying  bytes for MAC CEs to the mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *)nr_UESpec_TCI_StateInd_PDSCH, mac_ce_size);

//incrementing the mac pdu pointer

mac_pdu_ptr += (unsigned char) mac_ce_size;

//freeing the allocated memory

free(nr_UESpec_TCI_StateInd_PDSCH);

}

//TS38.321 Sec 6.1.3.13 Aperiodic CSI Trigger State Subselection MAC CE

if (ue_sched_ctl->UE_mac_ce_ctrl.aperi_CSI_trigger.is_scheduled) {

//Computing the number of octects to be allocated for Flexible array member

//of MAC CE structure

uint8_t num_octects = (ue_sched_ctl->UE_mac_ce_ctrl.aperi_CSI_trigger.highestTriggerStateSelected) / 8 + 1; //Calculating the number of octects for allocating the memory

//filling the subheader

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->R = 0;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->F = 0;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->LCID = DL_SCH_LCID_APERIODIC_CSI_TRI_STATE_SUBSEL;

((NR_MAC_SUBHEADER_SHORT *) mac_pdu_ptr)->L = sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t);

last_size = 2;

//Incrementing the PDU pointer

mac_pdu_ptr += last_size;

//allocating memory for CE structure

NR_TCI_PDSCH_APERIODIC_CSI *nr_Aperiodic_CSI_Trigger = (NR_TCI_PDSCH_APERIODIC_CSI *)malloc(sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t));

//initializing to zero

memset((void *)nr_Aperiodic_CSI_Trigger, 0, sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t));

//filling the CE Structure

nr_Aperiodic_CSI_Trigger->BWP_Id = (ue_sched_ctl->UE_mac_ce_ctrl.aperi_CSI_trigger.bwpId) & 0x3; //extracting LSB 2 bits

nr_Aperiodic_CSI_Trigger->ServingCellId = (ue_sched_ctl->UE_mac_ce_ctrl.aperi_CSI_trigger.servingCellId) & 0x1F; //extracting LSB 5 bits

nr_Aperiodic_CSI_Trigger->R = 0;

for(i = 0; i < (num_octects * 8); i++) {

if(ue_sched_ctl->UE_mac_ce_ctrl.aperi_CSI_trigger.triggerStateSelection[i])

nr_Aperiodic_CSI_Trigger->T[i / 8] = nr_Aperiodic_CSI_Trigger->T[i / 8] | (1 << (i % 8));

}

mac_ce_size = sizeof(NR_TCI_PDSCH_APERIODIC_CSI) + num_octects * sizeof(uint8_t);

// Copying  bytes for MAC CEs to the mac pdu pointer

memcpy((void *) mac_pdu_ptr, (void *)nr_Aperiodic_CSI_Trigger, mac_ce_size);

//incrementing the mac pdu pointer

mac_pdu_ptr += (unsigned char) mac_ce_size;

//freeing the allocated memory

free(nr_Aperiodic_CSI_Trigger);

}

if (ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.is_scheduled) {

((NR_MAC_SUBHEADER_FIXED *) mac_pdu_ptr)->R = 0;

((NR_MAC_SUBHEADER_FIXED *) mac_pdu_ptr)->LCID = DL_SCH_LCID_SP_ZP_CSI_RS_RES_SET_ACT;

mac_pdu_ptr++;

((NR_MAC_CE_SP_ZP_CSI_RS_RES_SET *) mac_pdu_ptr)->A_D = ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.act_deact;

((NR_MAC_CE_SP_ZP_CSI_RS_RES_SET *) mac_pdu_ptr)->CELLID = ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.serv_cell_id & 0x1F; //5 bits

((NR_MAC_CE_SP_ZP_CSI_RS_RES_SET *) mac_pdu_ptr)->BWPID = ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.bwpid & 0x3; //2 bits

((NR_MAC_CE_SP_ZP_CSI_RS_RES_SET *) mac_pdu_ptr)->CSIRS_RSC_ID = ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.rsc_id & 0xF; //4 bits

((NR_MAC_CE_SP_ZP_CSI_RS_RES_SET *) mac_pdu_ptr)->R = 0;

LOG_D(NR_MAC, "NR MAC CE of ZP CSIRS Serv cell ID = %d BWPID= %d Rsc set ID = %d\n", ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.serv_cell_id, ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.bwpid,

ue_sched_ctl->UE_mac_ce_ctrl.sp_zp_csi_rs.rsc_id);

mac_ce_size = sizeof(NR_MAC_CE_SP_ZP_CSI_RS_RES_SET);

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

if (ue_sched_ctl->UE_mac_ce_ctrl.csi_im.is_scheduled) {

mac_pdu_ptr->R = 0;

mac_pdu_ptr->LCID = DL_SCH_LCID_SP_CSI_RS_CSI_IM_RES_SET_ACT;

mac_pdu_ptr++;

CSI_RS_CSI_IM_ACT_DEACT_MAC_CE csi_rs_im_act_deact_ce;

csi_rs_im_act_deact_ce.A_D = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.act_deact;

csi_rs_im_act_deact_ce.SCID = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.serv_cellid & 0x3F;//gNB_PHY -> ssb_pdu.ssb_pdu_rel15.PhysCellId;

csi_rs_im_act_deact_ce.BWP_ID = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.bwp_id;

csi_rs_im_act_deact_ce.R1 = 0;

csi_rs_im_act_deact_ce.IM = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.im;// IF set CSI IM Rsc id will presesent else CSI IM RSC ID is abscent

csi_rs_im_act_deact_ce.SP_CSI_RSID = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.nzp_csi_rsc_id;

if ( csi_rs_im_act_deact_ce.IM ) { //is_scheduled if IM is 1 else this field will not present

csi_rs_im_act_deact_ce.R2 = 0;

csi_rs_im_act_deact_ce.SP_CSI_IMID = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.csi_im_rsc_id;

mac_ce_size = sizeof ( csi_rs_im_act_deact_ce ) - sizeof ( csi_rs_im_act_deact_ce.TCI_STATE );

} else {

mac_ce_size = sizeof ( csi_rs_im_act_deact_ce ) - sizeof ( csi_rs_im_act_deact_ce.TCI_STATE ) - 1;

}

memcpy ((void *) mac_pdu_ptr, (void *) & ( csi_rs_im_act_deact_ce), mac_ce_size);

mac_pdu_ptr += (unsigned char) mac_ce_size;

if (csi_rs_im_act_deact_ce.A_D ) { //Following IE is_scheduled only if A/D is 1

mac_ce_size = sizeof ( struct TCI_S);

for ( i = 0; i < ue_sched_ctl->UE_mac_ce_ctrl.csi_im.nb_tci_resource_set_id; i++) {

csi_rs_im_act_deact_ce.TCI_STATE.R = 0;

csi_rs_im_act_deact_ce.TCI_STATE.TCI_STATE_ID = ue_sched_ctl->UE_mac_ce_ctrl.csi_im.tci_state_id [i] & 0x7F;

memcpy ((void *) mac_pdu_ptr, (void *) & (csi_rs_im_act_deact_ce.TCI_STATE), mac_ce_size);

mac_pdu_ptr += (unsigned char) mac_ce_size;

}

}

}

// compute final offset

offset = ((unsigned char *) mac_pdu_ptr - mac_pdu);

//printf("Offset %d \n", ((unsigned char *) mac_pdu_ptr - mac_pdu));

return offset;

}

static uint32_t update_dlsch_buffer(frame_t frame, slot_t slot, NR_UE_info_t *UE)

{

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

sched_ctrl->num_total_bytes = 0;

sched_ctrl->dl_pdus_total = 0;

/* loop over all activated logical channels */

for (int i = 0; i < seq_arr_size(&sched_ctrl->lc_config); ++i) {

const nr_lc_config_t *c = seq_arr_at(&sched_ctrl->lc_config, i);

const int lcid = c->lcid;

const uint16_t rnti = UE->rnti;

LOG_D(NR_MAC, "UE %x: LCID %d\n", rnti, lcid);

memset(&sched_ctrl->rlc_status[lcid], 0, sizeof(sched_ctrl->rlc_status[lcid]));

if (c->suspended)

continue;

if (lcid == DL_SCH_LCID_DTCH && nr_timer_is_active(&sched_ctrl->transm_interrupt))

continue;

sched_ctrl->rlc_status[lcid] = nr_mac_rlc_status_ind(rnti, frame, lcid);

if (sched_ctrl->rlc_status[lcid].bytes_in_buffer == 0)

continue;

sched_ctrl->dl_pdus_total += sched_ctrl->rlc_status[lcid].pdus_in_buffer;

sched_ctrl->num_total_bytes += sched_ctrl->rlc_status[lcid].bytes_in_buffer;

LOG_D(MAC,

"%4d.%2d UE %04x LCID %d status: %d bytes, total buffer %d bytes %d PDUs\n",

frame,

slot,

UE->rnti,

lcid,

sched_ctrl->rlc_status[lcid].bytes_in_buffer,

sched_ctrl->num_total_bytes,

sched_ctrl->dl_pdus_total);

}

return sched_ctrl->num_total_bytes;

}

void finish_nr_dl_harq(NR_UE_sched_ctrl_t *sched_ctrl, int harq_pid)

{

NR_UE_harq_t *harq = &sched_ctrl->harq_processes[harq_pid];

harq->ndi ^= 1;

harq->round = 0;

add_tail_nr_list(&sched_ctrl->available_dl_harq, harq_pid);

}

void abort_nr_dl_harq(NR_UE_info_t* UE, int8_t harq_pid)

{

/* already mutex protected through handle_dl_harq() */

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

finish_nr_dl_harq(sched_ctrl, harq_pid);

UE->mac_stats.dl.errors++;

}

bwp_info_t get_pdsch_bwp_start_size(gNB_MAC_INST *nr_mac, NR_UE_info_t *UE)

{

bwp_info_t bwp_info;

if (!UE) {

bwp_info.bwpStart = nr_mac->cset0_bwp_start;

bwp_info.bwpSize = nr_mac->cset0_bwp_size;

return bwp_info;

}

NR_UE_DL_BWP_t *dl_bwp = &UE->current_DL_BWP;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

// 3GPP TS 38.214 Section 5.1.2.2 Resource allocation in frequency domain

// For a PDSCH scheduled with a DCI format 1_0 in any type of PDCCH common search space, regardless of which bandwidth part is the

// active bandwidth part, RB numbering starts from the lowest RB of the CORESET in which the DCI was received; otherwise RB

// numbering starts from the lowest RB in the determined downlink bandwidth part.

//

// 3GPP TS 38.214 Section 5.1.2.2.2 Downlink resource allocation type 1

// In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of

// contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size   PRBs

// except for the case when DCI format 1_0 is decoded in any common search space in which case the size of CORESET 0 shall be

// used if CORESET 0 is configured for the cell and the size of initial DL bandwidth part shall be used if CORESET 0 is not

// configured for the cell.

if (dl_bwp->dci_format == NR_DL_DCI_FORMAT_1_0

&& sched_ctrl->search_space->searchSpaceType

&& sched_ctrl->search_space->searchSpaceType->present == NR_SearchSpace__searchSpaceType_PR_common) {

if (sched_ctrl->coreset->controlResourceSetId == 0) {

bwp_info.bwpStart = nr_mac->cset0_bwp_start;

} else {

int additional_offset = (dl_bwp->BWPStart + 5) / 6 * 6 - dl_bwp->BWPStart;

bwp_info.bwpStart = dl_bwp->BWPStart + sched_ctrl->sched_pdcch.rb_start + additional_offset;

}

if (nr_mac->cset0_bwp_size > 0) {

bwp_info.bwpSize = min(dl_bwp->BWPSize, nr_mac->cset0_bwp_size);

} else {

bwp_info.bwpSize = min(dl_bwp->BWPSize, UE->sc_info.initial_dl_BWPSize);

}

} else {

bwp_info.bwpSize = dl_bwp->BWPSize;

bwp_info.bwpStart = dl_bwp->BWPStart;

}

return bwp_info;

}

/* Returns 0 for UE1 (25 PRBs), 1 for UE2 (55 PRBs), -1 for others.

* Uses CU-UE-ID when available, else connected_ue_list index. */

static int get_fixed_dl_prb_ue_index(NR_UE_info_t *UE, NR_UE_info_t **UE_list)

{

if (du_exists_f1_ue_data(UE->rnti)) {

int cu_id = du_get_f1_ue_data(UE->rnti).secondary_ue;

if (cu_id == 1) return 0;

if (cu_id == 2) return 1;

return -1;

}

for (int i = 0; i < MAX_MOBILES_PER_GNB + 1 && UE_list[i] != NULL; i++) {

if (UE_list[i] == UE)

return (i < 2) ? i : -1;

}

return -1;

}

/* Returns a stable UE "ID" for filtering purposes:

* - prefer CU-UE-ID (secondary_ue) when available (F1),

* - else fall back to connected_ue_list index. */

static int get_ueid_for_dl_filter(const NR_UE_info_t *UE, NR_UE_info_t **UE_list)

{

if (du_exists_f1_ue_data(UE->rnti))

return du_get_f1_ue_data(UE->rnti).secondary_ue;

for (int i = 0; i < MAX_MOBILES_PER_GNB + 1 && UE_list[i] != NULL; i++) {

if (UE_list[i] == UE)

return i;

}

return -1;

}

static bool allocate_dl_retransmission(gNB_MAC_INST *nr_mac,

post_process_pdsch_t *pp_pdsch,

int *n_rb_sched,

NR_UE_info_t *UE,

NR_UE_info_t **UE_list,

int beam_idx,

int current_harq_pid)

{

frame_t frame = pp_pdsch->frame;

slot_t slot = pp_pdsch->slot;

int CC_id = 0;

const NR_ServingCellConfigCommon_t *scc = nr_mac->common_channels->ServingCellConfigCommon;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_DL_BWP_t *dl_bwp = &UE->current_DL_BWP;

NR_UE_UL_BWP_t *ul_bwp = &UE->current_UL_BWP;

NR_sched_pdsch_t new_sched = sched_ctrl->harq_processes[current_harq_pid].sched_pdsch;

int layers = get_dl_nrOfLayers(sched_ctrl, dl_bwp->dci_format);

int pm_index = get_pm_index(nr_mac, UE, dl_bwp->dci_format, layers, nr_mac->radio_config.pdsch_AntennaPorts.XP);

// If the RI changed between current rtx and a previous transmission

// we need to verify if it is not decreased

// othwise it wouldn't be possible to transmit the same TBS

layers = (layers < new_sched.nrOfLayers) ? layers : new_sched.nrOfLayers;

pm_index = (layers < new_sched.nrOfLayers) ? pm_index : new_sched.pm_index;

const int coresetid = sched_ctrl->coreset->controlResourceSetId;

const int tda = get_dl_tda(nr_mac, slot);

AssertFatal(tda >= 0,"Unable to find PDSCH time domain allocation in list\n");

/* Check first whether the old TDA can be reused

* this helps allocate retransmission when TDA changes (e.g. new nrOfSymbols > old nrOfSymbols) */

NR_tda_info_t temp_tda = get_dl_tda_info(dl_bwp,

sched_ctrl->search_space->searchSpaceType->present,

tda,

scc->dmrs_TypeA_Position,

1,

TYPE_C_RNTI_,

coresetid,

false);

if (!temp_tda.valid_tda)

return false;

bool reuse_old_tda = (new_sched.tda_info.startSymbolIndex == temp_tda.startSymbolIndex) && (new_sched.tda_info.nrOfSymbols <= temp_tda.nrOfSymbols);

LOG_D(NR_MAC, "[UE %x] %s old TDA, %s number of layers\n",

UE->rnti,

reuse_old_tda ? "reuse" : "do not reuse",

layers == new_sched.nrOfLayers ? "same" : "different");

uint16_t *rballoc_mask = nr_mac->common_channels[CC_id].vrb_map[beam_idx];

bwp_info_t bwp_info = get_pdsch_bwp_start_size(nr_mac, UE);

int rbStart = bwp_info.bwpStart;

int rbStop = bwp_info.bwpStart + bwp_info.bwpSize - 1;

int rbSize = 0;

if (reuse_old_tda && layers == new_sched.nrOfLayers) {

/* Check that there are enough resources for retransmission */

while (rbSize < new_sched.rbSize) {

rbStart += rbSize; /* last iteration rbSize was not enough, skip it */

rbSize = 0;

const uint16_t slbitmap = SL_to_bitmap(new_sched.tda_info.startSymbolIndex, new_sched.tda_info.nrOfSymbols);

while (rbStart < rbStop && (rballoc_mask[rbStart] & slbitmap))

rbStart++;

if (rbStart >= rbStop) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not allocate DL retransmission: no resources\n", UE->rnti, frame, slot);

return false;

}

while (rbStart + rbSize <= rbStop && !(rballoc_mask[rbStart + rbSize] & slbitmap) && rbSize < new_sched.rbSize)

rbSize++;

DevAssert(rbSize > 0);

}

} else {

/* the retransmission will use a different time domain allocation, check

* that we have enough resources */

NR_pdsch_dmrs_t temp_dmrs = get_dl_dmrs_params(scc, dl_bwp, &temp_tda, layers);

const uint16_t slbitmap = SL_to_bitmap(temp_tda.startSymbolIndex, temp_tda.nrOfSymbols);

while (rbStart < rbStop && (rballoc_mask[rbStart] & slbitmap))

rbStart++;

if (rbStart >= rbStop) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not allocate DL retransmission: no resources\n", UE->rnti, frame, slot);

return false;

}

while (rbStart + rbSize <= rbStop && !(rballoc_mask[rbStart + rbSize] & slbitmap))

rbSize++;

DevAssert(rbSize > 0);

uint32_t new_tbs;

uint16_t new_rbSize;

bool success = nr_find_nb_rb(new_sched.Qm,

new_sched.R,

1, // no transform precoding for DL

layers,

temp_tda.nrOfSymbols,

temp_dmrs.N_PRB_DMRS * temp_dmrs.N_DMRS_SLOT,

new_sched.tb_size,

1, /* minimum of 1RB: need to find exact TBS, don't preclude any number */

rbSize,

&new_tbs,

&new_rbSize);

if (!success || new_tbs != new_sched.tb_size) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] allocation of DL retransmission failed: new TBS %d of new TDA does not match old TBS %d\n",

UE->rnti,

frame,

slot,

new_tbs,

new_sched.tb_size);

return false; /* the maximum TBsize we might have is smaller than what we need */

}

/* we can allocate it. Overwrite the time_domain_allocation, the number

* of RBs, and the new TB size. The rest is done below */

new_sched.tb_size = new_tbs;

new_sched.rbSize = new_rbSize;

new_sched.time_domain_allocation = tda;

new_sched.nrOfLayers = layers;

new_sched.pm_index = pm_index;

new_sched.dmrs_parms = temp_dmrs;

new_sched.tda_info = temp_tda;

}

/* Find a free CCE */

int CCEIndex = get_cce_index(nr_mac,

CC_id,

slot,

UE->rnti,

&sched_ctrl->aggregation_level,

beam_idx,

sched_ctrl->search_space,

sched_ctrl->coreset,

&sched_ctrl->sched_pdcch,

sched_ctrl->pdcch_cl_adjust);

if (CCEIndex<0) {

sched_ctrl->dl_cce_fail++;

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not find free CCE for DL DCI retransmission\n", UE->rnti, frame, slot);

return false;

}

/* Find PUCCH occasion: if it fails, undo CCE allocation (undoing PUCCH

* allocation after CCE alloc fail would be more complex) */

int alloc = -1;

if (!get_FeedbackDisabled(UE->sc_info.downlinkHARQ_FeedbackDisabled_r17, current_harq_pid)) {

int r_pucch = nr_get_pucch_resource(sched_ctrl->coreset, ul_bwp->pucch_Config, CCEIndex);

alloc = nr_acknack_scheduling(nr_mac, UE, frame, slot, UE->UE_beam_index, r_pucch, 0);

if (alloc < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not find PUCCH for DL DCI retransmission\n", UE->rnti, frame, slot);

return false;

}

}

sched_ctrl->cce_index = CCEIndex;

fill_pdcch_vrb_map(nr_mac, CC_id, &sched_ctrl->sched_pdcch, CCEIndex, sched_ctrl->aggregation_level, beam_idx);

new_sched.rbStart = rbStart - bwp_info.bwpStart;

new_sched.pucch_allocation = alloc;

new_sched.bwp_info = bwp_info;

post_process_dlsch(nr_mac, pp_pdsch, UE, &new_sched);

/* retransmissions: directly allocate */

*n_rb_sched -= new_sched.rbSize;

for (int rb = rbStart; rb < rbStart + new_sched.rbSize; rb++)

rballoc_mask[rb] |= SL_to_bitmap(new_sched.tda_info.startSymbolIndex, new_sched.tda_info.nrOfSymbols);

return true;

}

static void ack_reconfig(gNB_MAC_INST *mac, NR_UE_info_t *UE)

{

if (!UE->reconfigCellGroup) {

LOG_W(NR_MAC, "Received ACK for RRCReconfiguration, but nothing to apply!\n");

return;

}

ASN_STRUCT_FREE(asn_DEF_NR_CellGroupConfig, UE->CellGroup);

UE->CellGroup = UE->reconfigCellGroup;

UE->reconfigCellGroup = NULL;

NR_ServingCellConfigCommon_t *scc = mac->common_channels[0].ServingCellConfigCommon;

/* clean BWP structures */

clean_bwp_structures(UE->CellGroup->spCellConfig);

configure_UE_BWP(mac, scc, UE, false, NR_SearchSpace__searchSpaceType_PR_common, -1, -1);

}

static bool dlsch_to_schedule(const NR_UE_sched_ctrl_t *sched_ctrl, int frame)

{

/* Check DL buffer, TA to be sent and  beam switch needed*/

if (sched_ctrl->num_total_bytes > 0)

return true;

if (sched_ctrl->ta_apply == true)

return true;

if (sched_ctrl->UE_mac_ce_ctrl.tci_state_ind.is_scheduled)

return true;

// if none of the condition for dlsch to be scheduled are met

return false;

}

typedef struct UEsched_s {

float coef;

NR_UE_info_t * UE;

int selected_mcs;

} UEsched_t;

static int comparator(const void *p, const void *q)

{

const UEsched_t *pp = p;

const UEsched_t *qq = q;

if (pp->coef < qq->coef)

return 1;

else if (pp->coef > qq->coef)

return -1;

return 0;

}

static void pf_dl(gNB_MAC_INST *mac,

post_process_pdsch_t *pp_pdsch,

NR_UE_info_t **UE_list,

int max_num_ue,

int num_beams,

int n_rb_sched[num_beams])

{

frame_t frame = pp_pdsch->frame;

slot_t slot = pp_pdsch->slot;

/* throttle PRB sharing logs to avoid flooding:

* - print periodically every 128 frames at slot 0

* - also print immediately on sharing state change */

static int last_periodic_log_frame = -1;

static int last_share_active = 0;

/* hysteresis to avoid rapid ACTIVE/INACTIVE flapping */

const uint32_t share_on_thr = 20000;

const uint32_t share_off_thr = (20000 * 9) / 10;

/* 0: inactive, 1: UE1 high (UE2 shares), 2: UE2 high (UE1 shares) */

static int share_mode = 0;

static int last_buf_update_frame = -1;

static int last_buf_update_slot = -1;

NR_ServingCellConfigCommon_t *scc=mac->common_channels[0].ServingCellConfigCommon;

// UEs that could be scheduled

UEsched_t UE_sched[MAX_MOBILES_PER_GNB + 1] = {0};

int remainUEs[num_beams];

for (int i = 0; i < num_beams; i++)

remainUEs[i] = max_num_ue;

int numUE = 0;

int CC_id = 0;

int slots_per_frame = mac->frame_structure.numb_slots_frame;

/* Loop UE_info->list to check retransmission */

UE_iterator(UE_list, UE) {

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_DL_BWP_t *current_BWP = &UE->current_DL_BWP;

if (!nr_mac_ue_is_active(UE))

continue;

/* Do not schedule/serve UEs with UEID >= 3 */

const int ueid = get_ueid_for_dl_filter(UE, UE_list);

if (ueid >= 3)

continue;

NR_mac_dir_stats_t *stats = &UE->mac_stats.dl;

/* get the PID of a HARQ process awaiting retrnasmission, or -1 otherwise */

int harq_pid = sched_ctrl->retrans_dl_harq.head;

/* Calculate Throughput */

const float a = 0.01f;

const uint32_t b = stats->current_bytes;

UE->dl_thr_ue = (1 - a) * UE->dl_thr_ue + a * b;

stats->current_bytes = 0;

stats->current_rbs = 0;

/* Check if this UE should get TA. If we add the CE, ta_apply will be reset */

if (frame == sched_ctrl->ta_frame)

sched_ctrl->ta_apply = true;

int total_rem_ues = 0;

for (int i = 0; i < num_beams; i++)

total_rem_ues += remainUEs[i];

if (total_rem_ues == 0)

continue;

/* retransmission */

if (harq_pid >= 0) {

NR_beam_alloc_t beam = beam_allocation_procedure(&mac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame);

bool sch_ret = beam.idx >= 0;

/* Allocate retransmission */

if (sch_ret)

sch_ret = allocate_dl_retransmission(mac, pp_pdsch, &n_rb_sched[beam.idx], UE, UE_list, beam.idx, harq_pid);

if (!sch_ret) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] DL retransmission could not be allocated\n", UE->rnti, frame, slot);

reset_beam_status(&mac->beam_info, frame, slot, UE->UE_beam_index, slots_per_frame, beam.new_beam);

continue;

}

/* reduce max_num_ue once we are sure UE can be allocated, i.e., has CCE */

remainUEs[beam.idx]--;

} else {

/* skip this UE if there are no free HARQ processes. This can happen e.g.

* if the UE disconnected in L2sim, in which case the gNB is not notified

* (this can be considered a design flaw) */

if (sched_ctrl->available_dl_harq.head < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] UE has no free DL HARQ process, skipping\n",

UE->rnti,

frame,

slot);

continue;

}

update_dlsch_buffer(pp_pdsch->frame, pp_pdsch->slot, UE);

const uint32_t dl_buf_bytes = update_dlsch_buffer(pp_pdsch->frame, pp_pdsch->slot, UE);

if (pp_pdsch->slot == 0 && pp_pdsch->frame % 128 == 0) {

LOG_I(NR_MAC,

"%4d.%2d UE %04x DL buffer: %u bytes (%u PDUs)\n",

pp_pdsch->frame,

pp_pdsch->slot,

UE->rnti,

dl_buf_bytes,

UE->UE_sched_ctrl.dl_pdus_total);

}

if (!dlsch_to_schedule(sched_ctrl, frame))

continue;

/* Calculate coeff */

const NR_bler_options_t *bo = &mac->dl_bler;

const int max_mcs_table = current_BWP->mcsTableIdx == 1 ? 27 : 28;

const int max_mcs = min(sched_ctrl->dl_max_mcs, max_mcs_table);

int selected_mcs;

if (bo->harq_round_max == 1) {

int new_mcs = min(bo->max_mcs, max_mcs);

selected_mcs = max(bo->min_mcs, new_mcs);

sched_ctrl->dl_bler_stats.mcs = selected_mcs;

} else

selected_mcs = get_mcs_from_bler(bo, stats, &sched_ctrl->dl_bler_stats, max_mcs, frame);

int l = get_dl_nrOfLayers(sched_ctrl, current_BWP->dci_format);

const uint8_t Qm = nr_get_Qm_dl(selected_mcs, current_BWP->mcsTableIdx);

const uint16_t R = nr_get_code_rate_dl(selected_mcs, current_BWP->mcsTableIdx);

uint32_t tbs = nr_compute_tbs(Qm,

R,

1, /* rbSize */

10, /* hypothetical number of slots */

0, /* N_PRB_DMRS * N_DMRS_SLOT */

0 /* N_PRB_oh, 0 for initialBWP */,

0 /* tb_scaling */,

l) >> 3;

float coeff_ue = (float) tbs / UE->dl_thr_ue;

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] b %d, thr_ue %f, tbs %d, coeff_ue %f\n",

UE->rnti,

frame,

slot,

b,

UE->dl_thr_ue,

tbs,

coeff_ue);

/* Create UE_sched list for UEs eligible for new transmission*/

UE_sched[numUE].coef = coeff_ue;

UE_sched[numUE].UE = UE;

UE_sched[numUE].selected_mcs = selected_mcs;

numUE++;

}

}

qsort(UE_sched, numUE, sizeof(UEsched_t), comparator);

UEsched_t *iterator = UE_sched;

const int min_rbSize = 5;

/* Loop UE_sched to find max coeff and allocate transmission */

while (iterator->UE != NULL) {

NR_UE_sched_ctrl_t *sched_ctrl = &iterator->UE->UE_sched_ctrl;

const uint16_t rnti = iterator->UE->rnti;

NR_UE_DL_BWP_t *dl_bwp = &iterator->UE->current_DL_BWP;

NR_UE_UL_BWP_t *ul_bwp = &iterator->UE->current_UL_BWP;

if (sched_ctrl->available_dl_harq.head < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] UE has no free DL HARQ process, skipping\n",

iterator->UE->rnti,

frame,

slot);

iterator++;

continue;

}

NR_beam_alloc_t beam = beam_allocation_procedure(&mac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame);

if (beam.idx < 0) {

// no available beam

iterator++;

continue;

}

if (remainUEs[beam.idx] == 0 || n_rb_sched[beam.idx] < min_rbSize) {

reset_beam_status(&mac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

/* MCS has been set above */

int tda = get_dl_tda(mac, slot);

AssertFatal(tda >= 0,"Unable to find PDSCH time domain allocation in list\n");

const int coresetid = sched_ctrl->coreset->controlResourceSetId;

NR_tda_info_t tda_info = get_dl_tda_info(dl_bwp,

sched_ctrl->search_space->searchSpaceType->present,

tda,

scc->dmrs_TypeA_Position,

1,

TYPE_C_RNTI_,

coresetid,

false);

AssertFatal(tda_info.valid_tda, "Invalid TDA from get_dl_tda_info\n");

const uint16_t slbitmap = SL_to_bitmap(tda_info.startSymbolIndex, tda_info.nrOfSymbols);

uint16_t *rballoc_mask = mac->common_channels[CC_id].vrb_map[beam.idx];

bwp_info_t bwp_info = get_pdsch_bwp_start_size(mac, iterator->UE);

int rbStart = 0; // WRT BWP start

int rbStop = bwp_info.bwpSize - 1;

int bwp_start = bwp_info.bwpStart;

uint16_t max_rbSize = 1;

/* Custom fixed-PRB allocation with optional sharing based on DL buffer.

* Baseline:

*  - UE (CU-UE-ID 1): 25 PRBs, PRB 0..24

*  - UE (CU-UE-ID 2): 55 PRBs, PRB 25..79

* If exactly one UE's DL buffer exceeds dl_buf_share_threshold_bytes and the

* other UE is below threshold, the low-buffer UE gives half of its baseline

* PRBs to the high-buffer UE (still contiguous, still within PRB 0..79). */

int num_conn = 0;

for (int i = 0; i < MAX_MOBILES_PER_GNB + 1 && UE_list[i] != NULL; i++) num_conn++;

/* Only enable fixed mode when exactly two UEs are connected and they match

* the expected mapping (CU-UE-ID 1 and 2 when available). If a 3rd UE

* connects, fall back to dynamic scheduling to avoid interfering with it. */

bool fixed_two_ue_mode = false;

if (num_conn == 2 && bwp_info.bwpSize >= 80) {

int have_ue1 = 0, have_ue2 = 0;

for (int i = 0; i < num_conn; i++) {

int idx = get_fixed_dl_prb_ue_index(UE_list[i], UE_list);

have_ue1 |= (idx == 0);

have_ue2 |= (idx == 1);

}

fixed_two_ue_mode = have_ue1 && have_ue2;

}

int fixed_ue_idx = get_fixed_dl_prb_ue_index(iterator->UE, UE_list);

if (fixed_two_ue_mode && (fixed_ue_idx == 0 || fixed_ue_idx == 1)) {

/* compute current buffers for UE1/UE2 (if present) */

uint32_t buf_ue1 = 0, buf_ue2 = 0;

/* refresh both UE buffers once per slot so decision uses up-to-date data */

if (last_buf_update_frame != frame || last_buf_update_slot != slot) {

for (int i = 0; i < MAX_MOBILES_PER_GNB + 1 && UE_list[i] != NULL; i++) {

NR_UE_info_t *u = UE_list[i];

int idx = get_fixed_dl_prb_ue_index(u, UE_list);

if (idx == 0 || idx == 1)

update_dlsch_buffer(frame, slot, u);

}

last_buf_update_frame = frame;

last_buf_update_slot = slot;

}

for (int i = 0; i < MAX_MOBILES_PER_GNB + 1 && UE_list[i] != NULL; i++) {

NR_UE_info_t *u = UE_list[i];

int idx = get_fixed_dl_prb_ue_index(u, UE_list);

if (idx == 0)

buf_ue1 = u->UE_sched_ctrl.num_total_bytes;

else if (idx == 1)

buf_ue2 = u->UE_sched_ctrl.num_total_bytes;

}

int size_ue1 = 25;

int size_ue2 = 55;

/* update sharing mode:

* - enter share mode if any UE exceeds share_on_thr

* - once in share mode, do not return to 0; allow switching between 1/2

*   when the other UE becomes the larger-buffer UE */

if (buf_ue1 > share_on_thr || buf_ue2 > share_on_thr) {

if (buf_ue1 >= buf_ue2)

share_mode = 1;

else

share_mode = 2;

}

if (share_mode == 1) {

/* UE2 (low) shares half of its baseline PRBs to UE1 (high) */

int ue2_keep = 55 / 2; // floor(55/2)=27

size_ue2 = ue2_keep;

size_ue1 = 80 - ue2_keep; // 53

} else if (share_mode == 2) {

/* UE1 (low) shares half of its baseline PRBs to UE2 (high) */

int ue1_keep = 25 / 2; // floor(25/2)=12

size_ue1 = ue1_keep;

size_ue2 = 80 - ue1_keep; // 68

}

/* Log sharing decision with throttling */

const int share_active = (size_ue1 != 25 || size_ue2 != 55);

const int periodic = (slot == 0) && (frame % 128 == 0) && (last_periodic_log_frame != frame);

const int state_change = (share_active != last_share_active);

if ((periodic || state_change) && (fixed_ue_idx == 0)) {

if (periodic)

last_periodic_log_frame = frame;

last_share_active = share_active;

LOG_I(NR_MAC,

"[DL PRB SHARE] %4d.%2d thr_on=%uB thr_off=%uB buf(UE1=%uB UE2=%uB) split(UE1=%d UE2=%d) mode=%d %s\n",

frame,

slot,

share_on_thr,

share_off_thr,

buf_ue1,

buf_ue2,

size_ue1,

size_ue2,

share_mode,

share_active ? "ACTIVE" : "INACTIVE");

}

if (fixed_ue_idx == 0) {

rbStart = 0;

max_rbSize = size_ue1;

} else { // fixed_ue_idx == 1

rbStart = size_ue1;

max_rbSize = size_ue2;

}

rbStop = rbStart + max_rbSize - 1;

} else {

// Default dynamic frequency-domain allocation

while (rbStart < rbStop && (rballoc_mask[rbStart + bwp_start] & slbitmap))

rbStart++;

while (rbStart + max_rbSize <= rbStop && !(rballoc_mask[rbStart + max_rbSize + bwp_start] & slbitmap))

max_rbSize++;

}

if (max_rbSize < min_rbSize) {

LOG_D(NR_MAC,

"(%d.%d) Cannot schedule RNTI %04x, rbStart %d, rbSize %d, rbStop %d\n",

frame,

slot,

rnti,

rbStart,

max_rbSize,

rbStop);

reset_beam_status(&mac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

int CCEIndex = get_cce_index(mac,

CC_id,

slot,

iterator->UE->rnti,

&sched_ctrl->aggregation_level,

beam.idx,

sched_ctrl->search_space,

sched_ctrl->coreset,

&sched_ctrl->sched_pdcch,

sched_ctrl->pdcch_cl_adjust);

if (CCEIndex < 0) {

sched_ctrl->dl_cce_fail++;

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not find free CCE for DL DCI\n", rnti, frame, slot);

reset_beam_status(&mac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

/* Find PUCCH occasion: if it fails, undo CCE allocation (undoing PUCCH

* allocation after CCE alloc fail would be more complex) */

int alloc = -1;

if (!get_FeedbackDisabled(iterator->UE->sc_info.downlinkHARQ_FeedbackDisabled_r17, sched_ctrl->available_dl_harq.head)) {

int r_pucch = nr_get_pucch_resource(sched_ctrl->coreset, ul_bwp->pucch_Config, CCEIndex);

alloc = nr_acknack_scheduling(mac, iterator->UE, frame, slot, iterator->UE->UE_beam_index, r_pucch, 0);

if (alloc < 0) {

LOG_D(NR_MAC, "[UE %04x][%4d.%2d] could not find PUCCH for DL DCI\n", rnti, frame, slot);

reset_beam_status(&mac->beam_info, frame, slot, iterator->UE->UE_beam_index, slots_per_frame, beam.new_beam);

iterator++;

continue;

}

}

sched_ctrl->cce_index = CCEIndex;

fill_pdcch_vrb_map(mac, CC_id, &sched_ctrl->sched_pdcch, CCEIndex, sched_ctrl->aggregation_level, beam.idx);

int l = get_dl_nrOfLayers(sched_ctrl, dl_bwp->dci_format);

NR_sched_pdsch_t sched_pdsch = {

// rbSize below

.rbStart = rbStart,

.mcs = iterator->selected_mcs,

.R = nr_get_code_rate_dl(iterator->selected_mcs, dl_bwp->mcsTableIdx),

.Qm = nr_get_Qm_dl(iterator->selected_mcs, dl_bwp->mcsTableIdx),

// tb_size below

.dl_harq_pid = sched_ctrl->available_dl_harq.head,

.pucch_allocation = alloc,

.pm_index = get_pm_index(mac, iterator->UE, dl_bwp->dci_format, l, mac->radio_config.pdsch_AntennaPorts.XP),

.nrOfLayers = l,

.bwp_info = bwp_info,

.dmrs_parms = get_dl_dmrs_params(scc, dl_bwp, &tda_info, l),

.time_domain_allocation = tda,

.tda_info = tda_info,

};

sched_pdsch.action = NULL;

int srb1 = 1;

/* everything that's only 3 bytes is an ack. To be safe, use a bit more. */

if (iterator->UE->reconfigCellGroup && sched_ctrl->rlc_status[srb1].bytes_in_buffer > 10)

sched_pdsch.action = ack_reconfig;

// Fix me: currently, the RLC does not give us the total number of PDUs

// awaiting. Therefore, for the time being, we put a fixed overhead of 12

// (for 4 PDUs) and optionally + 2 for TA. Once RLC gives the number of

// PDUs, we replace with 3 * numPDUs

const int oh = 3 * 4 + (sched_ctrl->ta_apply ? 2 : 0);

//const int oh = 3 * sched_ctrl->dl_pdus_total + (sched_ctrl->ta_apply ? 2 : 0);

if (fixed_two_ue_mode && (fixed_ue_idx == 0 || fixed_ue_idx == 1)) {

/* Fixed 25/55 PRB allocation: bypass nr_find_nb_rb and set directly */

sched_pdsch.rbSize = max_rbSize;

sched_pdsch.tb_size = nr_compute_tbs(sched_pdsch.Qm,

sched_pdsch.R,

sched_pdsch.rbSize,

tda_info.nrOfSymbols,

sched_pdsch.dmrs_parms.N_PRB_DMRS * sched_pdsch.dmrs_parms.N_DMRS_SLOT,

0, 0,

sched_pdsch.nrOfLayers) >> 3;

} else {

nr_find_nb_rb(sched_pdsch.Qm,

sched_pdsch.R,

1, // no transform precoding for DL

sched_pdsch.nrOfLayers,

tda_info.nrOfSymbols,

sched_pdsch.dmrs_parms.N_PRB_DMRS * sched_pdsch.dmrs_parms.N_DMRS_SLOT,

sched_ctrl->num_total_bytes + oh,

min_rbSize,

max_rbSize,

&sched_pdsch.tb_size,

&sched_pdsch.rbSize);

}

post_process_dlsch(mac, pp_pdsch, iterator->UE, &sched_pdsch);

/* transmissions: directly allocate */

n_rb_sched[beam.idx] -= sched_pdsch.rbSize;

for (int rb = bwp_start; rb < sched_pdsch.rbSize; rb++)

rballoc_mask[rb + sched_pdsch.rbStart] |= slbitmap;

remainUEs[beam.idx]--;

iterator++;

}

}

static void nr_dlsch_preprocessor(gNB_MAC_INST *mac, post_process_pdsch_t *pp_pdsch)

{

NR_UEs_t *UE_info = &mac->UE_info;

if (UE_info->connected_ue_list[0] == NULL)

return;

NR_ServingCellConfigCommon_t *scc = mac->common_channels[0].ServingCellConfigCommon;

int bw = scc->downlinkConfigCommon->frequencyInfoDL->scs_SpecificCarrierList.list.array[0]->carrierBandwidth;

int num_beams = mac->beam_info.beam_allocation ? mac->beam_info.beams_per_period : 1;

int n_rb_sched[num_beams];

for (int i = 0; i < num_beams; i++)

n_rb_sched[i] = bw;

int average_agg_level = 4; // TODO find a better estimation

int max_sched_ues = bw / (average_agg_level * NR_NB_REG_PER_CCE);

// FAPI cannot handle more than MAX_DCI_CORESET DCIs

max_sched_ues = min(max_sched_ues, MAX_DCI_CORESET);

/* proportional fair scheduling algorithm */

pf_dl(mac, pp_pdsch, UE_info->connected_ue_list, max_sched_ues, num_beams, n_rb_sched);

}

nr_pp_impl_dl nr_init_dlsch_preprocessor(int CC_id)

{

return nr_dlsch_preprocessor;

}

nfapi_nr_dl_tti_pdsch_pdu_rel15_t *prepare_pdsch_pdu(nfapi_nr_dl_tti_request_pdu_t *dl_tti_pdsch_pdu,

const gNB_MAC_INST *mac,

const NR_UE_info_t *UE,

const NR_sched_pdsch_t *sched_pdsch,

const NR_PDSCH_Config_t *pdsch_Config,

bool is_sib1,

int harq_round,

int rnti,

int beam_index,

int nl_tbslbrm,

int pdu_index)

{

const NR_UE_DL_BWP_t *dl_bwp = UE ? &UE->current_DL_BWP : NULL;

const NR_ServingCellConfigCommon_t *scc = mac->common_channels[0].ServingCellConfigCommon;

nfapi_nr_dl_tti_pdsch_pdu_rel15_t *pdsch_pdu = &dl_tti_pdsch_pdu->pdsch_pdu.pdsch_pdu_rel15;

pdsch_pdu->pduBitmap = 0;

pdsch_pdu->rnti = rnti;

pdsch_pdu->pduIndex = pdu_index;

pdsch_pdu->BWPSize  = sched_pdsch->bwp_info.bwpSize;

pdsch_pdu->BWPStart = sched_pdsch->bwp_info.bwpStart;

pdsch_pdu->SubcarrierSpacing = dl_bwp ? dl_bwp->scs : *scc->ssbSubcarrierSpacing;

pdsch_pdu->CyclicPrefix = dl_bwp && dl_bwp->cyclicprefix ? *dl_bwp->cyclicprefix : 0;

// Codeword information

pdsch_pdu->NrOfCodewords = 1;

pdsch_pdu->targetCodeRate[0] = sched_pdsch->R;

pdsch_pdu->qamModOrder[0] = sched_pdsch->Qm;

pdsch_pdu->mcsIndex[0] = sched_pdsch->mcs;

pdsch_pdu->mcsTable[0] = dl_bwp ? dl_bwp->mcsTableIdx : 0;

pdsch_pdu->rvIndex[0] = nr_get_rv(harq_round % 4);

pdsch_pdu->TBSize[0] = sched_pdsch->tb_size;

pdsch_pdu->dataScramblingId = pdsch_Config && pdsch_Config->dataScramblingIdentityPDSCH ? *pdsch_Config->dataScramblingIdentityPDSCH : *scc->physCellId;

pdsch_pdu->nrOfLayers = sched_pdsch->nrOfLayers;

pdsch_pdu->transmissionScheme = 0;

pdsch_pdu->refPoint = is_sib1;

// DMRS

const NR_pdsch_dmrs_t *dmrs_parms = &sched_pdsch->dmrs_parms;

pdsch_pdu->dlDmrsSymbPos = dmrs_parms->dl_dmrs_symb_pos;

pdsch_pdu->dmrsConfigType = dmrs_parms->dmrsConfigType;

pdsch_pdu->SCID = dmrs_parms->n_scid;

pdsch_pdu->dlDmrsScramblingId = dmrs_parms->scrambling_id;

pdsch_pdu->numDmrsCdmGrpsNoData = dmrs_parms->numDmrsCdmGrpsNoData;

pdsch_pdu->dmrsPorts = (1 << sched_pdsch->nrOfLayers) - 1;  // FIXME with a better implementation

// Pdsch Allocation in frequency domain

pdsch_pdu->resourceAlloc = 1;

pdsch_pdu->rbStart = sched_pdsch->rbStart;

pdsch_pdu->rbSize = sched_pdsch->rbSize;

pdsch_pdu->VRBtoPRBMapping = 0; // non-interleaved

// Resource Allocation in time domain

const NR_tda_info_t *tda_info = &sched_pdsch->tda_info;

pdsch_pdu->StartSymbolIndex = tda_info->startSymbolIndex;

pdsch_pdu->NrOfSymbols = tda_info->nrOfSymbols;

/* Check and validate PTRS values */

if (dmrs_parms->phaseTrackingRS) {

bool valid_ptrs_setup = set_dl_ptrs_values(dmrs_parms->phaseTrackingRS,

pdsch_pdu->rbSize,

pdsch_pdu->mcsIndex[0],

pdsch_pdu->mcsTable[0],

&pdsch_pdu->PTRSFreqDensity,

&pdsch_pdu->PTRSTimeDensity,

&pdsch_pdu->PTRSPortIndex,

&pdsch_pdu->nEpreRatioOfPDSCHToPTRS,

&pdsch_pdu->PTRSReOffset,

pdsch_pdu->NrOfSymbols);

if (valid_ptrs_setup)

pdsch_pdu->pduBitmap |= 0x1; // Bit 0: pdschPtrs - Indicates PTRS included (FR2)

}

int dl_bw_tbslbrm = UE ? UE->sc_info.dl_bw_tbslbrm : sched_pdsch->bwp_info.bwpSize;

pdsch_pdu->maintenance_parms_v3.tbSizeLbrmBytes = nr_compute_tbslbrm(pdsch_pdu->mcsTable[0], dl_bw_tbslbrm, nl_tbslbrm);

pdsch_pdu->maintenance_parms_v3.ldpcBaseGraph = get_BG(sched_pdsch->tb_size << 3, sched_pdsch->R);

// Precoding and beamforming

pdsch_pdu->precodingAndBeamforming.num_prgs = 1;

pdsch_pdu->precodingAndBeamforming.prg_size = pdsch_pdu->rbSize;

pdsch_pdu->precodingAndBeamforming.dig_bf_interfaces = 1;

pdsch_pdu->precodingAndBeamforming.prgs_list[0].pm_idx = sched_pdsch->pm_index;

pdsch_pdu->precodingAndBeamforming.prgs_list[0].dig_bf_interface_list[0].beam_idx = beam_index;

return pdsch_pdu;

}

void post_process_dlsch(gNB_MAC_INST *nr_mac, post_process_pdsch_t *pdsch, NR_UE_info_t *UE, NR_sched_pdsch_t *sched_pdsch)

{

int CC_id = 0;

int module_id = nr_mac->Mod_id;

frame_t frame = pdsch->frame;

slot_t slot = pdsch->slot;

const NR_ServingCellConfigCommon_t *scc = nr_mac->common_channels[CC_id].ServingCellConfigCommon;

NR_UE_sched_ctrl_t *sched_ctrl = &UE->UE_sched_ctrl;

NR_UE_DL_BWP_t *current_BWP = &UE->current_DL_BWP;

const rnti_t rnti = UE->rnti;

/* POST processing */

const uint8_t nrOfLayers = sched_pdsch->nrOfLayers;

const uint32_t TBS = sched_pdsch->tb_size;

int8_t current_harq_pid = sched_pdsch->dl_harq_pid;

if (current_harq_pid < 0) {

/* PP has not selected a specific HARQ Process, get a new one */

current_harq_pid = sched_ctrl->available_dl_harq.head;

AssertFatal(current_harq_pid >= 0,

"no free HARQ process available for UE %04x\n",

UE->rnti);

remove_front_nr_list(&sched_ctrl->available_dl_harq);

sched_pdsch->dl_harq_pid = current_harq_pid;

} else {

/* PP selected a specific HARQ process. Check whether it will be a new

* transmission or a retransmission, and remove from the corresponding

* list */

if (sched_ctrl->harq_processes[current_harq_pid].round == 0)

remove_nr_list(&sched_ctrl->available_dl_harq, current_harq_pid);

else

remove_nr_list(&sched_ctrl->retrans_dl_harq, current_harq_pid);

}

NR_tda_info_t *tda_info = &sched_pdsch->tda_info;

NR_pdsch_dmrs_t *dmrs_parms = &sched_pdsch->dmrs_parms;

NR_UE_harq_t *harq = &sched_ctrl->harq_processes[current_harq_pid];

NR_sched_pucch_t *pucch = NULL;

DevAssert(!harq->is_waiting);

if (sched_pdsch->pucch_allocation >= 0) {

pucch = &sched_ctrl->sched_pucch[sched_pdsch->pucch_allocation];

add_tail_nr_list(&sched_ctrl->feedback_dl_harq, current_harq_pid);

harq->feedback_frame = pucch->frame;

harq->feedback_slot = pucch->ul_slot;

harq->is_waiting = true;

}

UE->mac_stats.dl.rounds[harq->round]++;

LOG_D(NR_MAC,

"%4d.%2d [DLSCH/PDSCH/PUCCH] RNTI %04x DCI L %d start %3d RBs %3d startSymbol %2d nb_symbol %2d dmrspos %x MCS %2d nrOfLayers %d TBS %4d HARQ PID %2d round %d RV %d NDI %d dl_data_to_ULACK %d (%d.%d) PUCCH allocation %d TPC %d\n",

frame,

slot,

rnti,

sched_ctrl->aggregation_level,

sched_pdsch->rbStart,

sched_pdsch->rbSize,

tda_info->startSymbolIndex,

tda_info->nrOfSymbols,

dmrs_parms->dl_dmrs_symb_pos,

sched_pdsch->mcs,

nrOfLayers,

TBS,

current_harq_pid,

harq->round,

nr_get_rv(harq->round % 4),

harq->ndi,

pucch ? pucch->timing_indicator : 0,

pucch ? pucch->frame : 0,

pucch ? pucch->ul_slot : 0,

sched_pdsch->pucch_allocation,

sched_ctrl->tpc1);

DevAssert(sched_pdsch->rbSize > 0);

const int bwp_id = current_BWP->bwp_id;

const int coresetid = sched_ctrl->coreset->controlResourceSetId;

/* look up the PDCCH PDU for this CC, BWP, and CORESET. If it does not exist, create it */

nfapi_nr_dl_tti_pdcch_pdu_rel15_t *pdcch_pdu = nr_mac->pdcch_pdu_idx[CC_id][coresetid];

if (!pdcch_pdu) {

LOG_D(NR_MAC, "creating pdcch pdu, pdcch_pdu = NULL. \n");

nfapi_nr_dl_tti_request_pdu_t *dl_tti_pdcch_pdu = &pdsch->dl_req->dl_tti_pdu_list[pdsch->dl_req->nPDUs];

memset(dl_tti_pdcch_pdu, 0, sizeof(nfapi_nr_dl_tti_request_pdu_t));

dl_tti_pdcch_pdu->PDUType = NFAPI_NR_DL_TTI_PDCCH_PDU_TYPE;

dl_tti_pdcch_pdu->PDUSize = (uint8_t)(2+sizeof(nfapi_nr_dl_tti_pdcch_pdu));

pdsch->dl_req->nPDUs += 1;

pdcch_pdu = &dl_tti_pdcch_pdu->pdcch_pdu.pdcch_pdu_rel15;

LOG_D(NR_MAC,"Trying to configure DL pdcch for UE %04x, bwp %d, cs %d\n", UE->rnti, bwp_id, coresetid);

NR_ControlResourceSet_t *coreset = sched_ctrl->coreset;

nr_configure_pdcch(pdcch_pdu, coreset, &sched_ctrl->sched_pdcch);

nr_mac->pdcch_pdu_idx[CC_id][coresetid] = pdcch_pdu;

}

nfapi_nr_dl_tti_request_pdu_t *dl_tti_pdsch_pdu = &pdsch->dl_req->dl_tti_pdu_list[pdsch->dl_req->nPDUs];

memset(dl_tti_pdsch_pdu, 0, sizeof(nfapi_nr_dl_tti_request_pdu_t));

dl_tti_pdsch_pdu->PDUType = NFAPI_NR_DL_TTI_PDSCH_PDU_TYPE;

dl_tti_pdsch_pdu->PDUSize = (uint8_t)(2 + sizeof(nfapi_nr_dl_tti_pdsch_pdu));

pdsch->dl_req->nPDUs += 1;

/* SCF222: PDU index incremented for each PDSCH PDU sent in TX control

* message. This is used to associate control information to data and is

* reset every slot. */

const int pduindex = nr_mac->pdu_index[CC_id]++;

// TODO: verify the case where maxMIMO_Layers is NULL, in which case

//       in principle maxMIMO_layers should be given by the maximum number of layers

//       for PDSCH supported by the UE for the serving cell (5.4.2.1 of 38.212)

long ue_supp_nl = ue_supported_dl_layers(scc, UE->capability);

long maxMIMO_Layers = UE->sc_info.maxMIMO_Layers_PDSCH ? *UE->sc_info.maxMIMO_Layers_PDSCH : ue_supp_nl;

if (maxMIMO_Layers < 1) {

LOG_D(NR_MAC, "Both maxMIMO_Layers_PDSCH and UE supported layers are not present, defaulting to 1\n");

maxMIMO_Layers = 1;

}

const int nl_tbslbrm = min(maxMIMO_Layers, 4);

const uint16_t fapi_beam = convert_to_fapi_beam(UE->UE_beam_index, nr_mac->beam_info.beam_mode);

nfapi_nr_dl_tti_pdsch_pdu_rel15_t *pdsch_pdu = prepare_pdsch_pdu(dl_tti_pdsch_pdu,

nr_mac,

UE,

sched_pdsch,

current_BWP->pdsch_Config,

false,

harq->round,

rnti,

fapi_beam,

nl_tbslbrm,

pduindex);

LOG_D(NR_MAC,"Configuring DCI/PDCCH in %d.%d at CCE %d, rnti %x\n", frame,slot,sched_ctrl->cce_index,rnti);

/* Fill PDCCH DL DCI PDU */

nfapi_nr_dl_dci_pdu_t *dci_pdu = prepare_dci_pdu(pdcch_pdu,

scc,

sched_ctrl->search_space,

sched_ctrl->coreset,

sched_ctrl->aggregation_level,

sched_ctrl->cce_index,

fapi_beam,

rnti);

pdcch_pdu->numDlDci++;

/* DCI payload */

const int rnti_type = TYPE_C_RNTI_;

dci_pdu_rel15_t dci_payload = prepare_dci_dl_payload(nr_mac,

UE,

rnti_type,

sched_ctrl->search_space->searchSpaceType->present,

pdsch_pdu,

sched_pdsch,

pucch,

current_harq_pid,

0,

false);

// Reset TPC to 0 dB to not request new gain multiple times before computing new value for SNR

sched_ctrl->tpc1 = 1;

NR_PDSCH_Config_t *pdsch_Config = current_BWP->pdsch_Config;

AssertFatal(pdsch_Config == NULL

|| pdsch_Config->resourceAllocation == NR_PDSCH_Config__resourceAllocation_resourceAllocationType1,

"Only frequency resource allocation type 1 is currently supported\n");

LOG_D(NR_MAC,

"%4d.%2d DCI type 1 payload: freq_alloc %d (%d,%d,%d), "

"nrOfLayers %d, time_alloc %d, vrb to prb %d, mcs %d tb_scaling %d ndi %d rv %d tpc %d ti %d\n",

frame,

slot,

dci_payload.frequency_domain_assignment.val,

pdsch_pdu->rbStart,

pdsch_pdu->rbSize,

pdsch_pdu->BWPSize,

pdsch_pdu->nrOfLayers,

dci_payload.time_domain_assignment.val,

dci_payload.vrb_to_prb_mapping.val,

dci_payload.mcs,

dci_payload.tb_scaling,

dci_payload.ndi,

dci_payload.rv,

dci_payload.tpc,

pucch ? pucch->timing_indicator : 0);

fill_dci_pdu_rel15(&UE->sc_info,

current_BWP,

&UE->current_UL_BWP,

dci_pdu,

&dci_payload,

current_BWP->dci_format,

rnti_type,

sched_ctrl->search_space,

sched_ctrl->coreset,

UE->pdsch_HARQ_ACK_Codebook,

nr_mac->cset0_bwp_size);

LOG_D(NR_MAC,

"coreset params: FreqDomainResource %llx, start_symbol %d  n_symb %d\n",

(unsigned long long)pdcch_pdu->FreqDomainResource,

pdcch_pdu->StartSymbolIndex,

pdcch_pdu->DurationSymbols);

if (harq->round != 0) { /* retransmission */

/* we do not have to do anything, since we do not require to get data

* from RLC or encode MAC CEs. The TX_req structure is filled below

* or copy data to FAPI structures */

LOG_D(NR_MAC,

"%d.%2d DL retransmission RNTI %04x HARQ PID %d round %d NDI %d\n",

frame,

slot,

rnti,

current_harq_pid,

harq->round,

harq->ndi);

if (!get_softmodem_params()->phy_test)

AssertFatal(harq->sched_pdsch.tb_size == TBS,

"UE %04x mismatch between scheduled TBS and buffered TB for HARQ PID %d\n",

UE->rnti,

current_harq_pid);

else if (harq->sched_pdsch.tb_size != TBS)

LOG_E(NR_MAC,

"Mismatch between scheduled TBS and buffered TB for HARQ PID %d. No RTX control in phy-test mode. "

"Possible causes: presence of CSI-RS or DLSCH scheduled in the mixed slot.\n",

current_harq_pid);

T(T_GNB_MAC_RETRANSMISSION_DL_PDU_WITH_DATA, T_INT(module_id), T_INT(CC_id), T_INT(rnti),

T_INT(frame), T_INT(slot), T_INT(current_harq_pid), T_INT(harq->round), T_BUFFER(harq->transportBlock.buf, TBS));

UE->mac_stats.dl.total_rbs_retx += sched_pdsch->rbSize;

nr_mac->mac_stats.dl.used_prb_aggregate += sched_pdsch->rbSize;

/* store last DL NPRB (RBs) for MAC statistics logging */

UE->mac_stats.DL_NPRB = sched_pdsch->rbSize;

} else { /* initial transmission */

LOG_D(NR_MAC, "Initial HARQ transmission in %d.%d\n", frame, slot);

// Flag HARQ process to start TCI timer at ACK

harq->start_tci_timer = sched_ctrl->UE_mac_ce_ctrl.tci_state_ind.is_scheduled;

uint8_t *buf = allocate_transportBlock_buffer(&harq->transportBlock, TBS);

/* first, write all CEs that might be there */

int written = nr_write_ce_dlsch_pdu(module_id,

sched_ctrl,

(unsigned char *)buf,

255, // no drx

NULL); // contention res id

buf += written;

uint8_t *bufEnd = buf + TBS - written;

DevAssert(TBS > written);

int dlsch_total_bytes = 0;

/* next, get RLC data */

start_meas(&nr_mac->rlc_data_req);

int sdus = 0;

if (sched_ctrl->num_total_bytes > 0) {

/* loop over all activated logical channels */

for (int i = 0; i < seq_arr_size(&sched_ctrl->lc_config); ++i) {

const nr_lc_config_t *c = seq_arr_at(&sched_ctrl->lc_config, i);

const int lcid = c->lcid;

if (sched_ctrl->rlc_status[lcid].bytes_in_buffer == 0)

continue; // no data for this LC        tbs_size_t len = 0;

int lcid_bytes=0;

while (bufEnd-buf > sizeof(NR_MAC_SUBHEADER_LONG) + 1 ) {

// we do not know how much data we will get from RLC, i.e., whether it

// will be longer than 256B or not. Therefore, reserve space for long header, then

// fetch data, then fill real length

NR_MAC_SUBHEADER_LONG *header = (NR_MAC_SUBHEADER_LONG *) buf;

/* limit requested number of bytes to what preprocessor specified, or

* such that TBS is full */

const rlc_buffer_occupancy_t ndata = min(sched_ctrl->rlc_status[lcid].bytes_in_buffer,

bufEnd-buf-sizeof(NR_MAC_SUBHEADER_LONG));

tbs_size_t len = nr_mac_rlc_data_req(module_id,

rnti,

true,

lcid,

ndata,

(char *)buf+sizeof(NR_MAC_SUBHEADER_LONG));

LOG_D(NR_MAC,

"%4d.%2d RNTI %04x: %d bytes from %s %d (ndata %d, remaining size %ld)\n",

frame,

slot,

rnti,

len,

lcid < 4 ? "DCCH" : "DTCH",

lcid,

ndata,

bufEnd-buf-sizeof(NR_MAC_SUBHEADER_LONG));

if (len == 0)

break;

T(T_GNB_MAC_LCID_DL, T_INT(rnti), T_INT(frame), T_INT(slot), T_INT(lcid), T_INT(len * 8), T_INT(nr_rlc_tx_list_occupancy(rnti, lcid)));

header->R = 0;

header->F = 1;

header->LCID = lcid;

header->L = htons(len);

buf += len+sizeof(NR_MAC_SUBHEADER_LONG);

dlsch_total_bytes += len;

lcid_bytes += len;

sdus += 1;

}

UE->mac_stats.dl.lc_bytes[lcid] += lcid_bytes;

}

} else if (get_softmodem_params()->phy_test || get_softmodem_params()->do_ra) {

/* we will need the large header, phy-test typically allocates all

* resources and fills to the last byte below */

LOG_D(NR_MAC, "Configuring DL_TX in %d.%d: TBS %d of random data\n", frame, slot, TBS);

if (bufEnd-buf > sizeof(NR_MAC_SUBHEADER_LONG) ) {

NR_MAC_SUBHEADER_LONG *header = (NR_MAC_SUBHEADER_LONG *) buf;

// fill dlsch_buffer with random data

header->R = 0;

header->F = 1;

header->LCID = DL_SCH_LCID_PADDING;

buf += sizeof(NR_MAC_SUBHEADER_LONG);

header->L = htons(bufEnd-buf);

for (; ((intptr_t)buf) % 4; buf++)

*buf = lrand48() & 0xff;

for (; buf < bufEnd - 3; buf += 4) {

uint32_t *buf32 = (uint32_t *)buf;

*buf32 = lrand48();

}

for (; buf < bufEnd; buf++)

*buf = lrand48() & 0xff;

sdus +=1;

}

}

stop_meas(&nr_mac->rlc_data_req);

// Add padding header and zero rest out if there is space left

if (bufEnd-buf > 0) {

NR_MAC_SUBHEADER_FIXED *padding = (NR_MAC_SUBHEADER_FIXED *) buf;

padding->R = 0;

padding->LCID = DL_SCH_LCID_PADDING;

buf += 1;

memset(buf,0,bufEnd-buf);

buf=bufEnd;

}

UE->mac_stats.dl.total_bytes += TBS;

UE->mac_stats.dl.current_bytes = TBS;

UE->mac_stats.dl.total_rbs += sched_pdsch->rbSize;

UE->mac_stats.dl.num_mac_sdu += sdus;

UE->mac_stats.dl.current_rbs = sched_pdsch->rbSize;

UE->mac_stats.dl.total_sdu_bytes += dlsch_total_bytes;

nr_mac->mac_stats.dl.used_prb_aggregate += sched_pdsch->rbSize;

/* store last DL NPRB (RBs) for MAC statistics logging */

UE->mac_stats.DL_NPRB = sched_pdsch->rbSize;

/* save retransmission information */

harq->sched_pdsch = *sched_pdsch;

/* save which time allocation has been used, to be used on

* retransmissions */

harq->sched_pdsch.time_domain_allocation = sched_pdsch->time_domain_allocation;

// reset TCI state

if (sched_ctrl->UE_mac_ce_ctrl.tci_state_ind.is_scheduled)

sched_ctrl->UE_mac_ce_ctrl.tci_state_ind.is_scheduled = false;

// ta command is sent, values are reset

if (sched_ctrl->ta_apply) {

sched_ctrl->ta_apply = false;

sched_ctrl->ta_update = 31;

sched_ctrl->ta_frame = (frame + 100) % MAX_FRAME_NUMBER;

LOG_D(NR_MAC, "%d.%2d UE %04x TA scheduled, setting next TA frame to %d\n", frame, slot, UE->rnti, sched_ctrl->ta_frame);

}

T(T_GNB_MAC_DL_PDU_WITH_DATA, T_INT(module_id), T_INT(CC_id), T_INT(rnti),

T_INT(frame), T_INT(slot), T_INT(current_harq_pid), T_BUFFER(harq->transportBlock.buf, TBS));

T(T_GNB_MAC_DL, T_INT(rnti), T_INT(frame), T_INT(slot), T_INT(sched_pdsch->mcs), T_INT(TBS));

}

const int ntx_req = pdsch->TX_req->Number_of_PDUs;

nfapi_nr_pdu_t *tx_req = &pdsch->TX_req->pdu_list[ntx_req];

tx_req->PDU_index  = pduindex;

tx_req->num_TLV = 1;

tx_req->TLVs[0].length = TBS;

tx_req->PDU_length = compute_PDU_length(tx_req->num_TLV, tx_req->TLVs[0].length);

memcpy(tx_req->TLVs[0].value.direct, harq->transportBlock.buf, TBS);

pdsch->TX_req->Number_of_PDUs++;

pdsch->TX_req->SFN = frame;

pdsch->TX_req->Slot = slot;

// RESET HARQ NDI and ROUND once they are used.

// as HARQ is disabled and there will no PUCCH being received.

if (sched_pdsch->pucch_allocation < 0) {

finish_nr_dl_harq(sched_ctrl, current_harq_pid);

}

}

void nr_schedule_ue_spec(module_id_t module_id,

frame_t frame,

slot_t slot,

nfapi_nr_dl_tti_request_t *DL_req,

nfapi_nr_tx_data_request_t *TX_req)

{

gNB_MAC_INST *gNB_mac = RC.nrmac[module_id];

int CC_id = 0;

/* already mutex protected: held in gNB_dlsch_ulsch_scheduler() */

AssertFatal(pthread_mutex_trylock(&gNB_mac->sched_lock) == EBUSY,

"this function should be called with the scheduler mutex locked\n");

if (!is_dl_slot(slot, &gNB_mac->frame_structure))

return;

NR_ServingCellConfigCommon_t *scc = gNB_mac->common_channels[CC_id].ServingCellConfigCommon;

int bw = scc->downlinkConfigCommon->frequencyInfoDL->scs_SpecificCarrierList.list.array[0]->carrierBandwidth;

gNB_mac->mac_stats.dl.total_prb_aggregate += bw;

nfapi_nr_dl_tti_request_body_t *dl_req = &DL_req->dl_tti_request_body;

post_process_pdsch_t pdsch = { frame, slot, dl_req, TX_req };

/* PREPROCESSOR */

gNB_mac->pre_processor_dl(gNB_mac, &pdsch);

}